Environmental Assessment

Expansion and Upgrade of Port Alsworth

Visitor Contact and Maintenance Facilities

# Lake Clark National Park and Preserve, Alaska

Prepared by:

United States Department of the Interior

National Park Service

Katmai National Park and Preserve

May 2003

TABLE OF CONTENTS

1.0  INTRODUCTION	1

1.1  Purpose and Need	1

1.2  Background	2

1.3  Issues Considered for Further Evaluation	3

1.4  Issues Eliminated from Further Consideration	3

2.0  ALTERNATIVES	4

2.1  Alternative A: No-Action (Environmentally Preferred Alternative)	4

2.2  Alternative B: Expansion and Upgrade of Visitor Contact and Maintenance Facilities

(NPS Preferred Alternative)	4

2.2.1  Mitigation Measures	7

3.0  AFFECTED ENVIRONMENT……………………………………………………….......................8

4.0  IMPACT ANALYSIS	.10

4.1  Cumulative Impacts Analysis Assumptions	10

4.2  Alternative A: No-Action (Environmentally Preferred Alternative)	10

4.3  Alternative B: Expansion and Upgrade of Visitor Contact and Maintenance Facilities

(NPS Preferred Alternative)	13

5.0  CONSULTATION AND COORDINATION	18

6.0  LIST OF PREPARERS AND CONTRIBUTORS	19

7.0  REFERENCES	19

APPENDIX

Appendix A – Summary Evaluation and Findings, ANILCA Section 810 (a)	20

LIST OF FIGURES

Figure 1: Regional and Vicinity Maps 	after page 1

Figure 2: NPS Administrative Site Existing Conditions……………………………………….. “  ”   “   ” 2

Figure 3: Alternative B: Proposed Action……………………………………………………… “  ”   “   ” 5

1.0  INTRODUCTION

###### 1.1  Purpose and Need

The National Park Service (NPS) is considering facility improvements at the Lake Clark field headquarters site in Lake Clark National Park and Preserve. The proposed project would entail expansion and upgrade of the visitor contact and maintenance facilities at the developed “airstrip” administrative site adjacent to the airstrip in Port Alsworth (Figure 1). The maintenance function would be transferred from the existing maintenance shop to the existing visitor center/hangar. Visitor contact functions would be transferred from the existing visitor center/hangar to the existing maintenance shop. The existing visitor center/hangar footprint would be expanded to approximately double its present size. The interiors of both buildings would be renovated and upgraded to better serve their new functions.

In addition, the park’s incinerator would be moved from its current location near the access road and airstrip to a new building to be constructed in the present storage yard (immediately south of the park sewage lagoon). A new warehouse would be constructed in the storage yard. The cleared area that constitutes the storage yard would be expanded to replace the outdoor storage space that would become occupied by the two new buildings and to provide sufficient storage and staging areas for boats, vehicles and materials. The entire storage yard would be fenced. Utility work related to these improvements would include providing water, sewer and fuel service to the new maintenance building and new visitor center. An underground fuel line would be installed for the existing warehouse. Fuel service to the new warehouse and the new incinerator would be provided from a new, shared 500-gallon double-wall aboveground tank with new underground piping to each building. Underground electrical service would be installed for the new incinerator building and new warehouse.

The purpose of this proposed project is to (1) improve the park’s visitor contact facilities to allow greater visitor satisfaction and (2) improve the park’s maintenance facilities to better accommodate the growing needs of park programs and satisfy regulatory health and safety requirements.

The proposed improvements are needed for several reasons. The current visitor contact room in the existing visitor center/hangar building is unfinished, poorly lit and crowded. It can only accommodate a few visitors at a time and there is no room for visitor groups to sit and rest. The visitor center has inadequate storage space available for collections, visitor contact information and library materials. Presently, no public restroom is available for visitors within the visitor center/hangar building. Visitors must walk uphill approximately 50 yards to use a single outhouse with no running water. No meeting space is available for interpreters to serve school groups that visit the park. Interpreters have no office space to assemble and store interpretive programs and displays. Collections of cultural resource materials cannot be safely stored in a secure area, risking the rapid deterioration of valuable items. Lack of proper collections storage at the park limits future on-site collections, as they must be transported to safe storage locations off-site. The park has no finished, large meeting space with a public restroom to hold public meetings and gatherings within the headquarters area.

The existing maintenance shop does not comply with current building utility codes and OSHA regulations regarding workplace safety. Maintenance employees have no restroom or safety shower facilities in the shop. The existing workspace and storage areas are inadequate to accommodate the minimal requirements of park facility operation and maintenance programs. The incinerator facility is located in close proximity to the airstrip and access road. Due to crowding, bags of recyclable materials are stored outside of the incinerator building, creating an unsightly distraction for park visitors who walk the area footpaths.

This environmental assessment presents and analyzes a “no action” and a proposed action alternative, and the associated environmental impacts. It has been prepared in accordance with the National Environmental Policy Act (NEPA) of 1969, and the regulations of the Council on Environmental Quality (40 CFR 1508.9).

###### 1.2  Background

The NPS Organic Act and the General Authorities Act prohibit impairment of park resources and values.  The NPS Management Policies 2001 uses the terms “resources and values” to mean the full spectrum of tangible attributes for which the park is established and are managed, including the Organic Act’s fundamental purpose and any additional purposes as stated in the park’s establishing legislation. The impairment of park resources and values may not be allowed unless directly and specifically provided by statute. The primary responsibility of the NPS is to ensure that park resources and values would continue to exist in a condition that would allow the American people to have present and future opportunities for enjoyment of them.

Lake Clark National Park and Preserve was established by the Alaska National Interest Lands Conservation Act (ANILCA) in 1980 "to protect the watershed necessary for the perpetuation of the red salmon fishery in Bristol Bay, to maintain unimpaired the scenic beauty and quality of portions of the Alaska Range and the Aleutian Range…and to protect habitat for and populations of fish and wildlife…" The 2.6 million acre park and adjoining 1.4 million acre preserve lie approximately 100 miles southwest of Anchorage.

Since 1982, the NPS has maintained its field headquarters at Port Alsworth, a small community on the east shore of Lake Clark. With a 2000 population of approximately 110, Port Alsworth consists primarily of private lodges serving fishermen, hunters, and other outdoor enthusiasts. It is accessible only by aircraft from Anchorage (approximately 175 air miles to the northeast) or Iliamna (approximately 38 air miles to the southwest), or by boat from Nondalton.

Most NPS management, housing, and maintenance functions are located on the west side of Hardenburg Bay, near the airstrip, on a 15.61-acre lot purchased in August 1986 (Figure 1). Existing NPS facilities at the airstrip site include the headquarters office building, the resources office building, visitor center/hangar, two permanent housing units, four weatherports used for seasonal housing, several outbuildings, maintenance shop, warehouse, incinerator building, a fuel tank farm, a sewage lagoon and leach field, and a long-term outside storage yard (Figure 2).

A small number of NPS facilities are also located on the “Point”, a small piece of land that forms the southern boundary of the bay (Figure 1).

Relationship of the Proposal to Other Park Planning

The 1982 General Management Plan/Development Concept Plan for Lake Clark National Park and Preserve called for expansion of the NPS facilities on the Point, because, at the time, this was the only property the NPS owned in the Port Alsworth area. Acquisition of the airstrip property in 1986 altered this direction, however. This enabled the park to expand its management functions to include land-based aircraft operations, research and additional housing and administrative support. The special site requirements of some functions and the availability of existing structures have been the major considerations in locating park operations and housing. All law enforcement, dispatch, backcountry patrol and search and rescue activities are based at the Point because it is the only logical site from which to conduct boat and floatplane operations. Other operations and maintenance functions are consolidated at the airstrip because it is a logical place to store the materials transported to and from the airstrip field management site. The current management direction is to use the airstrip property as the NPS primary base of operations.

The 1994 Site Facility Survey and Development Plan included the review of existing park facility conditions and work functions and a review of management documents regarding the development of visitor and maintenance facilities. The document recommended that several facilities would likely be needed in the future. These included a visitor contact facility (1,750 square feet), an incinerator building, a hangar (3,600 square feet) and two additional shops (wood and metal shop additions that would increase the size of the maintenance facility to approximately 4,000 square feet).  All new structures should be sited to take advantage of existing clearings. All new utilities could be installed parallel to or within existing pathways serving facilities.

# 1.3  Issues Considered for Further Evaluation

To focus the environmental assessment, the NPS selected specific issues for further analysis and eliminated others from further evaluation. Subsequent discussions of the affected environment and environmental impacts related to each alternative focus on these issues. A brief rationale for the selection of each topic is given below.

Aesthetics.  The expansion and upgrade of visitor contact and maintenance facilities could affect the aesthetics of the NPS airstrip site.

Air Quality. The air quality of the NPS airstrip site could be affected by construction activities of the proposed project.

Soundscape.  Noise generated by construction activity could affect the natural soundscape of the area.

Soils and Vegetation. Soils and vegetation in the proposed project area could be disturbed during excavation associated with the construction of new buildings, building additions and utility corridors.

Wildlife. Terrestrial wildlife (passerine birds and small mammals) could be affected by short-term displacement from preferred habitat and loss of habitat as a result of construction activities and facility placement.

Cultural Resources.  Archeological resources could be affected by new building construction/expansion and utility systems.

Visitor Use.  Improvements to the visitor contact and maintenance facilities could affect the satisfaction of visitors using the park administrative area.

Park Management.  The proposed project could have direct impacts on park operations, including maintenance, interpretation and cultural resources management.

# 1.4  Issues Eliminated from Further Consideration

A brief rationale dismissing specific topics from further consideration is included below.

Water Resources.  No actions would be taken that would impact the groundwater within the project area. No surface water is present in or near the project area.

Floodplains.  There are no rivers or streams in the vicinity of the proposed project and the area does not appear to be in a regulatory floodplain. Thus, no impacts to floodplains would occur.

Wetlands. The USFWS surveyed the airport parcel in 1994 for waters of the U.S., including wetlands. The USFWS concluded that the 15-acre administrative tract is non-wetland. Thus no impact to wetlands would occur (USFWS, 1994).

Fish.  The proposed facility expansion would occur well away from surface water and would have no direct or indirect impacts on fish or aquatic life.

Wilderness.  The proposed action would not occur within designated wilderness and, therefore, would not impact wilderness characteristics or values.

Threatened, Endangered, and Other Special Status Species. The Port Alsworth area was surveyed in 1993 and no threatened or endangered species were found. Consultation with the US Fish and Wildlife Service confirms that there are no known federal or state listed threatened or endangered species, federal candidate species, or state listed species of special concern within the project area or the Lake Clark general area (per phone conversation on 04/03/03 with Greg Balough, USFWS Ecological Services).

Subsistence.  The effects of the proposed action on subsistence uses and needs were dismissed from further analysis because the proposed action would not result in a significant restriction of subsistence uses. An ANILCA Section 810(a) summary evaluation and analysis is contained in Appendix A.

Local Economy.  Labor and some materials for this project may be obtained from the nearby community of Port Alsworth; however, any related impacts on the local economy likely would be negligible due to the small size and duration of the project.

Executive Order 12898, “Environmental Justice.”  Executive Order 12898, Federal Actions to Address Environmental Justice in Minority Populations and Low Income Populations, requires all federal agencies to identify and address disproportionately high and adverse human health or environmental effects of their programs and policies on minorities and low-income populations and communities. This project would not be expected to result in significant changes in the socioeconomic environment of the project area, and therefore would not be expected to have any direct or indirect impacts to minority or low-income populations or communities.

## 2.0  ALTERNATIVES

###### 2.1  Alternative A: No-Action (Environmentally Preferred Alternative)

Under the no-action alternative, the NPS would continue to operate and maintain the existing visitor contact and maintenance facilities at the airstrip site in Port Alsworth. The “Affected Environment” section describes existing facilities and functions. No facility expansions or upgrades would be completed. This alternative provides a baseline for evaluating the changes and impacts of the proposed action alternative.

###### 2.2  Alternative B: Expansion and Upgrade to Visitor Contact and Maintenance Facilities.                   (NPS Preferred Alternative)

This alternative would provide a visitor contact area and visitor center that would include a large meeting room in a finished space, a public restroom, storage and display space for visitor information, offices and a collections storage room. Visitor contact would occur within the centralized park administrative area.

The maintenance upgrade would enable the park to be in compliance with OSHA requirements and building utility codes as well as NPS standards for employee work environments. An expanded maintenance building and new warehouse and incinerator buildings would provide adequately sized work areas and storage space for maintenance and operations programs. Several maintenance functions, such as outdoor storage, recycling activities and incineration, would be better consolidated within the storage yard area for greater efficiency, security and avoidance of the public view.

The facility improvements at the park administrative area would consist of the project elements listed below:

- Convert the existing maintenance shop into a new visitor center

- Convert the existing visitor center/hangar into a new maintenance building

- Construct a new incinerator building

- Construct a new warehouse

- Upgrade the existing maintenance warehouse

- Provide utilities to serve the above facilities

- Expand and fence the storage yard

The NPS would expand and upgrade the visitor contact and maintenance facilities at the park’s field headquarters site in Port Alsworth. The proposed project would entail transferring the maintenance function from the existing maintenance shop to the existing visitor center/hangar and transferring the visitor center from the current visitor center/hangar to the current maintenance shop. The existing visitor center/hangar building footprint would be expanded to approximately double its present size. The interiors of both buildings would be renovated and upgraded to better serve their new functions. Also included would be moving the park’s incinerator from its current location near the airstrip to a new building to be located in the present storage yard, away from areas normally frequented by visitors and non-maintenance staff. A new warehouse would be constructed in the storage yard. This project would not involve demolition of any buildings. The cleared area that constitutes the storage yard would be expanded to replace the storage space that would become occupied by the two new buildings and to provide sufficient storage and staging areas for boats, vehicles and materials. The entire storage yard would be fenced. Utility work related to these improvements would include providing water, sewer and fuel service to the new maintenance building and new visitor center. An underground fuel line would be installed for the existing warehouse. A new, aboveground 500-gallon fuel tank with underground piping would provide fuel for the new warehouse and new incinerator. Underground electrical service would be installed to the new incinerator building and warehouse.

Project elements are described below and their locations displayed in Figure 3.

1.  Convert the existing maintenance shop into a new visitor center.  Materials and supplies associated with park maintenance activities would be moved from the shop to the current visitor center/hangar. The building’s interior would be renovated to accommodate the new park visitor center. This new facility would include a unisex restroom, consisting of a flush toilet and sink, accessible by doors leading from both inside and outside the building. Three offices, each approximately 100-120 square feet, would be occupied by park interpretive and resource management staff. A secure collections storage room of approximately 70 square feet would be built between two of the offices. Within the central building interior, a “common room”, of approximately 750 square feet, would serve as a display area, library and meeting room. A new covered front porch of approximately 200 square feet would be added to the west end of the building. The public restroom would be accessible from outside the building by this covered porch.

2.  Convert the existing visitor center/hangar into a new maintenance building.  Upon completion of the previous project element, all material, supplies, and furnishings in the current visitor center would be moved to the new visitor center. Subsequently, conversion of the existing visitor center/hangar into the new park maintenance building would commence. Primary work elements would consist of providing 24’ x 28’ (672 square feet) and 48’ x 28’ (1,344 square feet) additions on the east and west sides, respectively, of the existing structure. The building interior would be completely remodeled to reflect its new function. The new additions would require the placement of approximately 30 – 35 cubic yards of concrete to form floor slabs on which primarily wood structures of standard balloon-frame construction would be built.  Wall heights and rooflines would match those of the existing structure, and exterior surface materials would also match. To attain final proper grades for drainage and access, approximately 250-300 cubic yards of ground surface material, primarily gravel and rock, would need to be excavated and removed. It is anticipated that some of this material would be screened to provide approximately 50 cubic yards of aggregate to be used in mixing concrete for this project.

3.  Construct a new incinerator building.  The existing park incinerator, housed in a 400 square foot building approximately 100 feet northwest of the existing maintenance shop, would be moved into a newly constructed, 750 square foot building to be located in the park’s outside storage yard. This site is approximately 850 feet southeast of the present incinerator location and approximately 25 feet southeast of the sewage lagoon. The new building would consist of a slab-on-grade of approximately 11 cubic yards over which would be constructed a pre-fabricated metal building with walls of approximately eight feet in height and a three-in-12 maximum roof pitch. The existing incinerator building would remain in place and used for cold storage.

4.  Construct a new warehouse.  A new heated warehouse of approximately 960 square feet would be constructed in the storage yard area within approximately 50 feet south of the new incinerator building described in the preceding project element. This would involve placement of approximately 14 cubic yards of concrete to form a slab-on-grade on which would be erected a pre-fabricated metal building with walls of approximately eight foot height and a maximum three-in-12 roof pitch. This building would be insulated.

5.  Upgrade the existing maintenance warehouse.  The existing eight-foot double swing door would be replaced with an overhead garage door. Interior renovations would include installation of dry wall, insulation and windows.

6.  Provide utilities to serve the above facilities.  Utility work related to these improvements would include providing water and sewer service to the new maintenance shop, underground fuel lines to the new incinerator building and both the old and the new warehouses, a fuel tank to serve the new warehouse and new incinerator, and underground electrical service to the new incinerator building and warehouse. Specific tasks are as follows:

Water and Sewer:  Water and sewer service would be provided to the new maintenance building. A new one-inch underground domestic water line would be run approximately 20 feet from a tie-in to the existing two-inch underground main to the new west wing of the building.  Bury depth would be approximately six to eight feet. A new sewage lift station would be placed in the floor of the new west wing. From this new lift station, a new two-inch underground sewer force main (pressurized) would be placed, parallel to the domestic water line, for approximately 150 feet to the southeast corner of the existing maintenance shop. At this point, it would be connected to the existing four-inch underground gravity sewer main. Bury depth of this new line would vary from six to eight feet and according to code, it would need to be placed no closer than ten feet away from any water service. A total of approximately 170 linear feet of water and sewer lines would be required for this project.

Fuel:  New diesel fuel service would need to be provided to the existing warehouse, the new incinerator building, and the new warehouse. The old warehouse would be fed from an existing fuel tank approximately 50 feet away with new underground one-half inch double-walled pipe. Service to the new warehouse and the new incinerator would be provided from a new, shared 500-gallon double-wall aboveground tank with new underground one-half inch double-walled pipe to each building. A total of approximately 50 feet of fuel line would be installed. To accommodate the new location of the heating unit in the new visitor center, the present underground service would need to be removed from the building, approximately 30 feet back to the tank and new one-half inch double-walled pipe provided from the tank approximately 80 feet to the new service entry into the building. A new fuel line would be placed approximately 50 feet from the new maintenance building to the existing 500-gallon aboveground fuel tank. All underground fuel piping would be buried at a depth of approximately 24 inches. An approximate total of 260 linear feet of fuel line installation would be required.

Electrical:  Electrical service would be provided to the two new buildings to be built in the storage yard. New underground service would be run from the existing sewage pump building on the north side of the sewage lagoon approximately 200 feet to the new buildings. Service would be direct burial cable and would be buried at a depth of approximately 24 inches.

7.  Expand and fence the storage yard.  The park’s unfenced, existing storage yard would be expanded into the surrounding vegetation by approximately 8,400 square feet. This would alleviate the loss of outdoor storage area lost by construction of the two new buildings. At the conclusion of this work the entire area would be enclosed with eight-foot high chain link fence, both for security and to prevent damage to vegetation surrounding the area. Thus, the existing area (12,700 square feet) combined with the expansion (8,400 square feet) would create a total, fenced storage yard of approximately 21,100 square feet.

Construction schedule.  The major construction elements of this project would begin in June 2003 and last approximately 17 months. Equipment and materials would either be taken from the park’s existing inventory or flown in from Anchorage via a combination of regularly scheduled flights with area flight service providers and charter flights primarily originating from Anchorage. NPS workers would complete most of the work for this project. It is possible that additional workers would be hired from the local community. Major outdoor construction elements would be done during June-September of 2003 and 2004. The project would be completed by September, 2004.

2.2.1  Mitigation Measures

Cultural Resources. Should unknown resources be uncovered during project implementation, work would be stopped in the discovery area and the NPS would perform consultations in accordance with 36 CFR 800.11. A Determination of Eligibility would be conducted. If proposed excavation locations could not be adjusted to avoid cultural resource impacts, a Memorandum of Agreement (MOA) with the Advisory Council on Historic Preservation and the Alaska State Historic Preservation Office, that incorporates comments from consulting parties, would be executed. The MOA would specify measures to mitigate adverse effects. Furthermore, as appropriate, the NPS would abide by provisions of the Native American Graves Protection and Repatriation Act of 1992. Any artifacts recovered from park property at the project site would be accessioned, cataloged, preserved, and stored in compliance with the NPS Cultural Resource Management Guideline.

Site Rehabilitation. A site rehabilitation plan would be developed for the proposed project area. This plan would include the disturbed area in this project as well as the disturbed area from previous projects. Ground disturbance would be limited to a maximum buffer of twenty feet around new buildings and upgraded structures. Additional ground disturbance would occur for utility corridors (approximately 8,920 square feet) and the storage yard expansion (approximately 8,400 square feet). The storage yard expansion and fencing project elements would be done during the second year to allow ample time for rehabilitation plan development and implementation. No non-native species would be introduced into the area. Excavation, backfilling, and revegetation would be accomplished with a combination of hand tools and a diesel backhoe. Whenever possible, cut trees would be used as firewood and stumps would be pulled and burned on site. Organic debris would be burned or buried on site. In the undisturbed and/or heavily vegetated areas affected, mats of ground cover, birch seedlings, and the lowermost organic layer (primarily peat), would be salvaged and used to revegetate the areas surrounding new construction.

2.3  Environmentally Preferred Alternative

The no-action alternative is the environmentally preferred alternative, because it would cause no damage to the biological or physical environment, whereas, the NPS preferred alternative would result in minor environmental impacts to soils and vegetation (see “Impact Analysis” section below for more information about these impacts).

###### 3.0  AFFECTED ENVIRONMENT

Aesthetics. The administrative area contains an assortment of buildings and facilities that are interspersed throughout the 15.61-acre lot. These buildings are typically painted dark colors and blend in with the forested environment. The lot contains gravel paths that link the facilities, housing and park functions. A storage yard, of approximately 12,700 square feet, lies to the south of the sewage lagoon. It is used mainly for materials storage. The administrative area affords a spectacular, unobstructed view of surrounding mountains, including Tanalian Mountain to the east.

Air Quality. Air quality is excellent within the administrative area. Even so, dust levels increase on a temporary, intermittent basis, correlating to periods of high winds or aircraft activity on the two, nearby private airstrips.

Water Resources.  The closest surface waters are Hardenburg Bay (Lake Clark) and the Tanalian River/Dry Creek. Runoff from the project site would not reach these waterbodies, which are located about 1,600 feet and 2,000 feet, respectively, from the project site. Private residential and lodge development surround Hardenburg Bay, which is heavily used by floatplanes and boats during the summer months. Dry Creek, a side channel of the Tanalian River, flows seasonally during periods of high runoff. Water quality in both waters is considered good.

Soundscape.  The project area is used primarily as a year-round base of operations for Lake Clark park. Summer recreation activity increases in the project area, as well as in the surrounding area. Although the project area is predominantly a natural area, the natural soundscape is intermittently impacted during the summer season by noise associated with increased aircraft, ATV traffic and residential activities.

Soils and Vegetation.  In undisturbed areas, black spruce (Picea mariana) mixed with paper birch (Betula papyrifera) and a few white spruce (Picea glauca) dominate the Port Alsworth area. A deep mat of mosses (mostly Sphagnum sp. and Hylocomium splendens) and dwarf shrubs (primarily Ledum groenlandicum, Vaccinium vitis-idaea, and Betula glandulosa) form a dense layer. Soils consist of well-drained, uniformly graded gravels overlain by a thick (six to twelve inch) tundra vegetative mat.

In the general area of the administrative complex, black spruce and Labrador tea dominate the forested areas. Disturbed areas of the administrative site are lightly revegetated with poplar (Populus balsamifera), paper birch, willow (Salix bebbiana) and grass (Calamagrostis canadensis). The ground adjacent to the existing buildings has already been disturbed by past activities. Much of the original vegetation/soil mat has been removed, leaving only mineral soil or gravel. The soil and gravel are generally well drained although discontinuous ice lenses do occur in other areas around Port Alsworth.

Undisturbed vegetation surrounding the existing storage yard footprint is black spruce woodland; the climax successional stage for the area. Ground cover is 100% and consists of a thick entangled mat of mosses, 8”-12”deep, intermixed with a shrub layer dominated by labrador tea. Resin birch, low-bush cranberry and black spruce of diameter 3”-6” are common.

Wildlife.  The project site is utilized primarily by passerine birds that are common in the park such as chickadees, redpolls, warblers, ruby-crowned kinglets, robins and gray jays. Red squirrels, voles, shrews, foxes and showshoe hares inhabit the surrounding forest. Larger mammals such as black bears, brown bears and moose occasionally cross through the project area. A path runs east-west across the southern border of the 15.61-acre administrative lot. It is unknown to what degree this path is used by wildlife, but it is commonly used by park employees.

Cultural Resources.  The NPS historic preservation officer has determined that no known cultural resources exist within the 15-acre administrative developed site.

Visitor Use.  Approximately 300-400 visitors sign in annually at the visitor center located in the park’s administrative site, with most use occurring during the summer months. Visitors can view the center’s exhibits and walk along the several, gravel paths throughout the administrative site.

Park Management.  Existing NPS facilities at the airstrip site include the headquarters office building, the resources office building, a visitor center/hangar, two permanent housing units, four weatherports used for seasonal housing, a maintenance shop, a warehouse, an incinerator building, a fuel tank farm, a sewage lagoon and leach field, and a long-term outside storage yard (Figure 2).

The existing visitor center/hangar is divided into two sections, with the present visitor center occupying the south end of the building and comprising roughly 390 square feet. The remainder of the building is an open single room with a concrete floor, used primarily for storage and flight staging activities. The building has no toilet facilities. At 390 square feet, the visitor center portion has insufficient space to allow a full range of interpretive and community out-reach activities. The location of the building, adjacent to the airstrip, results in the majority of the visiting public walking through an active fixed-wing aircraft and helicopter staging and fueling area to access the visitor center.

The existing maintenance shop is approximately 1,200 square feet. Within this limited space are such diverse activities as vehicle and boat repair and maintenance, carpentry, painting, and welding. The shop has no toilet or shower facilities. Severely undersized for the breadth of activities that it must support, the shop building has been cited for numerous health and safety violations over the years. The building is located in the main area of the administrative complex, and as a consequence presents an unsightly appearance to the visiting public.

Similarly located in the main area of the administrative complex is the park’s incinerator facility. This too presents an unattractive appearance to the visiting public. The building in which the incinerator is housed lacks adequate storage space for recyclable material. As a result, bags of recyclable materials must be stored outside of the building, creating an unpleasant distraction for visitors. During the summer months, park staff must move this material indoors to avoid attracting bears.

The existing warehouse is approximately 800 square feet. It is currently used, as is the existing visitor center/hangar, for storage of not only park supplies and equipment, but also those of researchers from both other federal agencies and outside the federal government working on long-term park projects. As base park operations and collateral research projects continue to increase, inside space for storage and staging has become in critically short supply.

### 4.0  IMPACT ANALYSIS

4.1  Cumulative Impact Analysis Assumptions

Cumulative impacts are defined as the incremental impacts to the environment resulting from adding the proposed action to other past, present, and reasonably foreseeable future actions (also referred to as regional actions), regardless of what agency (federal or non-federal) or person undertakes those actions.  Cumulative impacts may result from singularly minor but collectively significant actions taking place over a period of time (CEQ Sec 1508.7).

Cumulative impacts are analyzed in section 4.0 by considering the past, present, and reasonable foreseeable future actions taken by the NPS, the State of Alaska and individuals in the Port Alsworth area. These include the following:

- Past construction of numerous NPS and private structures, including offices, storage facilities, commercial lodges, private residences, utilities, roads, trails, and two private airstrips.

- Past, present and future operation of the above facilities and infrastructures.

- Future actions such as the possible construction of NPS structures (four duplex housing units and a hangar, with associated roads and utility lines) on NPS-owned land near the airstrip, a Port Alsworth landfill/waste transfer facility and a State of Alaska airport. The state airport is included in the top ten priority needs list for the Lake and Peninsula Borough (per phone conversation on 5/3/03 with Marvin Smith, Lake and Peninsula Borough). However, the airport proposal has yet to be evaluated under a Department of Transportation and Public Facilities, multi-year master planning process that allows the opportunity for public involvement. The extent, location and associated infrastructure of the proposed airport and waste transfer facility are unknown and cannot be fully evaluated at this time.

4.2  Alternative A: No-Action (Environmentally Preferred Alternative)

Aesthetics.  With this alternative, no impacts to the aesthetics of the park administrative area would occur since no soil or vegetation would be disturbed and no structures would be built.

Cumulative Impacts.  Existing facilities at the 15.61-acre site have modified the visual landscape from one of a natural setting to one where the sights and sounds of man are noticeable. The development of additional facilities would continue the trend toward development, causing minor, negative changes to aesthetics, since additional man-made facilities and infrastructure would be present in the park airstrip and Port Alsworth areas.

Conclusion.  The no-action alternative would not impact park aesthetics. The level of impacts on aesthetics anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Air Quality.  Air quality would be unchanged because there would be no emissions or dust created by construction activity.

Cumulative Impacts.  Construction activities for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would negatively impact air quality to a negligible degree during short-term periods. These effects would be primarily from fugitive dust associated with construction activities. Transfer facility and airport operations would be expected to result in negligible, long-term negative impacts to air quality in the greater Port Alsworth area because of machinery emissions.

Conclusion.  This alternative would generate no impacts to air quality. The level of impacts on air quality anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Soundscape.  No impacts would be expected since there would be no excavation or construction.

Cumulative Impacts.  Construction noise from future projects would adversely impact the natural soundscape in the short-term to a negligible degree. Long-term noise from operation of the outlying transfer facility and state airport would be expected to negatively impact the soundscape of the area to a negligible degree.

Conclusion.  This alternative would not impact the natural soundscape of the project area. The level of impacts on soundscape anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Soils and Vegetation.  Under this alternative, no impacts to soils or vegetation would occur, since no new excavation or ground disturbance is proposed. Existing impacts to vegetation and soils from past construction activities would continue.

Cumulative Impacts.  Since a revegetation plan would not be implemented with this alternative, ground disturbance from past NPS projects would not be addressed and corrected, resulting in continued, minor, negative, long-term impacts. Construction and related ground disturbance activities from future NPS projects would be expected to result in minor, negative, short-term impacts to soils and vegetation in the administrative area. Soils and vegetation would most likely be impacted to a minor degree with future non-NPS projects. To mitigate impacts, state airport and road projects would be expected to include best management practices and revegetation to conserve soils.

Conclusion.  No impacts would occur under this alternative. Ground disturbance from past NPS projects would continue uncorrected, resulting in continued, minor, negative, long-term impacts. However, the level of impacts on soils and vegetation anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Wildlife.  No impacts to wildlife would occur because no new construction would occur.

Cumulative Impacts.  Construction activities for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would create a minor, negative, short-term impact to wildlife due to disturbance and habitat loss in the area. However, because this area would represent a very small percentage of the available wildlife habitat in the region, only negligible, long-term adverse effects to wildlife would be expected.

Conclusion.  No wildlife impacts would be expected with this alternative. The level of impacts on wildlife anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Cultural Resources.  Cultural resources in the administrative area would be unaffected, since no new ground disturbance would occur.

Cumulative Impacts.  No impacts would be expected from future actions to the administrative area, since no cultural resources have been known to be impacted from past activities in the project area. Construction activities on unsurveyed sites (possibly for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities) would be regulated under state and federal requirements for cultural resource protection. Any impacts to cultural resources in unsurveyed areas would need to be evaluated on a site and project-specific basis by cultural resource professionals.

Conclusion.  No impacts would occur in the administrative area under this alternative. The level of impacts on cultural resources anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Visitor Use.  Under this alternative, visitor contact activities would continue in the existing visitor center/hangar. Park visitors would continue to experience the effects of the existing substandard visitor contact facility. This facility would continue to be crowded and inadequate to fulfill park requirements to provide safe, comfortable facilities and a high quality visitor experience. The visiting public would continue to have no convenient access to standard toilet facilities. Visitors would still be subjected to full view of park maintenance activities and garbage storage and incineration, thereby further degrading the quality of the visitor experience. The park would continue to lack suitable space for interpretive programs and public meetings.

Cumulative Impacts.  A new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would create minor, short-term disturbances for visitors during construction. With the construction of a new state airport, aviation safety and park visitation numbers would be expected to improve, as visitors were afforded greater transportation reliability and safety to and from Port Alsworth. However, the current visitor contact facility would be unable to provide adequate services to these additional visitors, continuing a negative impact to visitor use.

Conclusion.  The no-action alternative would result in negative effects to the current visitor experience. Crowded and inadequate visitor facilities and visitor access to maintenance functions would continue to detract from the visitor experience.

Park Management.  With this alternative, park management would continue to be negatively impacted in the short-term and long-term. Employee safety and health would continue to be compromised under current conditions. The current electrical wiring system would remain in non-compliance with NPS and OSHA requirements and would continue to be a safety concern. Employee and visitor safety would remain an issue, due to the close proximity of fixed-wing aircraft and helicopter operations to other maintenance functions and the visitor center. The ability of the park to reach out to the community would remain limited by the lack of space to accommodate large meetings and develop interpretive programs.

Park maintenance activities would continue to be based in the existing maintenance shop. Staff working in this facility would continue to lack convenient access to toilet or shower facilities. Performance of the wide range of maintenance and repair activities required to allow the park to function would remain hazardous due to both a lack of space and related safety considerations, such as separation of maintenance tasks (carpentry, metal working, painting, etc.) and ventilation. The park’s ability to effectively segregate trash for recycling would be restricted by the continuing inadequacy of indoor space in which to store this material.

Adequate warehouse storage space for equipment, material and supplies needed for park operations in Port Alsworth and the park backcountry would continue to be in critically short supply. This would continue to adversely impact the ability of both park staff and outside researchers to accomplish their work.

Cumulative Impacts.  Construction activities for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would create minor, short-term disturbances to everyday park management. However, minor, long-term, positive impacts would be expected to park management in the areas of aviation safety, employee housing and waste handling.

Conclusion.  This alternative would continue to negatively impact park management operations, both in the short-term and long-term as a result of inadequate and unsafe visitor and maintenance facilities.

4.3  Alternative B: Expansion and Upgrade of Visitor Contact and Maintenance Facilities

(NPS Preferred Alternative)

Aesthetics.   During construction and excavation activities over the course of the two-year proposed project, the administrative lot would exhibit short-term, minor, negative impacts to aesthetics. The presence of additional on-site structures would continue to modify the natural landscape of the area to one with a more developed appearance. About 1.26 acres or 8.1% of the administrative area would be affected by project development and ground disturbance activities. The implementation of a revegetation plan for the proposed project, as well as past projects within the administrative lot, would gradually be expected to improve the landscape of the area over the course of approximately two to four years..

Cumulative Impacts.  Construction activities for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would cause minor, negative, short-term impacts to the aesthetics of the general area. The new structures would create a minor, negative long-term effect to aesthetics. To help mitigate these effects, future state projects would be expected to include erosion control and revegetation. It is expected that a similar revegetation plan would apply to future NPS projects. Thus, future projects would be expected to have a minor, negative cumulative impact to the aesthetics of the area.

Conclusion.  This alternative would have a short-term, minor, adverse effect on the aesthetics of the administrative site as result of construction activities. The additional structures would give the area a more developed appearance. However, aesthetics would be expected to improve at the administrative site over two to four years as a revegetation plan is implemented. The level of impacts on aesthetics anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Air Quality.  Air quality would be adversely affected during excavation and construction activities from June to September of 2003 and 2004. These construction activities would cause some negligible short-term and localized impacts to ambient air quality due to airborne dust and machinery emissions. Loose soil would have potential to be windblown from the work site during excavation activities until the ground cover is established. The amount of dust would be limited due to the small work site location and site protection from prevailing winds. No increase in incinerator operations is expected in the future, as staff size should remain relatively stable. No long-term effects would be anticipated to the air quality in the park administrative site.

Cumulative Impacts.  Construction activities for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would generate airborne dust and emissions that would cause short-term negligible impacts on air quality. The transfer facility and airport operations would be expected to result in negligible, long-term, negative impacts to air quality in the greater Port Alsworth area primarily through machinery emissions.

Conclusion.  Construction activities associated with this alternative would generate dust and machinery emissions that would have a short-term negligible impact on air quality. No long-term impacts to air quality would occur from the operation of the facilities. The level of impacts on air quality anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Soundscape.  Noise levels from construction machinery at the project site would be expected to cause a negative, negligible, short-term impact. Facility operations and visitor use of the area would create low levels of noise that would be audible but would have little effect on the area soundscape.

Cumulative Impacts.  Construction noise from future area projects would cause an adverse, short-term, negligible impact on the natural soundscape. Long-term noise from operation of the outlying transfer facility and state airport would also be expected to negatively impact the soundscape of the area to a negligible level.

Conclusion.  Construction noise would cause short-term, negligible impacts to the soundscape of the area. Over the long-term, facility operations and visitor use of the area would create low levels of noise that would have a negligible effect on the natural soundscape. The level of impacts on soundscape anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Soils and Vegetation.  This alternative would be expected to result in minor, short-term and long-term negative impacts to soils and vegetation in the project area. A total ground disturbance of approximately

54,870 square feet, or 1.26 acres (about 8.1% of the administrative site), would be expected. This consists of new and upgraded buildings (24,846 square feet), utilities (8,920 square feet), existing storage yard (12,700 square feet) and storage yard expansion (8,400 square feet). Even so, all but approximately 8,400 square feet of this disturbance would occur in soil that has previously been disturbed. No non-native species would be introduced into the project area. Soils would be disturbed during excavation for the new buildings, additions and utility lines. Ground disturbance would be limited to a maximum buffer of twenty feet around the new incinerator building, new maintenance warehouse and existing visitor center/hangar additions. Although included in the conservative calculation above, the existing maintenance shop and warehouse upgrades and the existing incinerator building would be expected to require little to no ground disturbance along each exterior wall. During construction of the maintenance building additions, approximately three birch stems (at 2.5” diameter), one-half dozen clumps of birch saplings (at 1" to 1.5" diameter stems and 10'-15' tall) and a dozen small poplar saplings (2-4 years old) would be cleared from the area. In the storage yard, approximately 10,600 square feet of previously disturbed soil would be excavated to construct the new warehouse, new incinerator and associated utility lines. Excavation, backfilling, and revegetation would be accomplished with a combination of hand tools and a diesel backhoe.

To accommodate the storage yard expansion, approximately 8,400 square feet of undisturbed black spruce woodland with a ground cover of moss and shrubs would be cleared. To complete this expansion, fewer than 14 spruce and birch trees (diameter 6” or greater), 25 spruce and birch trees (diameter 3”-5”) and 59 spruce and birch trees (1”-2” diameter) would be cleared from the area. The total vacant ground area of the completed, fenced storage yard would be approximately 21,100 square feet. It would have estimated rectangular dimensions of 138’ x 170’, minus a rectangle in the northeast corner of 65’ x 36’. This portion of the project would be done during the second year to allow ample time for development and implementation of a site rehabilitation plan.

Up to 300 cubic yards of soil material, primarily gravel and rock, would have to be removed from the immediate vicinity of the present visitor center/hangar to level the area and achieve necessary final grades. Some excavated material would be screened to provide approximately 50 cubic yards of aggregate to be used in mixing concrete for this project. The remaining excavated material would be hauled to various access paths and parking lots and spread to fill potholes and level eroded sections. Soil erosion along access paths would be reduced and smoother walking surfaces would reduce the risk of pedestrian injuries.

All utility lines for this project would be installed in previously disturbed ground, most of it in existing utility corridors. Approximately 200 linear feet of soil, to a depth of up to 24 inches, would be disturbed by the installation of electrical wiring. Approximately 170 linear feet of soil, at a depth of six to eight feet, would be disturbed by the installation of the one-inch water line and two-inch sewer line from the maintenance building to the existing, main water and sewer lines. For fuel service, approximately 260 linear feet of soil, to a depth of two feet, would be disturbed. At an average disturbance width of 20 feet for the sewer and water lines and 12 feet for the electrical and fuel lines, the total disturbed area for utility lines would be approximately 8,920 square feet.

A site rehabilitation plan would be developed and implemented that encompasses this project, as well as previously disturbed sites within the 15.61 administrative site. As much as possible, revegetation efforts would take place concurrently with construction activities. As would be outlined in this plan, efforts would be made to salvage viable groundcover plants and reduce storage time by expeditiously transplanting them to suitable sites. Mats of ground cover, birch seedlings, and the lowermost organic layer (primarily peat), would be salvaged and used to revegetate the areas surrounding new construction. This type of vegetation is common throughout the park and, as much as possible, the majority of salvageable material would be reused. The erosion control of past and the proposed project sites would be positively impacted, as the transplants took firm hold and landscape work was completed around the buildings. With the successful implementation of a site rehabilitation plan, long-term impacts to soils and vegetation at the administrative site would be expected to be minor.

Cumulative Impacts.  Construction and related ground disturbance activities from future NPS projects would be expected to result in minor, negative, short-term impacts to soils and vegetation in the administrative area. Since a site revegetation plan would be implemented with this alternative, ground disturbance from past projects would be addressed and corrected, resulting in a reduced, minor, negative impact. Outlying non-NPS future projects would be expected to have minor negative effects, partly mitigated by the use of best management practices and revegetation.

Conclusion.  Construction and ground disturbing activities would have a minor long-term impact of soils and vegetation in the area. Approximately 1.26 acres, or 8.1% of the administrative lot would be affected, but the majority of this area has previously been disturbed. The implementation of a site rehabilitation plan would mitigate effects to soils and vegetation from ground disturbing activities as well as from past projects. The level of impacts on soils and vegetation anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Wildlife.  The project site is inhabited and utilized primarily by passerine birds. Small mammals inhabit the surrounding forest. Construction activity associated with the proposed project would mainly occur over a period of two years during the months of June through September. Animals could be temporarily displaced due to noise and activities associated with facility construction. However, these minor impacts on wildlife in the area would be short-term. It is expected that any displaced wildlife would have no difficulty becoming established elsewhere on NPS or adjacent private property.

Construction of the incinerator and warehouse in the storage yard would disturb approximately 10,600 square feet of previously disturbed grassy ground habitat. The storage yard expansion would disturb approximately 8,400 square feet of undisturbed black spruce woodland and moss and shrub habitat to the west and south of the existing yard. Wildlife may use the east-west trail along the southern boundary of the administrative lot, as well as other trails parallel to this one further south. As much as possible, the storage yard fence boundary would be situated at least approximately 100 feet north of the southern boundary trail to minimize disruption to potential wildlife movement. Also, as much as possible, brush and trees in the undisturbed area of the proposed expansion would be cut after July 20th and before May 1st to avoid impacts to nesting birds. For these reasons, as well as the limited scope and area of the project, long-term wildlife impacts would be negligible.

Cumulative Impacts.  Construction activities for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would create a negative, minor, short-term impact to wildlife due to disturbance and habitat loss. However, because this area would represent a very small percentage of the available wildlife habitat in the region, negligible, long-term, negative effects to wildlife would be expected.

Conclusion.  Minor short-term and negligible long-term impacts to wildlife would be expected with this alternative from habitat loss and displacement of wildlife. The level of impacts on wildlife anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the parks enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Cultural Resources.  NPS cultural resource specialists conducted a field survey of the park administrative area in 1993. This survey revealed no evidence of archeological resources. Even so, cultural resource specialists would monitor the project site during the excavation activities. Should unknown cultural resources be uncovered during project implementation, work would be stopped in the discovery area. The NPS would perform consultations according to 36 CFR 800.11 and, as appropriate, provisions of the Native American Graves Protection and Repatriation Act of 1992. A Determination of Eligibility would be conducted. If adjustments could not be made to the project to avoid cultural resource disturbance, a Memorandum of Agreement (MOA) with the Advisory Council on Historic Preservation and the Alaska State Historic Preservation Office that incorporates comments from consulting parties would be executed. The MOA would specify measures to mitigate adverse effects. Any artifacts recovered from park property at the project site would be accessioned, cataloged, preserved, and stored in compliance with NPS Cultural Resource Management Guideline.

Since no cultural resources are known to be located on the park airstrip site, no impacts would be anticipated.

Cumulative Impacts.  No impacts would be expected from future actions to the NPS administrative lot, since no cultural resources have been known to be impacted from past activities. Construction activities on unsurveyed sites (for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities) would be regulated by state and federal requirements for cultural resource protection. Any impacts to unsurveyed areas would need to be evaluated on a site and project-specific basis.

Conclusion.  No impacts would be expected to occur on the administrative lot under this alternative. The level of impacts on cultural resources anticipated from this alternative would not result in impairment of park resources that fulfill specific purposes identified in the park’s enabling legislation or that are key to the natural and cultural integrity of the park and preserve.

Visitor Use.  Visitors to the park administrative site could observe or hear construction and excavation activities, resulting in a negative, minor impact to the visitor experience. Visitors to the park backcountry would be unaffected by construction activities.

Moderate, long-term beneficial effects would be expected, however, since the project would provide substantially improved visitor facilities. The NPS Organic Act and NPS  Management Policies 2001 direct the NPS to provide for visitor use and enjoyment of park resources. By moving the visitor center to a new larger, more visually attractive facility, safely away from fixed wing aircraft and helicopter operations, providing public restrooms, and moving the incinerator away from areas of public use, visitor safety, comfort, and enjoyment would be enhanced.

Cumulative Impacts.  Construction activities for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would create minor, short-term disturbances for visitors. However, long-term, positive effects to visitors would be expected. With the construction of a new state airport, aviation safety and park visitation numbers would be expected to improve, as visitors were afforded greater transportation reliability and safety to and from Port Alsworth.

Conclusion.  This alternative would result in minor, negative, short-term effects during construction as park visitors would be able to observe and hear construction activities which could degrade the visitor experience. Even so, moderate long-term benefits would be expected with substantially improved visitor facilities.

Park Management. The proposed action is in general accordance with the 1994 Site Facility Survey and Development Plan. The new visitor contact facility would be approximately 1,300 square feet and the maintenance building would be expanded to approximately 3,500 square feet. The location of the new maintenance facility would be in approximately the same location as that identified in the plan. However, since the field headquarters offices were relocated in 1998-1999 to a site different than that shown on the plan, the decision has been made to locate the visitor contact facility near the offices, as the plan recommended, rather than in the exact geographic area indicated.

Providing a larger facility for maintenance activities would enable park operation, maintenance and repair of equipment and facilities to be accomplished in a more efficient and safe manner than is currently possible. Park maintenance staff would be provided with toilet and shower facilities. Numerous existing safety and building code violations would be eliminated.

Moving the park incinerator to a larger building away from both non-maintenance park activities and the visiting public would improve staff and visitor safety and enhance the aesthetics of the site. The park would be better able to effectively reduce its solid waste stream through segregating and recycling large portions of the trash generated by daily park operations.

Providing a new warehouse would greatly increase the ability of the park to properly store equipment and supplies vital for operations in both Port Alsworth and the backcountry. This would protect the investment represented by this material and allow park staff and outside researchers to accomplish more work with greater efficiency.

The new visitor center would provide adequate storage space available for collections, visitor contact information and library materials. Ample meeting space would be available for interpreters to serve school groups that visit the park. Interpreters would have adequate office space to assemble and store interpretive programs and displays. Collections of cultural resource materials would be safely stored in a secure area without risk of deterioration of valuable items. Proper collections storage would allow some  future collections to be retained at the park, without the need for transport to storage locations off-site. The new visitor center would offer the park a large meeting space with a public restroom to hold public meetings and gatherings within the headquarters area.

The expansion and improvements to the storage yard would consolidate maintenance functions such as waste handling, recycling and the staging and storage of materials. Long-term property management needs would be satisfied with adequate space for storage of vehicles and boats. Vehicles and boats, currently stored in several locations within the NPS administrative lot, would be moved into the storage yard for improved park operations and security. Storage yard expansion and fencing would be scheduled during the second year of the project to allow sufficient time to prepare and implement a site rehabilitation plan. Clearing of brush and trees would take place primarily after July 20th and prior to May 1st to avoid impacts to nesting birds.

This alternative would result in a short-term minor impact in park management due to disruption during construction activities. However, on a long-term basis, the proposed project would positively affect normal day-to-day NPS operations to a moderate degree. Park maintenance staff would be able to better perform routine and special project maintenance and repair activities in a safer environment. Park interpretive staff would be better able to share the park experience by utilizing the new, expanded visitor contact facility. All park operations would benefit from the expansion of heated storage areas. NPS staff residing at the airstrip site would not be expected to experience an appreciable impact to their living conditions. Operation of the expanded facilities would not require any baseline increase in park staff, and would not place a burden on current staff time and resources or on the park's budget.

Cumulative Impacts.  Construction activities for a new NPS hangar and housing units and the proposed outlying state airport and village waste transfer facilities would create minor, short-term disturbances during construction, but minor, positive long-term benefits for aviation safety, employee housing and waste handling.

Conclusion.  Some short-term minor impacts to park management functions would be expected due to disruption during construction activities. However, this alternative would improve park management functions by enabling park operations to be accomplished in a more efficient and safe manner than is currently possible.

# 5.0  CONSULTATION AND COORDINATION

Persons, Organizations and Agencies Contacted

The following agencies, organizations, and individuals were consulted in the preparation of this document.

U.S. Fish and Wildlife, Ecological Services:

#### Greg Balough, Branch Chief, was contacted by telephone on April 3, 2003 for Endangered Species Act, Section 7 consultation.

State of Alaska, Division of Governmental Coordination, Alaska Coastal Zone Management Program (ACMP):

Cynthia Zuelow-Osborne, ACMP Project Review Coordinator, was contacted by telephone on April 1, 2003 for consultation regarding requirements under the ACMP.

# Lake and Peninsula Borough, ACMP District Coordination:

#### Coastal Zone District Representative Marvin Smith was contacted by telephone on April 3, 2003 for consultation regarding requirements under the District ACMP and on May 2, 2003 for information regarding future development projects in the Port Alsworth area.

## 6.0  LIST OF PREPARERS AND CONTRIBUTORS

Helen Lons, Biologist, Katmai National Park and Preserve

#### Glen Yankus, Environmental Protection Specialist, NPS Alaska Support Office

#### Paul Button, Chief of Maintenance, Lake Clark National Park and Preserve

#### Penny Knuckles, Chief of Resource Management, Lake Clark National Park and Preserve

Lee Fink, Chief of Operations, Lake Clark National Park and Preserve

Mark Mullins, Maintenance Supervisor, Lake Clark National Park and Preserve

Mary McBurney, Subsistence Program Manager, Lake Clark National Park and Preserve

Dale Vinson, Cultural Resources Compliance Officer, Lake Clark National Park and Preserve

# 7.0  REFERENCES

Alaska Department of Community and Economic Development

2003	Alaska Community Database – Community Information Summary.

http://www.dced.state.ak.us/mra/CF_CIS.cfm.

National Park Service. 1999.  Resources Management Plan, Lake Clark National Park and Preserve, Port Alsworth, Alaska.

National Park Service. 1994.  Site Facility Survey and Site Development Plan, Lake Clark National Park and Preserve, Port Alsworth, Alaska.

U.S. Fish and Wildlife Service.  Letter dated November 15, 1994 to Ralph Tingey, Superintendent, Lake Clark National Park and Preserve from Mr. Jonathan V. Hall, Regional Wetlands Coordinator - Alaska, U.S. Fish and Wildlife Service.

National Park Service. 1984.  General Management Plan, Environmental Assessment, Lake Clark National Park and Preserve, Alaska, Denver Service Center.

APPENDIX A

SUMMARY EVALUATION AND FINDINGS

ANILCA, SECTION 810(a)

Appendix A

Alaska National Interest Land Conservation Act (ANILCA)

Section 810 (a) Summary Evaluations and Findings

BACKGROUND

Subsistence uses, as defined by the Alaska National Interest Land Conservation Act (ANILCA), section 803, means "the customary and traditional uses by rural Alaska residents of wild, renewable resources for direct personal or family consumption as food, shelter, fuel, clothing, tools, or transportation; for the making and selling of handicraft articles out of non-edible byproducts of fish and wildlife resources taken for personal or family consumption; for barter, or sharing for personal or family consumption; and for customary trade." Subsistence activities include hunting, fishing, trapping, and collection of berries, edible plants, and wood or other materials.

Subsistence uses are allowed within Lake Clark National Preserve in accordance Titles II and VIII of the ANILCA. Lands and waters within Lake Clark National Park are available to subsistence uses.

I.  INTRODUCTION

This section was prepared to comply with Title VIII, Section 810 of the ANILCA. It summarizes the evaluations of potential restrictions to subsistence uses that could result from the proposed action by the National Park Service (NPS) to upgrade and expand visitor contact and maintenance facilities at the field headquarters site for Lake Clark National Park and Preserve in Port Alsworth, Alaska.

###### II.  EVALUATION PROCESS

Section 810(a) states:

“In determining whether to withdraw, reserve, lease, or otherwise permit the use, occupancy, or disposition of public lands...the head of the federal agency... over such lands ... shall evaluate the effect of such use, occupancy, or disposition on subsistence uses and needs, the availability of other lands for the purposes sought to be achieved, and other alternatives which would reduce or eliminate the use, occupancy or disposition of such lands which would significantly restrict subsistence uses shall be effected until the head of such Federal agency–

(1) gives notice to the appropriate State agency and the appropriate local committees and      regional councils established pursuant to Section 805;

(2) gives notice of, and holds, a hearing in the vicinity of the area involved; and

(3) determines that (A) such a significant restriction of subsistence uses is necessary, consistent with sound management principles for the utilization of the public lands, (B) the proposed activity would involve the minimal amount of public lands necessary. . . and (C) reasonable steps would be taken to minimize adverse impacts upon subsistence uses and resources resulting from such actions.”

The ANILCA created new units and additions to existing units of the national park system in Alaska.  Lake Clark National Park and Preserve was created by the ANILCA Title II Section 201(7)(a) for the following purposes:

“To protect the watershed necessary for perpetuation of the red salmon fishery in Bristol Bay; to maintain unimpaired the scenic beauty and quality of portions of the Alaska Range and the Aleutian Range, including active volcanoes, glaciers, wild rivers, lakes, waterfalls, and alpine meadows in their natural state; and to protect habitat for and populations of fish and wildlife including but not limited to caribou, Dall sheep, brown/grizzly bears, bald eagles, and peregrine falcons.”

The potential for significant restriction of subsistence uses must be evaluated for the proposed action’s effect upon “...the availability of other lands for the purposes sought to be achieved and other alternatives which would reduce or eliminate the use, occupancy, or disposition of public lands needed for subsistence purposes.” (Section 810 (a), ANILCA).

###### III.  PROPOSED ACTION ON FEDERAL PUBLIC LANDS

The NPS is considering facility improvements at the Lake Clark field headquarters site in Lake Clark National Park and Preserve. The proposed project would entail expansion and upgrade of the visitor contact and maintenance facilities at the developed “airport” lot adjacent to the runway in Port Alsworth. The maintenance function would be transferred from the existing maintenance shop building to the existing visitor center/hangar. Visitor contact functions would be transferred from the existing visitor center/hangar to the existing maintenance shop building. The existing visitor center/hangar footprint would be expanded to approximately double its present size. The interiors of both buildings would be renovated and upgraded to better serve their new functions.

In addition, the park’s incinerator would be moved from its current location near the airstrip to a new building to be constructed in the present storage yard (immediately south of the park sewage lagoon), away from areas normally frequented by visitors and non-maintenance  staff.  A new warehouse would be constructed in the storage yard. The cleared area that constitutes the storage yard would be slightly expanded to replace the storage area occupied by the two new buildings. The entire storage yard area would be fenced. Utility work related to these improvements would include providing water and sewer service to the new maintenance building and new visitor center. Underground fuel lines would be installed for the new incinerator building and the existing and new warehouses. Underground electrical service would be installed to the new incinerator building and new warehouse.

# IV.  AFFECTED ENVIRONMENT

A summary of the affected environment pertinent to subsistence uses is presented here.

Lake Clark National Park and Preserve is located in southcentral Alaska adjacent to Cook Inlet and was established in 1980 by Title II Section 201(7) of the ANILCA. Subsistence uses are allowed within Lake Clark National Park and Preserve in accordance with Title II, Section 201(1) and Title VIII of the ANILCA. Section 803 of the ANILCA defines subsistence uses as:  “the customary and traditional uses by rural Alaska residents of wild, renewable resources for direct personal or family consumption as food, shelter, fuel, clothing, tools, or transportation; for the making and selling of handicraft articles out of non-edible by-products of fish and wildlife resources taken for personal or family consumption; for barter, or sharing for personal or family consumption; and for customary trade.”

Located in Game Management Units (GMU) 9A, 9B, 16B and 17B, Lake Clark National Park (which encompasses 2,439,000 acres) and Preserve (which encompasses 1,214,000 acres) contains exceptional geologic features, scenery, wildlife, and human history. These GMUs also include other federal public lands such BLM administered lands in 9B, 16B and 17B; the Denali National Park and Preserve in 16B; and the Upper Mulchatna Controlled Use Area in 17B. The site of the proposed action is located in GMU 9B, the Kvichak River drainage, and within the boundaries of Lake Clark National Park and Preserve.

In accordance with Title 36 CFR Part 13 regulations, residents of the NPS designated resident zone communities of Iliamna, Lime Village, Newhalen, Nondalton, Pedro Bay and Port Alsworth and residents residing within Park boundaries are qualified to engage in subsistence uses within Lake Clark National Park and Preserve. Local rural residents who do not live in these communities or in the Park, but who have customarily and traditionally engaged in subsistence activities within the park and preserve may continue to do so with a subsistence use permit issued by the park superintendent.

Major resources used by resident zone communities for subsistence are caribou, brown bear, moose, beaver, Dall sheep, snowshoe hare, fox, lynx, mink, wolf, wolverine, ptarmigan, waterfowl, otter, marine mammals, salmon, trout, Dolly Varden, grayling, pike, longnose suckers, humpback and round whitefish, halibut, crab, clams, berries, wild edible plants, and wood.

Federal subsistence fishery regulations currently allow residents of the Iliamna-Lake Clark drainage to subsistence fish for salmon and other freshwater fish for customary and traditional uses. Federal game regulations for GMU 9B allow residents of resident zone communities to harvest black bears, brown bears, caribou, Dall sheep, moose, coyotes, muskrats, beaver, red foxes, arctic foxes, lynx, wolves, otters, wolverine, marten, weasels, marten, mink, grouse and ptarmigan.

The following harvest figures represent rough estimates extrapolated from several sources for a relatively typical year for the region and the 1983 estimates for the park and preserve (ADF&G).

Subsistence Resources			Annual					1983 Estimated

Portion					Regional Harvest			Park/Preserve

Bears					107 animals				10 animals

Moose					164 animals				65 animals

Caribou					179 animals				100 animals

Dall sheep				107 animals				7 animals

Furbearers				2,421 animals				530 animals

Small game				1,786 animals				1,200 animals

Waterfowl				1,750 animals				930 animals

Fish					164,286 fish				16,560 fish

Berries					67,429 pounds				7,200 pounds

Plants					16,186 pounds				2,000 pounds

Firewood				1,321 cords				75 cords

House logs				714 logs				100 logs

“Bears” includes both brown and black bears. “Furbearers” include beaver, coyote, ground and red squirrel, land otter, lynx, marten, mink, muskrat, red fox, weasel, wolverine, and wolf. “Small game” includes gallinaceous birds such as rock and willow ptarmigan and spruce grouse, as well as porcupines and snowshoe hares. “Waterfowl” include bufflehead, eider, golden-eye, harlequin, mallard, old-squaw, pintail, scaup, scoter, green-wing teal, wigeon, cranes, geese and swans. “Fish”–in approximate order of importance– include sockeye salmon, rainbow trout, Dolly Varden, whitefish, grayling, northern pike, burbot, candlefish (least cisco) and longnose suckers (Stickman et al., 2003) “Berries” include blueberries, high and low bush cranberries, nagoon berries, bear berries and salmonberries. “Plants” include Labrador tea, rose hips and other edible plants. “Firewood” refers to spruce, birch and cottonwood cut into cords for home heating. These wood species, in addition to willows and alders, are also used for crafts. “House logs” are primarily white spruce.

A subsistence harvest in any given year may vary considerably from one year to the next due to spatial and temporal factors and natural causes such as weather, climate change and natural population cycles. The primary species taken for subsistence are moose, caribou, fish (primarily sockeye salmon) and berries. By weight in pounds, the overall subsistence pattern of the region is characterized by the following proportions (ADF&G, 1983):

Edible Subsistence Resource			Percentage

Bears					       1

Moose					       35

Caribou					       10

Dall Sheep				        3

Furbearers				        5

Small Game				        2

Waterfowl				        2

Fish					        20

Berries					        15

Plants					        7

TOTAL				      100

The Port Alsworth area lies within the boundaries of GMU 9B and within the preserve boundary of Lake Clark National Park and Preserve. Several studies of subsistence use in the area include the Final Environmental Statement for the Proposed Lake Clark National Park (NPS); the park general management plan; Resource Use and Subsistence in the Vicinity of the Proposed Lake Clark National Park (Behnke 1978); Subsistence Production and Exchange in the Iliamna Lake Region, Southwest Alaska, 1982-1983 (Morris 1983), and Nondalton Traditional Ecological Knowledge of Freshwater Fish (Stickman et al., 2003).

## Fishing is the primary subsistence activity in the immediate Port Alsworth area and salmon is the predominant fish harvested for subsistence. Harvest information collected by the Alaska Department of Fish and Game Commercial Fisheries Division in 2000 reported that the community took a total of 3,200 sockeye salmon for subsistence.

In addition, over three-quarters of Port Alsworth households hunt large land mammals including moose, caribou, bears and Dall sheep. Over one-half the households pick berries and nearly one-third rely on wood resources for fireweed and house logs.

V.  SUBSISTENCE USES AND NEEDS EVALUATION

To determine the potential impact on subsistence activities, three evaluation criteria were analyzed relative to current subsistence resources that could be impacted.

The evaluation criteria are:

- The potential to reduce important subsistence fish and wildlife populations by (a) reductions in abundance; (b) redistribution of subsistence resources; or (c) loss of habitat.

2.    Potential impacts the action may have on access for subsistence hunters and fishermen.

- The potential for the action to increase competition among hunters and fishermen for

subsistence resources.

1.  The potential to reduce populations:

There should be no significant reduction in populations of subsistence fish and wildlife resources as a result of the proposed action. For the most part, fishing, berry picking and trapping are the only subsistence activities that take place in the immediate vicinity of Port Alsworth. The proposed construction should have no long-term effect on fish populations, and should have no impact on the availability and overall abundance of berries in the area.

Due to the small area of previously undisturbed ground that would be impacted by this project and the fact that the construction is proposed to occur in a previously developed area, the potential to impact wildlife is very small. The occasional hunting and trapping activities that occur in the vicinity should not be affected.

The proposed project is not expected to alter subsistence habitats or result in any measurable reduction in or redistribution of wildlife or other subsistence resources. Provisions of the ANILCA, the Federal Subsistence Board, and NPS regulations provide the tools for adequate protection of fish and wildlife populations within Lake Clark National Park and Preserve while ensuring a subsistence priority for local rural residents. In addition, the superintendent may enact closures and/or restrictions if necessary to protect subsistence opportunities or to assure the continued viability of a particular fish or wildlife population.

2.  Restriction of Access:

All rights of access for subsistence harvest on NPS lands are granted by Section 811 of the ANILCA. Lake Clark National Park and Preserve are managed according to legislative mandates, NPS management policies and guidelines within the approved Lake Clark National Park and Preserve General Management Plan. The proposed action is not expected to limit or restrict the access of subsistence users to natural resources within the park or preserve. The superintendent may enact closures and/or restrictions if necessary to protect subsistence opportunities or to assure the continued viability of a particular fish or wildlife population.

- Increase in Competition:

The proposed action is not expected to result in increased competition for fish, wildlife or other resources that would significantly impact subsistence users. NPS regulations and provisions of the ANILCA mandate that if and when it is necessary to restrict taking of fish or wildlife, subsistence users are given a priority over other user groups. Continued implementation of the ANILCA provisions should mitigate any increased competition from resource users other than subsistence users. The superintendent may enact closures and/or restrictions if necessary to protect subsistence opportunities or to assure the continued viability of a particular fish or wildlife population.

# VI.  AVAILABILITY OF OTHER LANDS

The nature of the proposed action is site-specific to the Port Alsworth field headquarters. Since there are no other land inholdings available within the project area, no other lands are suitable for the project. The proposed action is consistent with NPS mandates and the General Management Plan and is not expected to impact subsistence uses.

# VII.  ALTERNATIVES CONSIDERED

No other alternatives were considered in this analysis, as the nature of the proposed action is both site and project-specific. Subsistence users also utilize other Federal, State and private lands within the region.

###### VIII.  FINDINGS

This analysis concludes that the proposed action would not result in a significant restriction of subsistence uses.