Finding of No Significant Impact

Expansion and Upgrade of Port Alsworth Visitor Contact and Maintenance Facilities

Lake Clark National Park and Preserve

June 2003

The National Park Service (NPS) has prepared an environmental assessment (EA) that evaluates the expansion and upgrade of visitor contact and maintenance facilities at the headquarters site for Lake Clark National Park and Preserve (LACL) in Port Alsworth, Alaska. The project will entail the expansion and upgrade of the visitor contact and maintenance facilities at the developed “airstrip” administrative site adjacent to the airstrip in Port Alsworth. The maintenance function will be transferred from the existing maintenance shop to the existing visitor center/hangar. Visitor contact functions will be transferred from the existing visitor center/hangar to the existing maintenance shop. The existing visitor center/hangar footprint will be expanded to approximately double its present size. The interiors of both buildings will be renovated and upgraded to better serve their new functions.

In addition, the park’s incinerator will be moved from its current location near the access road and airstrip to a new building to be constructed in the present storage yard. A new warehouse will be constructed in the storage yard. The cleared area that constitutes the storage yard will be expanded to replace the outdoor storage space that will become occupied by the two new buildings and to provide sufficient storage and staging areas for boats, vehicles and materials. The entire storage yard will be fenced. Utility work related to these improvements will include providing water, sewer and fuel service to the new maintenance building and new visitor center. An underground fuel line will be installed for the existing warehouse. Fuel service to the new warehouse and the new incinerator will be provided from a new, shared 500-gallon double-wall aboveground tank with new underground piping to each building. Underground electrical service will be installed for the new incinerator building and new warehouse.

The purpose of this proposed project is to (1) improve the park’s visitor contact facilities to allow greater visitor satisfaction and (2) improve the park’s maintenance facilities to better accommodate the growing needs of park programs and satisfy regulatory health and safety requirements.

The proposed improvements are needed for several reasons. The current visitor contact room in the existing visitor center/hangar building is unfinished, poorly lit and crowded. It can only accommodate a few visitors at a time and there is no room for visitor groups to sit and rest. The visitor center has inadequate storage space available for collections, visitor contact information and library materials. Presently, no public restroom is available for visitors within the visitor center/hangar building. Visitors must walk uphill approximately 50 yards to use a single outhouse with no running water. No meeting space is available for interpreters to serve school groups that visit the park. Interpreters have no office space to assemble and store interpretive programs and displays. Collections of cultural resource materials cannot be safely stored in a secure area, risking rapid deterioration of valuable items. Lack of proper collections storage at the park limits future on-site collections, as they must be transported to safe storage locations off-site. The park has no finished, large meeting space with a public restroom to hold public meetings and gatherings within the headquarters area.

The existing maintenance shop does not comply with current building utility codes and OSHA regulations regarding workplace safety. Maintenance employees have no restroom or safety shower facilities in the shop. The existing workspace and storage areas are inadequate to accommodate the minimal requirements of park facility operation and maintenance programs. The incinerator facility is located in close proximity to the airstrip and access road. Due to crowding, bags of recyclable materials are stored outside of the incinerator building, creating an unsightly distraction for park visitors who walk the area footpaths.

## Public Involvement

The EA had a 30-day public comment period beginning May 16, 2003 and ending June 14, 2003. The park mailed the EA to 32 agencies, organizations and individuals. Comments were received from the State of Alaska Office of the Governor ANILCA Implementation Program, the National Marine Fisheries Service (NMFS), and the National Parks Conservation Association (NPCA).

The State ANILCA Implementation Program submitted comments by the Alaska Department of Transportation and Public Facilities (ADOT&PF). The ADOT&PF has no specific concerns with the scope or overall purpose and need for the project as outlined in the EA. The ADOT&PF offered an updated status of the Department’s future projects in Port Alsworth. The ADOT&PF comments added more specific planning information for cumulative impacts in the region, but did not change the EA’s conclusions about the effect of the alternatives.

The NMFS stated that the described action will not result in any adverse effect to essential fish habitat (EFH), no EFH Assessment or EFH Conservation Recommendations are required and the NMFS has no objection to the project.

The NPCA supported the NPS plan to expand and upgrade the visitor contact and maintenance facilities. The NPCA noted that the new visitor contact facility would satisfy existing shortcomings and is clearly needed to improve the park visitor experience. They also stated that the new maintenance facility and the relocation of the incinerator would greatly increase worker and visitor safety, numerous safety and building code violations would be eliminated and the visitor experience would be improved. The NPCA emphasized that construction should be limited as much as possible to the previously developed footprint in the area. The NPS concurs and has been consolidating primary park functions at the administrative site, as described in the EA.

# Alternatives

The EA evaluated two alternatives; the Preferred Alternative and a No Action Alternative. Under the No-Action Alternative, LACL would continue to operate and maintain the existing visitor contact and maintenance facilities at the airstrip site in Port Alsworth. No facility expansions or upgrades would be completed.

## Mitigating Measures

Mitigation to be taken in conjunction with implementing the NPS Preferred Alternative include:

Cultural Resources.  Cultural resource specialists will monitor the project site during excavation activities. Should unknown resources be uncovered during project implementation, work will be stopped in the discovery area and the NPS will consult according to 36 CFR 800.11 and, as appropriate, provisions of the Native American Graves Protection and Repatriation Act of 1992. Any artifacts recovered from park property will be accessioned, cataloged, preserved, and stored in compliance with the NPS Cultural Resource Management Guideline.

Soils and Vegetation.  A site rehabilitation plan will be developed for the proposed project area. This plan will include the disturbed area in this project as well as the disturbed areas from previous NPS projects. No non-native species will be introduced into the project area. The project site will be revegetated using native plant materials.

The storage yard expansion and fencing project elements will be done during the second year to allow ample time for rehabilitation plan development and implementation. Excavation, backfilling, and revegetation will be accomplished with minimum equipment; a combination of hand tools and a diesel backhoe. Whenever possible, cut trees will be used as firewood and stumps will be pulled and burned on site. Organic debris will be burned or buried on site. In the undisturbed and/or heavily vegetated areas affected, mats of ground cover, birch seedlings, and the lowermost organic layer (primarily peat), will be salvaged and used to revegetate the areas surrounding new construction.

Wildlife.  As much as possible, brush and trees in the undisturbed area of the proposed expansion will be cut after July 20th and before May 1st to avoid impacts to nesting birds.

# Environmentally Preferred Alternative

The No-Action Alternative is the environmentally preferred alternative because it would cause no further damage to the biological or physical environment. Continued use of existing facilities under the No Action Alternative would continue negative impacts to visitor use. Crowded and inadequate visitor facilities and visitor access to maintenance functions would continue to detract from the visitor experience. This alternative would continue to negatively impact park management operations, both in the short-term and long-term as a result of inadequate and unsafe visitor and maintenance facilities.

## Environmental Consequences of the Preferred Alternative

As documented in the EA, the NPS has determined the Preferred Alternative can be implemented with no significant adverse effect to aesthetics, air quality, soundscape, soils and vegetation, wildlife, cultural resources, visitor use, and park management. The environmental effects of the Preferred Alternative are summarized below.

Aesthetics.   During construction and excavation activities over the course of the two-year project, the administrative lot will exhibit short-term, minor, negative impacts to aesthetics. The additional structures will give the area a more developed appearance. However, aesthetics will be expected to improve at the administrative site over two to four years as a revegetation plan is implemented.

Air Quality.  During June to September of 2003 and 2004, construction activities associated with this alternative will generate dust and machinery emissions that will have a short-term negligible impact on air quality. No long-term impacts to air quality will occur at the park administrative site.

Soundscape.  Noise levels from construction machinery at the project site will cause a negative, negligible, short-term impact on the natural soundscape. Over the long-term, facility operations and visitor use of the area will create low levels of noise that will have a negligible effect on the natural soundscape.

Soils and Vegetation.  This project will result in minor, short-term and long-term negative impacts to soils and vegetation in the project area. A total ground disturbance of approximately 54,870 square feet, or 1.26 acres (about 8.1% of the administrative site), will occur. This consists of new and upgraded buildings (24,846 square feet), utilities (8,920 square feet), existing storage yard (12,700 square feet) and storage yard expansion (8,400 square feet). Even so, all but approximately 8,400 square feet of this disturbance will occur in soil that has previously been disturbed. All utility lines for this project will be installed in previously disturbed ground, most of it in existing utility corridors. No non-native species will be introduced into the project area. Soils will be disturbed during excavation for the new buildings, additions and utility lines. Ground disturbance will be limited to a maximum buffer of twenty feet around the new incinerator building, new maintenance warehouse and existing visitor center/hangar additions.

To accommodate the storage yard expansion, approximately 8,400 square feet of undisturbed black spruce woodland with a ground cover of moss and shrubs will be cleared. This portion of the project will be done during the second year to allow ample time for development and implementation of a site rehabilitation plan. With the implementation of a site rehabilitation plan, long-term impacts to soils and vegetation at the administrative site will be minor.

Wildlife.  Minor short-term and negligible long-term impacts to wildlife will occur with this project from habitat loss and displacement of wildlife. Animals could be temporarily displaced due to noise and activities associated with facility construction. However, these minor impacts on wildlife in the area will be short-term. It is expected that any displaced wildlife will have no difficulty becoming established elsewhere on NPS or adjacent private property.

As much as possible, the storage yard fence boundary will be situated at least approximately 100 feet north of the southern boundary trail to minimize disruption to potential wildlife movement. Also, as much as possible, brush and trees in the undisturbed area of the proposed expansion will be cut after July 20th and before May 1st to avoid impacts to nesting birds. For these reasons, as well as the limited scope and area of the project, long-term wildlife impacts will be negligible.

Cultural Resources.  NPS cultural resource specialists conducted a field survey of the park administrative area in 1993. This survey revealed no evidence of archeological resources. Even so, cultural resource specialists will monitor the project site during the excavation activities. Since no cultural resources are known to be located on the park airstrip site, no impacts are anticipated.

Visitor Use.  This alternative will result in minor, negative, short-term effects during construction as park visitors will be able to observe and hear construction activities, which could degrade the visitor experience. Even so, moderate long-term benefits will be expected with substantially improved visitor facilities.

Park Management.  Some short-term minor impacts to park management functions will occur due to disruption during construction activities. However, this alternative will improve park management functions by enabling park operations to be accomplished in a more efficient and safe manner than is currently possible.

## Decision

The NPS decision is to select the Preferred Alternative, which will involve the expansion and upgrade of the visitor contact and maintenance facilities at the headquarters site for LACL in Port Alsworth, Alaska. This decision includes mitigating measures on cultural resources, soils and vegetation, and wildlife.

### Rationale for the Decision

Construction activities associated with the Preferred Alternative will result in negligible to minor, negative, short-term impacts to aesthetics, air quality, soundscape, soils and vegetation, wildlife, visitor use and park management. However, over the long-term, this project will improve aesthetics, visitor use and park management at LACL.

The levels of adverse impacts to park resources anticipated from the Preferred Alternative will not result in an impairment of park resources that fulfill specific purposes identified in the establishing legislation or that are key to the natural or cultural integrity of the park.

Though it is the environmentally preferred alternative, the No Action was not selected for implementation, because it would not meet the purpose of the project as described above.

The Preferred Alternative complies with the Endangered Species Act, the National Historic Preservation Act, and Executive Orders 11988 and 11990. There will be no significant restriction of subsistence activities as documented by the Alaska National Interest Lands Conservation Act, Title VIII, Section 810(a) Summary Evaluation and Findings.

I find that the Preferred Alternative does not constitute a major federal action significantly affecting the quality of the human environment. Therefore, in accordance with the National Environmental Policy Act of 1969 and regulations of the Council on Environmental Quality (40 CFR 1508.9), an environmental impact statement will not be prepared for the project.

Recommended:  _____________________________________		 ________________

Deborah O. Liggett					 Date

Superintendent, Lake Clark National Park and Preserve

Approved:  _____________________________________		 ________________

Robert L. Arnberger					 Date

Regional Director, Alaska