##### National Park Service

U.S. Department of the Interior

Glacier National Park

Montana

Belly River Utilities Improvements

Environmental Assessment /Finding of Effect

May 2006

## Environmental Assessment and

Finding of Effect

_____________________________________________________________________________

Belly River Utilities Improvements

Glacier National Park • Montana

Summary

Improvements to the existing utilities at the Belly River Ranger Station Historic District are proposed. Two alternatives are analyzed: 1) a No Action Alternative which would maintain the current utility conditions, and 2) a Preferred Alternative which would make improvements to the water, sewer, and propane systems. Improvements would entail trenching and replacing approximately 1200 feet of water line and 480 feet of sewer line. In addition, a new water intake filtration system, new septic system with leach field, and 40 feet of new propane lines would be installed. An excavator would be flown in via helicopter for trenching. Up to six helicopter flights would be required to deliver needed materials including the excavator.

The resources determined to potentially be affected by the alternatives are Historic Buildings, Archeological Resources, Soils, Wildlife, Special Status Species, Water Quality, Natural Sounds, Recommended Wilderness, Visitor Experience, and Health and Safety. The No Action Alternative would have no impacts on Archeological Resources or Natural Sounds, and minor, adverse impacts to Historic Buildings, Wildlife, Special Status Species, and Recommended Wilderness. Visitor Experience, Soils, Water Quality, and Health and Safety would be adversely affected at the moderate level from the No Action Alternative. The Preferred Alternative would result in negligible, adverse impacts to Archeological Resources and minor, adverse effects to Wildlife, and Special Status Species. Moderate, adverse impacts to Soils and Recommended Wilderness would result from the Preferred Alternative. There would be minor, beneficial impacts to Historic Buildings, Soils, Wildlife, Visitor Experience, and moderate, beneficial impacts to Water Quality and Health and Safety from the Preferred Alternative.

Public Comment

Please provide comments and concerns on this environmental assessment (EA) through the Park’s planning website (http://parkplanning.nps.gov/parkHome.cfm?parkId=61) by selecting this project. Or write to: Superintendent, Glacier National Park, Attn: Belly River Utility EA, West Glacier, Montana 59936. This EA will be on public review for 2 weeks. Comments are due June 9, 2006. Please note that names and addresses of people who comment become part of the public record. If you wish us to withhold your name and/or address, you must state this prominently at the beginning of your comment. We will make all submissions from organizations, businesses, and from individuals identifying themselves as representatives or officials of organizations or businesses available for public inspection in their entirety.

Table of Contents

Summary	i

Introduction	1

Background	1

Purpose and Need	1

Public Involvement (Scoping)	2

Relationship of the Proposed Action to Previous Planning Efforts	4

Impact Topics	4

Impact Topics Considered	4

Topics Eliminated from Detailed Study	5

Alternatives Considered	8

No Action Alternative	8

Preferred Alternative (Proposed Action)	8

Alternatives Considered But Eliminated	10

Environmentally Preferred Alternative	11

Summary	11

Affected Environment and Environmental Consequences	13

Summary of Consultation and Compliance with Federal and State Regulations	31

Preparers and Consultants	31

List of Environmental Assessment Recipients	31

Summary of Compliance with Federal and State Regulations	32

References	33

Appendix A: Special Status Wildlife Species in Glacier National Park	Error! Bookmark not defined.

# Introduction

Background

Glacier National Park is located on the Canadian border in the northwestern section of Montana. The park is in the northern Rockies, and contains the rugged mountains of the Continental Divide. Together with Canada’s Waterton National Park, it forms the Waterton-Glacier International Peace Park, which is listed as a World Heritage Site and Biosphere Reserve. Outstanding natural and cultural resources are found in both parks.

Glacier National Park is an investment in the heritage of America. Its primary mission is the preservation of natural and cultural resources, ensuring that current and future generations have the opportunity to experience, enjoy, and understand the legacy of Waterton-Glacier International Peace Park.

The purpose of Glacier National Park is to:

- preserve and protect natural and cultural resources unimpaired for future generations (1916 Organic Act);

- provide opportunities to experience, understand, appreciate, and enjoy Glacier National Park consistent with the preservation of resources in a state of nature (1910 legislation establishing Glacier National Park); and

- celebrate the on-going peace, friendship, and goodwill among nations, recognizing the need for cooperation in a world of shared resources (1932 International Peace Park legislation).

Glacier’s significance is explained relative to its natural and cultural heritage:

- Glacier’s scenery dramatically illustrates an exceptionally long geological history and the many geological processes associated with mountain building and glaciation;

- Glacier offers relatively-accessible, spectacular scenery and an increasingly rare primitive wilderness experience;

- Glacier is at the core of the “Crown of the Continent” ecosystem, one of the most ecologically intact areas remaining in the temperate regions of the world;

- Glacier’s cultural resources chronicle the history of human activities (prehistoric people, Native Americans, early explorers, railroad development, and modern use and visitation) and show that people have long placed high value on the area’s natural features; and

- Waterton-Glacier is the world’s first international peace park.

The proposed project would occur at the Belly River Ranger Station Historic District (Map 1). This District is located in the backcountry of the Goat Haunt-Belly River Geographic Area, in the northeastern section of the park. The site is at an elevation of 4640 feet approximately five miles from the Canadian border. The Ranger Station and associated buildings are located on the eastern edge of a large meadow which is fenced and used to pasture horses and mules in the summer. The Ranger Station predates the establishment of Glacier National Park (1910) and is the only ranger station in the park not accessible by road.

Purpose and Need

The purpose of this project is to address the inefficient, inadequate, and unsafe utility systems at the Belly River Ranger Station Historic District. This Ranger Station has served backcountry trail crews and backcountry rangers, resource personnel, and other Park, or research, personnel in the Belly River area since 1910. Two backcountry rangers are stationed at the site full-time throughout the summer; and one large group of 6-12 employees is staged at the site for approximately 4 weeks each summer to conduct trail and preservation work in the backcountry. The site has served as a contact station for visitors and supported access to wilderness features of the area for over 90 years and maintaining the integrity of the Historic District is a priority. The National Park Service’s Management Guidelines 2001 (NPS 2000a) state that the NPS “will provide for the long-term preservation of, public access to, and appreciation of, the features, materials, and qualities contributing to the significance of cultural resources” (5.3.5. Treatment of Cultural Resources) and “all uses of historic structures are subject to preservation and public safety requirements” (5.3.5.4.7 Use of Historic Structures).

The proposed action would upgrade the water, wastewater, and propane systems in the district. Proposed changes are intended to protect resources and improve conditions for employees using the site during administrative duties. The objectives are to replace failing water and sewage systems, reduce on-going maintenance and operation costs, and prevent pollution caused by the sewage system. Not taking action could set a precedent for the maintenance of historic facilities, eventually result in closing of facilities, and endanger continued protection of wilderness values, park resources, and public enjoyment of those resources.

The Water System. The current water system functions by taking water from nearby Gable Creek and piping it downhill to the ranger station and two other buildings. Water treatment is achieved with a sand filter in the stream, and later particulate and charcoal filters in the buildings. The water system is classified by Montana DEQ as non public (residential-use only), and locks have been placed on exterior faucets at the site. However, occasionally the locks have been left off of the faucets and the park has been cited by the U.S. Public Health Service for allowing public access to the water. Additional, inadequacies with the existing water system include:

- The water intake is within a dam built in 1983 and the perforated intake pipes are prone to clogging with silt.

- Existing service lines to the buildings are exposed ½” copper pipe on the exterior of the buildings which is considered a substandard system.

- Underground, the copper pipes are joined directly to 1 ½” galvanized steel pipe which creates corrosion problems in the galvanized line. This underground line failed in 1995 from corrosion and other sections are also highly corroded.

The Sewer System. The concerns with the sewer system are from both an employee health standpoint and possible impacts to the environment. There are three existing sewer systems, all of which are substandard or have failed, which serve three buildings. Sources include one flush toilet located in the Ranger Station and sinks and shower facilities located in three buildings. The existing outhouses located across the creek from the District would not be altered. Problems with the sewer system include:

- None of the three septic tanks (cesspools) has leach field disposal lines.

- Two systems are located within 15 feet of a stream, which is not in compliance with DEQ standards and threatens the water quality of the stream.

- Sewage in the form of gray water occasionally surfaces on the ground next to the trails quarters and it contains food nutrients that could attract wildlife.

The Propane System. Safety is the primary issue with the existing propane system. Currently, there are three separate fuel systems, one for each of the three buildings, and propane bottles not in use are stored in a fourth building. The current system of propane storage requires frequent transport of heavy (i.e. 60 lb) propane tanks to the individual buildings and puts a potentially flammable fuel source in each building.

Public Involvement (Scoping)

Scoping is an early and open process to determine which environmental issues and alternatives should be addressed in an environmental assessment. Glacier National Park conducted both internal scoping with appropriate National Park Service (NPS) staff and external scoping with the public and interested and affected groups and agencies.

The interdisciplinary process of internal scoping defined the purpose and need for the project, identified potential alternatives to address these needs, determined what the issues were and what resources would be affected and identified the relationship, if any, of the proposed action to other planning efforts at the park.

Public scoping began with letters announcing the project and a press release issued on February 2, 2006. Letters were sent to the park’s mailing list for EAs, which includes various federal, state, and local agencies, such as the U.S. Fish and Wildlife Service (USFWS) and the Blackfeet Tribal Business Council, and the Confederated Salish and Kootenai Tribes. The announcement was also posted on the National Park Service’s public comment website.

In accordance with 36CFR800.8(c), Glacier National Park also notified the Montana State Historic Preservation Officer and the Advisory Council on Historic Preservation that it intended to prepare a combined Environmental Assessment/Finding of Effect for the proposed project.

Three replies were received during scoping for this project. Responses to these concerns and questions are in italics below and in the environmental assessment where noted below.

A former ranger at the site sent the following comments:

- Extreme care in regards to excessive noise and activity must be of high priority because of the site’s wilderness status. The park was concerned about excessive noise during the project and selected alternatives that would shorten the length of the project. The use of an excavator and helicopter would ensure the work would be completed in one summer as opposed to extending impacts across two summers. Impacts due to noise and worker activity are addressed in the Environmental Consequences section under the topics of Wildlife, Special Status Species, and Recommended Wilderness.

- The timing of the project needs to consider that elk use the site until mid-June and again in September, so work should occur in July and August. There would be some unavoidable disturbance of elk at the nearby sulfur lick. Work would occur June-September to allow for completion in one year. Impacts to elk are addressed in the Environmental Consequences section under Wildlife.

- Air-lifting and use of mechanical equipment is inappropriate for the site. The use of an excavator and helicopter would ensure the work would be completed in one summer as opposed to extending impacts across two summers.

- A crew of people should be hired to hand-dig all the trenches. The water line trenching should follow the old lines and trail. The use of an excavator and helicopter would ensure the work would be completed in one summer as opposed to extending impacts across two summers. Water line trenching would follow the old lines and trail (see description of Preferred Alternative).

- The entire area is an important archeological site and hand-digging would enhance chances of recovering objects. An archeologist should be on-site during the entire excavation period, and perhaps a graduate student project would report on the findings. Test excavations conducted in 2005 documented a low likelihood of disturbing artifacts in the area of potential effect (see Archeological Resources in Environmental Consequences section).

- Post-project restoration will be important, though the area has been greatly modified. Trench lines should be seeded with meadow species such as rough fescue, Idaho fescue, and shrubby cinquefoil. Seeding with native species would occur after work is completed.

- Basement water lines in the ranger station need attention. These were replaced during rehabilitation of the building in 2005.

- A single septic tank and leach field is a good idea. The leach field could be placed in the pasture, far from Gable Creek. How would the tank be brought to the site? See description of Preferred Alternative.

- The existing outhouses are a problem because of the high ground water table and consequent drainage downhill. One outhouse is necessary for use during winter patrols. Perhaps a chemical toilet should be considered, since the composting model is not very efficient. No work is scheduled for the outhouses under this project.

- A good propane system is essential for cooking, refrigeration, and for the thermoelectric generator. Existing woodshed can serve as central storage building. Perhaps propane lights would be good. Under the Preferred Alternative, the woodshed would be used as the central storage building. Propane lights were not considered appropriate for the site.

Another former employee had similar comments regarding the project:

- The ranger station is archeologically significant and the footprint of the work should be minimized. Agreed. The proposed action took the significance of the site into account when defining the area to be disturbed. Only the minimum amount of ground disturbance required to complete the project would be disturbed.

- Disturbance in the meadow should be minimized where wildlife traverse to the mineral lick. Access to the mineral lick would not be altered but some noise disturbance would occur during completion of the work (see impacts to Wildlife under Environmental Consequences section).

- Will the proposed waterline for fire protection have enough pressure to provide fire protection? A portable fire pump and hose have been kept near the woodshed in the past. Perhaps a new portable water pump and a better water storage area along the creek would provide better pressure. A new hydrant and waterline was removed from the proposed project and fire protection would be provided by the portable water pump. The pump would have a new water storage area providing more pressure.

- In the past, locks have been placed on the faucets. Will the faucets be unlocked once the water lines are replaced? No, the remaining faucet will not be unlocked as it does not meet the criteria for a public water source.

- Are the sewage improvements necessary given that the Ranger Station is hardly utilized as it was during the 1980s? Where is sewage surfacing? Only the ranger station produces sewage, two other buildings produce gray water. Perhaps a different solution would create less disturbance than what is proposed. Toilets should not be located in other buildings, the outhouses are adequate. The Ranger Station is still utilized on a regular basis and is an important contact station in this section of the park. Rehabilitating the sewage system is also integral to maintaining the integrity of the Historic District. Gray water occasionally surfaces next to the trails quarters. No new toilets are proposed.

- No changes to the propane system are needed. The only leaks experienced were with the connections to the propane stoves and refrigerator. Also, the amount of propane used is much less than in previous years. The current system of propane storage requires frequent transport of heavy (i.e. 60 lb) propane tanks to the individual buildings and puts a potentially flammable fuel source in each building.

A third respondent asked if the water or wastewater systems met the definition of a public system (25 or more people for 60 or more days a year). This is not how the Montana DEQ determines if a water system is deemed a public water system (see Purpose and Need section).

Relationship of the Proposed Action to Previous Planning Efforts

In 2003, several improvements to the Belly River Ranger Station Historic District were proposed by the park’s Facilities Management Division. The changes were intended to address life, health, and safety concerns for employees living and working at the site, as well as environmental protection concerns. Maintaining the integrity of the historic district was also a priority. The improvements included upgrades to the utility system as well as building maintenance and rehabilitation. The park’s management team determined that additional review of the proposals was warranted because of the site’s status as a historic district and its location within recommended wilderness. After review by the Management Team and staff specialists, the Management Team approved some building maintenance projects such as interior plumbing replacement, new roofs and exterior painting and staining under a Categorical Exclusion. These improvements were mostly completed in 2005. This proposed project addresses the utility improvements agreed upon by the Management Team. Those features of the utility systems in the original proposal which were not agreed to are discussed below under Alternatives Considered but Dismissed.

The Belly River Ranger Station Historic District is located within the backcountry zone as identified in the park’s General Management Plan (GMP; NPS 1999). According to the GMP, most of the park will be managed “for its wild character and for the integrity of Glacier’s natural heritage, while traditional visitor services and facilities would remain.” The proposed action is consistent with the objectives of Glacier National Park’s General Management Plan/Environmental Impact Statement and Record of Decision (1999).

# Impact Topics

Impact topics to be analyzed in this environmental assessment were identified based on: 1) federal laws, regulations, and Executive Orders and on 2001 NPS Management Policies (NPS 2000a); 2) NPS staff knowledge of the Park's resources; 3) internal staff discussion of potential alternative’s impacts; and 4) public scoping. These impact topics were evaluated to see if they should be carried forward for detailed analysis or if they should be eliminated from further consideration. Eight impact topics were identified for analysis in this document. A brief rationale for the selection of these impact topics is given below, as well as the rationale for dismissing other potential impact topics from further consideration.

Impact Topics Considered

### Historic Buildings/Structures/District

The proposed utility improvements include modifications to existing utility features in the Belly River Ranger Station Historic District. Therefore, the appropriateness of those changes to the character of the historic district is analyzed.

### Archeological Resources

Previous archeological surveys have identified sites in the Belly River area. Therefore, impacts to this resource are analyzed.

### Soils

The proposed utility upgrades would require substantial excavation. In addition, improvements to the sewage system could reduce contamination to soils in the area.

### Wildlife

The Goat-Haunt-Belly River area contains exceptional wildlife habitat. Several species of wildlife, including elk, use the Belly River Valley during important seasonal movements and could be impacted by the project.

### Special Status (Federal and State Listed) Species

Some special status species such as grizzly bears could be impacted by the project; therefore impacts to this resource are analyzed.

### Water Quality

The proposed changes address potential problems with both pollution of an adjacent stream and the safety of water used by employees working at the site. Therefore, impacts to this resource are analyzed.

### Natural Soundscapes

The proposed action includes the use of an excavator for trenching and a helicopter for transporting materials, both of which would impact the natural noise levels at the site. Therefore, impacts to this resource are analyzed.

### Recommended Wilderness

Wilderness qualities may be affected temporarily by the construction activities required to implement the proposed action. The use of mechanical equipment to conduct work in the wilderness must be analyzed as required by the Wilderness Act. The process for determining the most appropriate method for completing the work and its impacts to Recommended Wilderness are discussed.

### Visitor Experience

Visitors to the Belly River at the time of construction could be impacted by noise and increased human presence. Therefore, this topic is analyzed.

### Health and Safety

One of the main purposes for undertaking these utility changes is to improve health and safety for employees working and living at the historic district. Therefore, this topic is analyzed.

Topics Eliminated from Detailed Study

NEPA, Council on Environmental Quality (CEQ) regulations, and NPS procedures for implementing NEPA specify that an EA should address only those resource areas potentially subject to impacts. In addition, the level of analysis should be commensurate with the anticipated level of environmental impact. The following resource topics would only be impacted at an intensity level of minor or less by the proposed project and, consequently, were dismissed from further analysis.

### Vegetation

The National Park Service strives to maintain all components and processes of naturally evolving park unit ecosystems, including the natural abundance, diversity, and ecological integrity of plants (NPS 2000a). The Belly River Valley is a mix of deciduous forest, coniferous forest, grassland, and riparian communities. Along the Belly River floodplain are willows intermixed with black cottonwood, spruce, and aspen. The Ranger Station lies above the river on an alluvial terrace dominated by fescue grassland. Vegetation around the buildings proposed for improvements primarily consists of non-native grasses introduced during past disturbances. During trenching some native forbs could be disturbed; however, the top four inches of soil containing local seeds would be conserved and replaced over the disturbed area upon completion. In addition, the disturbed areas would be reseeded with native plants. Impacts to native vegetation from the proposed project would be negligible and this topic has been dismissed from further discussion.

### Ethnographic Resources

No ethnographic resources have been identified by the Confederated Salish and Kootenai Tribes or the Blackfeet Tribal Business Council in the Belly River area and no concerns were raised during scoping for this project. However, Glacier National Park recognizes that the tribes hold a body of knowledge that may result in the identification of ethnographic resources in the area in the future. If ethnographic resources are identified, consultation will occur in accordance with federal legislation and regulations and National Park Service policy. This topic was dismissed from further analysis.

### Cultural Landscapes

No associative cultural landscape has been identified in the Belly River Ranger Station Historic District. Project impacts on the National Register aspects of integrity, including design, location, setting, and feeling of the Belly River Ranger Station Historic District are evaluated under historic buildings and structures.

### Museum Collections

There are no Glacier National Park museum collections items stored or exhibited at the Belly River Ranger Station Historic District. Therefore, museum collections was dismissed from further analysis.

### Wetlands

For regulatory purposes under the Clean Water Act, the term wetlands means "those areas that are inundated or saturated by surface or ground water at a frequency and duration sufficient to support, and that under normal circumstances do support, a prevalence of vegetation typically adapted for life in saturated soil conditions. Wetlands generally include swamps, marshes, bogs, and similar areas." Executive Order 11990 Protection of Wetlands requires federal agencies to avoid, where possible, adversely impacting wetlands. Further, Section 404 of the Clean Water Act authorizes the U.S. Army Corps of Engineers to prohibit or regulate, through a permitting process, discharge or dredged or fill material or excavation within waters of the United States. National Park Service policies for wetlands as stated in 2001 Management Policies and Director’s Order (DO) 77-1 Wetlands Protection, strive to prevent the loss or degradation of wetlands and to preserve and enhance the natural and beneficial values of wetlands. In accordance with DO 77-1 Wetlands Protection, proposed actions that have the potential to adversely impact wetlands must be addressed in a Statement of Findings for wetlands.

For this project, work would be conducted approximately 800 feet from the riparian habitat along the edge of the Belly River, some of which qualify as wetlands. There are no wetlands associated with Gable Creek. There is not expected to be any erosion into nearby waterways and no adverse effects to wetlands would occur. A Statement of Findings for wetlands will not be prepared, and the impact topic of wetlands has been dismissed.

### Floodplains

Executive Order 11988 Floodplain Management requires all federal agencies to avoid construction within the 100-year floodplain unless no other practicable alternative exists. The National Park Service under 2001 Management Policies and Director’s Order 77-2 Floodplain Management will strive to preserve floodplain values and minimize hazardous floodplain conditions. The 100-year floodplain has not been mapped for waterways in this area.  The Ranger Station lies approximately 820 feet from the bank of the Belly River and approximately 60 feet higher. According to Director’s Order 77-2 Floodplain Management, this project is an excepted action from compliance with the Executive Order because it is an historic site whose location is integral to its significance. Furthermore, the proposed project does not propose building a new structure within the floodplain. Therefore, a Statement of Findings for floodplains will not be prepared and this topic has been dismissed from further analysis.

### Prime and Unique Farmlands

In 1980, the Council on Environmental Quality directed that Federal Agencies must assess the effects of their actions on farmland soils classified by the U.S. Department of Agriculture’s Natural Resources Conservation Service as prime or unique. There are no “prime or unique farmlands” in Glacier National Park (NPS 1999); therefore it was dismissed as an impact topic.

### Air Quality

The Clean Air Act establishes specific programs that provide special protection for air resources and air quality related values associated with National Park Service units. Section 118 of the Clean Air Act requires a park unit to meet all federal, state, and local air pollution standards. Glacier National Park is classified as a mandatory Class I area under the Clean Air Act, where emissions of particulate matter and sulfur dioxide are to be restricted. The act gives the federal land manager the responsibility for protecting air quality and related values (i.e., including visibility, vegetation, wildlife, soils, water quality, cultural resources, recreational resources, and public health) in Class I lands from adverse air pollution impacts; and to consider, in consultation with EPA, whether proposed industrial facilities will have an adverse impact on these values. Federal land managers are also required to determine whether existing industrial sources of air pollution must be retrofitted to reduce impacts on Class I areas.

Air quality is considered good in Glacier National Park. There are no major metropolitan areas within 125 miles of the park, and no regional smog typical of highly populated areas with a high amount of vehicle traffic. Exhaust emitted by a mechanical excavator and by a helicopter transporting the equipment to the site would be negligible and temporary. The Class I air quality designation would not be affected by the proposal. Therefore, air quality has been dismissed as an impact topic.

### Visual Resources

The new utility systems proposed in this project would be installed underground and would not result in long term change to the visual resources in the area. The only new visible structures are the hatch and vent pipes above the drying tank. The hatch would be made of aluminum and be approximately 42" X 42" square.  The vent pipes would be 2” -3” in diameter and reach 18” above the ground. Both the hatch and pipes would be painted to blend with surrounding vegetation and would have no more than a minor impact on visual resources. Views relative to the historic district are evaluated within the Historic Buildings and Structures section. Visual Resources was dismissed as an impact topic.

#### Environmental Justice

Executive Order 12898 “General Actions to Address Environmental Justice in Minority Populations and Low-Income Populations,” requires all federal agencies to identify and address disproportionately high and adverse human health or environmental effects of their programs and policies on minorities and low-income populations or communities. The proposed action would not have disproportionate health or environmental effects on minorities or low-income populations or communities as defined in the Environmental Protection Agency's Environmental Justice Guidance. Therefore, Environmental Justice was dismissed as an impact topic in this document.

#### Socioeconomics

The proposed action would neither change local and regional land use nor appreciably impact local businesses or other agencies. The construction would be performed by park employees so no new jobs would be created by the construction. Some local businesses may receive a small increase in revenue from the park purchasing or renting needed materials and equipment; however, any increase in revenue would be temporary and negligible. Therefore, this topic has been dismissed.

### Park Operations

Changes that may require a new level of maintenance or staffing, or that may affect existing facilities are considered under this topic. The proposed changes are designed to improve facilities and reduce the level of maintenance required at the site. However, current staffing levels would not change and the overall impacts to park operations would be no greater than minor. Therefore, this topic was dismissed.

# Alternatives Considered

No Action Alternative

The No Action Alternative describes the conditions that would continue to exist if the Proposed Action is not implemented. This alternative provides a baseline for evaluating the changes and related environmental impacts that would occur under the Proposed Action. Under this alternative, the utility improvements at the Belly River Ranger Station would not be implemented (Table 1). The water system would continue to erode and require intermittent maintenance. The water intake pipe would continue to periodically clog and require maintenance. The sewage system would continue to release gray water and organic material on to the surface. Some of this would continue to enter the creek. A 6’x8’ area of ground would continue to be saturated with gray water and organic material. Individual propane bottles would continue to be transported and connected within each building. Digging at the site would occur only in locations where repairs were immediately needed.

Preferred Alternative (Proposed Action)

The Proposed Action presents the National Park Service’s preferred alternative and is the proposed undertaking for compliance with Section 106 of the National Historic Preservation Act. Because of the hardness of the soil at the site, some of the required digging would occur using a diesel-powered mini-excavator, and the remainder would occur using hand tools. The excavator is expected to be used for 4-6 weeks starting in summer 2006. To conserve the seed bank on top of the trenches, the top four inches of soil would be excavated first and set aside. It would be placed back on top once new pipes are in place. Hand-held power tools, powered by a generator, would occasionally need to be used for constructing the new sewage tanks and some pipe fitting, but this would be for very short periods of time. All of the work would occur during the summer (June-September) of 2006 between the hours of 8 a.m. and 5 p.m.

Under this alternative, the following improvements to the water, sewage, and propane systems would be made:

Water System. All of the existing galvanized water lines would be replaced and a new intake system would be installed as described below. The proposed line would generally follow the existing water line route, grade, and depth under existing foot trails. The trench would be approximately 2 ft. x 2 ft. and approximately 1200 feet long.

- The water intake system would be upgraded by removing the perforated pipe currently buried in the sand at the dam and replacing it with a Hi-Q infiltrator chamber wrapped in geofabric. Rock (2-3 inches in size) would be used to backfill and cover the chamber. Additional layers of geofabric and geogrid would be placed above this to protect the system. A 3 to 4 inch layer of sand would then be laid as a filter above the infiltration system. A back flush system would be installed for the chamber to allow silts and sediments that settle into the gravel filter to be expelled without having to access the structure.

- The old galvanized water supply pipe would be replaced with 3-inch high density polyethylene (HDPE) pipe from the intake to the utility house. Then a 2-inch HDPE water main would be installed from the utility house alongside the buildings and ending near the main cabin. Each of the three buildings requiring water would have a 3/4" water service pipe that would branch off the main and follow existing routes.

- All of the external faucets would be removed from the buildings and the lines would be capped.

Sewer System. The proposed improvements would connect the three buildings into a single septic tank system and leach field. Approximately 480 feet of 2 ft. x 2 ft. trenches would be dug. The current system of seepage pits and cesspools would be replaced with a septic system to improve health and safety, and to make long-term maintenance more efficient. Most of the digging would be done using a mini-excavator.

- Sewer pipes from buildings 188, 492, and 1069 would be connected by a 4-inch sewer main and then connected to a new septic tank. Flow would be strictly by gravity.

- A 1,000 gallon prefabricated copolymer septic tank meeting state codes would be buried in the ground.

- Two field lines would be installed in the corral using Infiltrator TM chambers. The chambers are 6 feet long and weigh 23 pounds each. The preliminary length of these lines would be 80 feet.

- A 750 gallon prefabricated copolymer tank would be buried downhill from the septic tank to treat settled sludge. There is sufficient slope to the ground to allow the contents of the septic tank to be emptied by gravity. The drying tank would have a gravel floor to allow the septic tank effluent to percolate through the soil into the groundwater. A reinforced geofabric liner would be placed in the tank to retain the sludge and allow effluent to percolate away. During winter shut down procedures each fall, the valve at the septic tank would be opened and the contents flushed down to the drying tank.  The tank would have a hatch and air vents allowing trapped water to evaporate from the tank during the winter and through the next operating season. The hatch would be made of aluminum and be approximately 42" X 42" square. The hatch cover would be painted to blend with the surrounding vegetation. Vent pipes would be 2" or 3" in diameter, terminate 18" above the ground, and be painted to blend with the surrounding vegetation. Prior to flushing the septic tank contents, the geofabric liner would be pulled from the tank, through the hatch, with the trapped dried sludge and either packed or flown out. A new liner would be placed in the drying tank to capture the next batch.

- Required aggregate materials would be flown and/or packed in to the site.

Propane Fuel System. The proposal is to construct a central tank farm with an underground distribution system to the buildings. Approximately 40 feet of new buried propane line would be required. The remainder of the propane lines would be installed in trenches dug for the water lines. All propane lines would be buried and hidden from view.

- The central tank would be located in Building 490 (Wood Shed).

- Plastic piping (1-inch) would be installed underground along trenches dug for the water system.

- An exterior valve and pressure regulator would be installed in the Wood Shed and existing propane piping and fittings would be replaced.

Required Helicopter Flights: A maximum of six helicopter flights to the site would be needed to deliver the materials required for the improvements proposed under this alternative. Fewer flights may be conducted depending on: the amount of aggregate needed, the size of the helicopter, and if flights can be combined with other projects. All materials would be delivered via sling-loading and be packaged in bundles or nets. Some items that would be flown in, most notably an excavator, are too large to be packed in with stock. The loads would be flown directly to the site from a location approximately 5 miles away along Highway 17 south of the Chief Mountain Border Station. The helicopter would not land at the Belly River Ranger Station.

The following conditions to minimize the effects of helicopters on park visitors and wildlife are described in the park’s Aviation Management Plan (NPS 2000b):

- The flight should occur between 10:00 am and 4:00 pm.

- The helicopter should maintain a 2000 feet AGL altitude over the park.

- A helicopter manager should be on board for each trip to insure that conditions are met, safety is observed, and bear activity is monitored.

Every attempt would be made to follow the recommendations described above. However, due to the short distance of the flight and safety concerns relating to the weight of the loads to be transported, the helicopter would not be able to achieve the recommended altitude (2,000 feet AGL) over the park.

Alternatives Considered But Eliminated

Several additional improvements were originally proposed for the Belly River Historic District and presented to the park’s Management Team. Some of these such as replacing roofs, installing interior plumbing and exterior painting and staining were approved and completed in 2005 under a categorical exclusion.  Some improvements, including utility suggestions, were not approved because they were deemed incompatible with the Historic District and its location within Recommended Wilderness or unnecessary. Suggestions that were dismissed include:

- Install a micro-hydroturbine generator to provide additional electrical power for employees. This was determined to be incompatible within recommended wilderness. While it is a cleaner source of energy than propane, it would likely result in employees increasing use of and reliance on electrical tools and appliances within the area. Maintenance needs would increase with electrical power.

- Install a yard hydrant off the main water line for additional fire protection. It was determined that the pressure generated by such a hydrant would not exceed that currently provided by the existing portable water pump.

- Replace the existing Trail Crew Bunk House (Bldg #1069) with a new log building that would contain a new kitchen and flush bathroom. It was determined that the existing building contained enough space for current operations. In addition, the building is located within the Historic District; and though it is not listed as a contributing structure, it is approaching 50 years old and would need to be reevaluated.

- Transport all needed materials to the site using pack animals. Approximately 400 mule trips would be needed to haul the required materials and equipment to the site. The amount of time needed to conduct this many pack trips would prohibit the work from being completed in one summer season. There would also be impacts to the trails from the extensive use by pack animals and the presence of such frequent and large pack trains could impact visitors to the area. In addition, some materials needed to properly improve the utilities (i.e. septic and drying tanks) are too large to transport by pack animal.

- Conduct all digging using hand tools. The soils at the site are extremely hard and have high clay content. It would take a crew of six people, which is the maximum number of people the park can budget for this project, approximately 25 weeks to conduct all of the required trenching using hand tools. This length of time would not allow the work to be completed in a single summer extending the time utility services are interrupted at the site.

Environmentally Preferred Alternative

The environmentally preferred alternative is determined by applying the criteria suggested in the National Environmental Policy Act of 1969 (NEPA), which is guided by the Council on Environmental Quality (CEQ). The CEQ provides direction that the “environmentally preferable alternative is the alternative that will promote the national environmental policy as expressed in NEPA Section 101”:

- fulfill the responsibilities of each generation as trustee of the environment for succeeding generations;

- assure for all generations safe, healthful, productive, and esthetically and culturally pleasing surroundings;

- attain the widest range of beneficial uses of the environment without degradation, risk of health or safety, or other undesirable and unintended consequences;

- preserve important historic, cultural and natural aspects of our national heritage and maintain, wherever possible, an environment that supports diversity and variety of individual choice;

- achieve a balance between population and resource use that will permit high standards of living and a wide sharing of life’s amenities; and

- enhance the quality of renewable resources and approach the maximum attainable recycling of depletable resources.

The Preferred Alternative fulfills all six criteria better than the No Action Alternative. The Preferred Alternative accomplishes criteria 1, 2, 3, and 6 by eliminating sewage in the form of gray water pollution, or the threat of pollution, into the soils, groundwater, and stream. Criteria 2 and 4 are fulfilled by providing long-term maintenance solutions and by protecting the integrity of the historic district. The proposed improvements to the district would provide safer and more livable conditions for employees and volunteers stationed at the site, thus fulfilling criteria 5. The No Action Alternative does not fulfill any of the criteria because it allows for the continued release of sewage in the form of gray water into the environment, continues persistently unsafe conditions for inhabitants, and allows for the continued deterioration of a historic site. Therefore, the Preferred Alternative is the environmentally preferred alternative.

Summary

Table 1 provides a summary comparison of No Action and the Preferred Alternative (proposed action) and how they address or not address the project objectives/issues. Table 2 summarizes the impacts of each alternative on selected resources.

Table 1. Summary comparison of alternatives analyzed in this document and the degree to which they meet the project objectives (in italics).

| Project Objective/Issue | No Action Alternative | ##### Preferred Alternative |
| --- | --- | --- |
| Address clogging water intake system | The current perforated pipe and filtering system would continue to be used. Occasional maintenance would be required when the system clogs with silt. The water system would continue to clog and would not meet project objectives. | The water intake system would be replaced with a Hi-Q infiltrator chamber wrapped in geofabric. Layers of rock, geofabric, geogrid, and sand would be placed above this to protect the system. The chamber is large enough that no clogging should occur. A back flush system would be installed to allow sediment in the gravel filter to be expelled without having to access the structure. The new intake system would prevent the water system from becoming clogged and would meet the project objectives. |
| Replace corroded water pipes | Water pipes would be fixed on an as- needed basis and digging would occur only where immediate repairs were required. All digging would occur using hand tools. Water pipes would continue to corrode and would result in pipes occasionally breaking. The project objectives would not be met. | All galvanized water pipes would be replaced with HDPE pipe. Trenching for pipe replacement would be done using a mini-excavator flown in by helicopter. The new pipes would replace old, corroded pipes and reduce the occurrences of pipe breaks. The project objectives would be met. |
| Address potential for environmental contamination from sewer system | The current sewer system, which consists of three septic tanks that do not have leach field disposal lines, would remain in place. Sewage, in the form of gray water, occasionally surfaces on the ground from one system and it contains food nutrients that attract wildlife. The potential for gray water surfacing and for sewage reaching Gable Creek would continue. The project objectives would not be met. | Sewer lines would be attached to a new drying tank that would be installed downhill from the septic tank to treat settled sludge. A reinforced geofabric liner would be placed in the tank to retain sludge and allow effluent to percolate away. Prior to annual flushing of the septic tank contents, the geofabric liner would be pulled from the tank and either packed or flown out. A new liner would be placed in the drying tank to capture the next batch. The new system would contain all sewage and prevent environmental contamination, thus meeting the project objectives. |
| Ensure the safe use and storage of propane bottles | The current system which consists of separate fuel systems for each building would be maintained. Propane bottles are stored in one building, then transported and used within each building. Unsafe transport and storage of propane bottles within living quarters would continue. The project objectives would not be met. | A central tank farm with an underground distribution system to the buildings would be installed. All bottles would remain within the storage building. All propane bottles would be safely stored within a single building not used as living quarters. The project objectives would be met. |

Table 2. Summary comparison of effects on resources under the No Action and Preferred alternatives. Some resources may not be changed at all from current conditions and these topics have consequently been given a rating of “no effect”.

| Impact Topic | No-Action Alternative | Preferred Alternative |
| --- | --- | --- |
| Historic Buildings and Structures | Minor, long-term, site-specific, and adverse | Minor, long-term, site-specific, and beneficial |
| Archeological Resources | No effect | Negligible, Long-term, site-specific, and adverse |
| Soils | Moderate, long-term, local, and adverse | Minor-Moderate, short-term, local, adverse and beneficial |
| Wildlife | Minor, long-term, local, and adverse | Minor, short-term, local, adverse and beneficial |
| Special Status Species | Minor, long-term, local, and adverse | No effect to Minor, short-term, local, and adverse |
| Water Quality | Moderate, long-term, local, and adverse | Moderate, long-term, local, and beneficial |
| Recommended Wilderness | Minor, long-term, site-specific, and adverse | Moderate, short-term, local, and adverse |
| Natural Soundscapes | Minor, long-term, site-specific, adverse | Moderate, short-term, local, and adverse |
| Visitor Experience | Moderate, long-term, site-specific, and adverse | Moderate, short-term, local, adverse and beneficial |
| Health and Safety | Moderate, long-term, site-specific, and adverse | Moderate, long-term, site-specific, and beneficial |

# Affected Environment and Environmental Consequences

Methodology

The effects of each alternative are assessed for direct, indirect, and cumulative impacts on selected natural, cultural, and other resources. Potential impacts are described in terms of type (are the effects beneficial or adverse?), context (are the effects site-specific, local, widespread, or regional?), duration (are the effects short-term or long-term?), and intensity (are the effects negligible, minor, moderate, or major?). Because definitions of intensity vary by impact topic, intensity definitions are provided for each impact topic analyzed in this EA. Effects to historic properties listed in or eligible for listing in the National Register of Historic Places also have been described in accordance with the National Historic Preservation Act of 1966, as amended, and its implementing regulations, 36 CFR 800.

Impairment of Park Resources or Values

National Park Service Management Policies 2001 (NPS 2000a) require analysis of potential effects to determine whether or not actions would impair park resources or values. The fundamental purpose of the National Park System, established by the Organic Act and reaffirmed by the General Authorities Act, as amended, begins with a mandate to conserve park resources and values. NPS managers must always seek ways to avoid, or to minimize to the greatest degree practicable, actions that would adversely affect park resources and values.

These laws give the National Park Service the management discretion to allow impacts to park resources and values when necessary and appropriate to fulfill the purposes of a park, as long as the impact does not constitute impairment of the affected resources and values. Although Congress has given the National Park Service the management discretion to allow certain impacts within parks, that discretion is limited by the statutory requirement that the National Park Service must leave park resources and values unimpaired, unless a particular law directly and specifically provides otherwise.

The prohibited impairment is an impact that, in the professional judgment of the responsible NPS manager, would harm the integrity of park resources or values, including the opportunities that otherwise would be present for the enjoyment of those resources or values. An impact to any park resource or value may constitute an impairment. Impairment may result from NPS activities in managing the park, from visitor activities, or from activities undertaken by concessionaires, contractors, and others operating in the park. An impact would be more likely to constitute an impairment to the extent that it has a major or severe adverse effect upon a resource or value whose conservation is:

- Necessary to fulfill specific purposes identified in the establishing legislation or proclamation of the park;

- Key to the natural or cultural integrity of the park or to opportunities for enjoyment of the park; or

- Identified as a goal in the park’s general management plan or other relevant NPS planning documents.

Each alternative was analyzed to determine if impacts constituted an impairment to park resources and values.

Cumulative Impacts

The Council on Environmental Quality (CEQ) regulations, which implement the National Environmental Policy Act of 1969 (42 USC 4321 et seq.), require assessment of cumulative impacts in the decision-making process for federal projects. Cumulative impacts are defined as "the impact on the environment which results from the incremental impact of the action when added to other past, present, and reasonably foreseeable future actions regardless of what agency (federal or non-federal) or person undertakes such other actions" (40 CFR 1508.7). Cumulative impacts are considered for both the no-action and preferred alternatives.

Cumulative impacts were determined by combining the impacts of the preferred alternative with other past, present, and reasonably foreseeable future actions. Therefore, it was necessary to identify other ongoing or reasonably foreseeable future projects at Glacier National Park and, if applicable, the surrounding region. The following are past, present and reasonably foreseeable future actions that have occurred or could occur in the vicinity of the project area:

- Maintenance and rehabilitation improvements to buildings at Belly River Ranger Station Historic District, and four administrative flights to deliver materials (mostly completed in 2005).

- Replace tent platforms at Belly River Ranger Station with new platforms (summer 2006).

- Routine trail and building use and maintenance (ongoing).

Impacts to Cultural Resources and Section 106 of the National Historic Preservation Act

In this environmental assessment/assessment of effect, impacts to cultural resources are described in terms of type, context, duration, and intensity, which is consistent with the regulations of the Council on Environmental Quality (CEQ) that implement the National Environmental Policy Act (NEPA). To comply with Section 106 of the NHPA (36 CFR) Part 800, Protection of Historic Properties), and in accordance with the Advisory Council on Historic Preservation’s regulations implementing Section 106 of the NHPA, impacts to historic properties were identified and evaluated by (1) determining the area of potential effects; (2) identifying cultural resources present in the area of potential effects that were either listed in or eligible to be listed in the National Register of Historic Places; (3) applying the criteria of adverse effect to affected cultural resources either listed in or eligible to be listed in the National Register; and (4) considering ways to avoid, minimize or mitigate adverse effects.

Under the Advisory Council’s regulations a determination of either adverse effect or no adverse effect must also be made for affected National Register eligible cultural resources. An adverse effect occurs whenever an impact alters, directly or indirectly, any characteristic of a cultural resource that qualifies it for inclusion in the National Register (e.g. diminishing the integrity of the resource’s location, design, setting, materials, workmanship, feeling, or association). Adverse effects also include reasonably foreseeable effects caused by the preferred alternative that would occur later in time, be farther removed in distance or be cumulative (36 CFR Part 800.5, Assessment of Adverse Effects). A determination of no adverse effect means there is an effect, but the effect would not diminish in any way the characteristics of the cultural resource that qualify it for inclusion in the National Register.

CEQ regulations and the National Park Service’s Conservation Planning, Environmental Impact Analysis and Decision-making (Director’s Order #12) also call for a discussion of the appropriateness of mitigation, as well as an analysis of how effective the mitigation would be in reducing the intensity of a potential impact, e.g. reducing the intensity of an impact from major to moderate or minor. Any resultant reduction in intensity of impact due to mitigation, however, is an estimate of the effectiveness of mitigation under NEPA only. It does not suggest that the level of effect as defined by Section 106 is similarly reduced. Although adverse effects under Section 106 may be mitigated, the effect remains adverse.

A Section 106 summary is included in the impact analysis sections under the preferred alternative for cultural resource topics. The Section 106 Summary is intended to meet the requirements of Section 106 and is an assessment of the effect of the undertaking (implementation of the alternative) on cultural resources, based upon the criterion of effect and criteria of adverse effect found in the Advisory Council’s regulations.

### Historic Buildings and Structures

The Belly River Ranger Station Historic District consists of the ranger station (contributing), barn/stable (contributing), woodshed (contributing), fire cache (contributing), trail quarters (non-contributing), and two tent platforms (non-contributing)(Map 1). The buildings at the Historic District were constructed from 1912-1928 except the trail quarters which was built about 1960. Improvements were made to some of the buildings in the 1960s and 1970s, then again in 2005. The District was listed in the National Register of Historic Places in 1986 for its association with the administrative development of the park and for its rustic architectural design.

Before the Belly River Ranger Station was established as a ranger district in Glacier National Park, it was a Flathead Forest Reserve administrative site. The Forest Reserve was established in 1897 as a reservation of forested lands for the public good. Access to the Belly River Ranger Station followed rough trails or “corduroy roads”. Eventually, dependable routes gave most stations an easier approach, except at Belly River. The Belly River Ranger Station is unique in that it is the only station not accessible by established road; it is only accessible by horseback or hiking and still has relatively few modern conveniences. Of all of the East Side ranger stations, only the Belly River Ranger Station maintains the traditional configuration of buildings (station/residence, barn, woodshed, and fire cache), with few intrusions and with excellent physical integrity. (Historical Research Associates 1985)

Impact Analysis – Historic Buildings and Structures

Intensity Level Definitions: Impacts to historic properties were assessed based on the effects of the alternative on the Historic District. The thresholds for this impact assessment are as follows:

Negligible: Impact(s) is at the lowest levels of detection - barely perceptible and not measurable. For purposes of Section 106, the finding of effect would be no adverse effect.

Minor: Impact would alter a character defining feature(s) of the building or district, but would not diminish the integrity of the resource to the extent that its National Register eligibility is jeopardized. For purposes of Section 106, the finding of effect would be no adverse effect.

Moderate: Impact would alter a character defining feature(s) of the building or district to the extent that it is no longer eligible for listing in the National Register. For purposes of Section 106, the finding of effect would be adverse effect.

Major: Impact would alter a character defining feature(s) of a National Historic Landmark, diminishing its integrity to the extent that its designation is threatened. For purposes of Section 106, the finding of effect would be adverse effect.

No Action Alternative:  This alternative would not change current conditions within the Historic District. The buildings have recently been rehabilitated and that should ensure their integrity for many years. However, the water system would continue to erode and require occasional maintenance. The sewage system would continue to release gray water and organic material on to the surface and into the creek. These unsanitary conditions that negatively impact wildlife could eventually lead to a decision to reduce the use of the Historic District and eventually result in its deterioration. The result of not upgrading the utilities would be a minor, long-term, site-specific, adverse impact to the Historic District.

Preferred Alternative:  Most of the work proposed under this alternative would occur below ground and would not result in changes to the appearance of the Historic District. During construction, there would be a temporary, adverse visual impact to the Historic District; however, the burying of some water lines currently located above ground would be a long-term visual improvement to the Historic District. Consolidating the propane bottles into a single building would remove flammable fuel containers improving fire safety in the Historic District. Part of the Historic District’s significance is its association with park administrative development. These improvements would enable the buildings to remain functional and provide necessary facilities for park administration. The improvements would help ensure the continued integrity of the site. Overall, the impacts to the Historic District from the Preferred Alternative would be minor, long-term, site-specific, and beneficial.

Section 106:  In accordance with 36 CFR 800.5(b), Glacier National Park has reached a finding of no adverse effect for the preferred alternative.

Cumulative Effects: The No Action Alternative would not create, remove, or alter any historic buildings within the Historic District and would not contribute cumulatively to any other projects occurring in the District. In combination with the recent improvements to the buildings in the District, the utility improvements would enhance the longevity and integrity of the buildings in the District. On-going maintenance and use of the site would continue to ensure the integrity of the site. Cumulative impacts to the District from the Preferred Alternative would be moderate, long-term, localized, and beneficial.

Conclusion: The result of the No Action Alternative would be minor, long-term, site-specific, adverse impacts to the integrity of the Historic District because of continued failure of the sewage and water systems. Overall, the utility improvements to the Historic District from the Preferred Alternative would be minor, long-term, site-specific, and beneficial by ensuring the continued integrity of the site. For purposes of Section 106, the finding would be no adverse effect.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Archeological Resources

Early humans have utilized the Glacier National Park area since about 10,000 years ago. Archaeological evidence shows seasonal use of park areas for hunting, fishing, and plant harvesting. Early people also used some valleys and mountain passes as travel corridors. Archeological field inventories were carried out in the Belly and Mokowanis River valleys, from the International Boundary south into the headwaters of the two principal tributaries, by Dr. Brian O.K. Reeves in 1994. Reeves found that the main stem of the Belly River, including its south and west branches, exhibit a distinctive precontact Native American settlement pattern associated with bison hunting (Reeves 1996, Reeves 2003).

In compliance with Section 106 of the National Historic Preservation Act (NHPA), Glacier National Park conducted a cultural resource survey and test excavations during the summer of 2005 for the purpose of identifying and evaluating cultural resources in the area of potential effect of the proposed utility improvements. The entire Ranger Station area was intensively surveyed in 10-foot transects, including the length of the streambed to the existing impoundment dam. Test excavations were performed at three sites proposed for disturbance and one was selected outside the area of disturbance near the creek to test an area of relatively little disturbance. Results of the survey and test excavations are documented in a separate report (Riley 2005) that will be submitted to the State Historic Preservation Office for review.  In summary, for purposes of this project, the test excavations did not identify artifacts in the proposed areas of disturbance

Impact Analysis – Archeological Resources

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: Impact is at the lowest levels of detection, barely measurable, with no perceptible consequences, either adverse or beneficial, to archeological resources. For purposes of Section 106, the determination of effect would be no adverse effect.

- Minor: Disturbance of a site(s) is confined to a small area with little, if any, loss of important information potential. For purposes of Section 106, the determination of effect would be no adverse effect.

- Moderate: Disturbance of the site(s) would not result in a substantial loss of important information. For purposes of Section 106, the determination of effect would be adverse effect.

- Major: Disturbance of the site(s) is substantial and results in the loss of most or all of the site and its potential to yield important information. For purposes of Section 106, the determination of effect would be adverse effect.

- Short-term: Any disturbance of an archeological resource would be permanent, thus there are no short-term impacts.

- Long-term: Any disturbance of an archeological resource would be permanent, thus all impacts are long-term.

No Action Alternative: Ground disturbance would occur when emergency repairs of water or sewage lines were necessary. Thus digging would only occur in previously disturbed areas and no new archeological resources are expected to be uncovered. However, employees conducting the digging would be instructed to report any objects found to the Cultural Resource Specialist. There would be no effect on archeological resources.

Preferred Alternative: The findings of the 2005 cultural resource survey and test excavations document low likelihood of disturbing artifacts in the area of potential effect. Consequently, archeological resources are not expected to be encountered during digging for this project. In addition, most of the excavation for water line and propane line replacement would occur in areas previously disturbed. Consequently, the park’s Cultural Resource Specialist has determined that the area has been adequately surveyed and the probability of discovering cultural resources is highly unlikely. However, an archeologist or para-archeologist would monitor initial ground-disturbing activities and a decision to continue monitoring will be made based upon further examination of disturbed soils. Should artifacts be identified during the project, work would cease in the immediate area and the cultural resource specialist would be notified. Additional consultation in accordance with 36 CFR 800.13 for post-review discoveries would occur. The impacts to archeological resources from the Preferred Alternative would be long-term, site-specific, negligible, and adverse considering the types of artifacts recovered during test excavations.

Section 106:  In accordance with 36 CFR 800.5(b), Glacier National Park has reached a finding of no historic properties (archeological) affected for the preferred alternative.

Cumulative Effects: Ground disturbance that would occur to perform emergency repairs combined with ground disturbance from ongoing trail maintenance activities would occur in previously disturbed areas that have been surveyed for archeological resources. Therefore no cumulative impacts would occur under the No Action alternative. The No Action Alternative does not add a new action with which to combine with other projects. Therefore, no cumulative impacts would occur under the No Action Alternative. No past or planned excavations or other activities that could harm archeological resources are planned for the Belly River area. Therefore, no cumulative impacts to archeological resources are expected from the Preferred Alternative.

Conclusion: There would be no effect to archeological resources from the No Action Alternative. The impacts to archeological resources from the Preferred Alternative would be long-term, site-specific, negligible, and adverse based upon the cultural resource survey and test excavations conducted at the site.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Soils

The National Park Service will strive to understand and preserve the soil resources of park units and to prevent, to the extent possible, the unnatural erosion, physical removal, or contamination of the soil, or its contamination of other resources (NPS 2000a).  The soils present at the site are clay-rich conifer and aspen forest soils located on glacial moraines and landslides. Below the top layer of soils, the subsoil is gravelly and can contain 15-45% rock. In the Belly River area, these soils support aspen forests with scattered conifers and a diverse understory. The potential for successful revegetation is high due to a high silt and clay content which result in high moisture and nutrient holding capacity. Erosion potential is also high (Dutton et al. 2001). Clay soils can contract and expand depending upon the amount of water they contain. This soil type is common in this part of the park.

Impact Analysis – Soils

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: Soil productivity or soil fertility would not be affected or the effect would be below or at the lower end of detection. Any effects to soil productivity or soil fertility would be slight and not measurable.

- Minor: The effects to soil productivity or soil fertility would be detectable, but small. The area affected would be localized.

- Moderate: The effect to soil productivity or soil fertility would be readily apparent. Effects would result in a change in soils over a relatively wide area or multiple locations.

- Major: The effect on soil productivity or soil fertility would be readily apparent and would substantially change the character of soils over a large area.

- Short-term: After implementation, would recover in less than 3 years.

- Long-term: After implementation, would take more than 3 years to recover or effects would be permanent.

No Action Alternative: Digging, with hand tools, would occur only when a repair to a water or sewer line needed to be made. All of the locations where potential digging could occur would be in previously disturbed soils. Soils would be replaced after the digging was completed including the replacement of the topsoil. Revegetation should occur naturally although if a large enough area was disturbed some native seeds could be dispersed at the site. Erosion should not occur because the area around the District is relatively flat. The sewage system would continue to expel untreated gray water into soils at the site. The sewage would remain at the site because of the low permeability of the soil. The overall impact to soils from the No Action Alternative would be moderate, long-term, localized, and adverse.

Preferred Alternative: Approximately 1680 feet of 2 ft. x 2 ft. trenches would be excavated under this alternative. Most of this area was previously disturbed during the installation of the original water and sewer lines. An additional 40 feet of new trenching would occur for the propane lines in sites where no lines currently exist. Because of their location adjacent to buildings and within the historic district compound, the soils in these new trenches are also largely disturbed by compaction and their productivity is low. The soil would be replaced upon completion of the project. In addition, topsoil would be conserved and replaced and additional seeding of the disturbed areas would occur. Erosion is not expected to increase during or after the project because of the flat topography of the site. Revegetation should occur rapidly. Improvements proposed for the sewage system would keep untreated sewage in the form of gray water from contaminating soils. The overall impact to soils from the Preferred Alternative would be minor to moderate, long-term, localized, adverse and beneficial.

Cumulative Effects: Soil disturbance has and would continue to occur during emergency repairs, combined with ongoing trail maintenance activities which result in soil disturbance. Under No Action, the cumulative impacts to soils would be negligible, short term, localized and adverse. Some soil excavation adjacent to buildings occurred during structural improvements in 2005 but the amount was negligible and the soils were previously disturbed by compaction. No other recent projects have included soil excavation or changed the productivity of local soils. In combination with the proposed project, the cumulative impact to soils would be minor, short-term, localized, and adverse.

Conclusion: The No Action Alternative would result in minor, long-term, localized, and adverse impacts due to the continued release of sewage in the form of gray water into the soil. The overall impact to soils from the Preferred Alternative would be minor to moderate, short-term, localized, beneficial and adverse. The beneficial effects of reducing the sewage contamination diminish the minor adverse effects of soil excavation.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Wildlife

The National Park Service strives to maintain all components and processes of naturally evolving park unit ecosystems, including the natural abundance, diversity, and ecological integrity of animals (NPS 2000a). Elk migrate through the area when moving between their wintering grounds to the north and their summer areas to the south. According to the park’s Wildlife Observation Reporting Form (WORF) database, during the period 1990-2003, there were 82 observations of elk in the valley near the ranger station between May and September. Forty-nine (60%) of the recorded observations occurred in May. Most individuals have moved through by the middle of June when trenching would begin and return again in the fall (only 4 observations in September; 29 during June, July and August). There is also a natural mineral lick located nearby that attracts animals year-round. Other species known to occur in the area include moose, white-tailed deer, mule deer, yellow-bellied marmots (which are rare in the park), Columbian ground squirrels, coyotes, short-tailed weasels, northern goshawks, northern saw-whet owls, osprey, pileated woodpeckers, and many other species of birds, rodents, and insects. It should be noted that the observations from the WORF database reflect where and when people are present and not necessarily the areas most frequently used by wildlife; nocturnal activity or activity in forest cover near the ranger station is largely undocumented, and not all sightings of wildlife in the area are recorded, thus the database is not a comprehensive source of wildlife use. Lacking better documentation, the database serves as the best available source.

Impact Analysis – Wildlife

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: Effects would be at or below the level of detection and the changes would be so slight that they would not be of any measurable or perceptible consequence to the wildlife species’ population.

- Minor: Effects on wildlife would be detectable, although the effects would be localized and would be small and of little consequence to the species’ population.

- Moderate: Effects on wildlife would be readily detectable and widespread, with consequences at the population level.

- Major: Effects on wildlife would be obvious and would have substantial consequences to wildlife populations in the region.

- Short-term: After implementation, would recover in less than 1 year.

- Long-term: After implementation, would take more than 1 year to recover or effects would be permanent.

No Action Alternative: This alternative would not change conditions for wildlife at the site. Repairs to water and sewer lines would infrequently occur and digging would be conducted by hand. There would be no change in noise levels. Sewage in the form of gray water would continue to surface at the site creating a wildlife attractant. This attractant could lead to unwanted human/wildlife encounters at the site, possibly leading to adverse impacts on wildlife. If wildlife such as bears became habituated to the saturated site containing gray water and organic material, they may need to be removed from the area. The overall impacts on wildlife from the No Action Alternative would be long-term, localized, minor, and adverse.

Preferred Alternative: Animals in the vicinity of the District would be temporarily disturbed by noise from the helicopter and a mini-excavator, and from the presence of a work crew during much of the summer in 2006. Wildlife present near the District have habituated to some extent to human presence and both administrative and commercial overflights occasionally occur in the area. However, many wildlife species probably avoid the immediate vicinity of the historic district during the summer when the area is inhabited by employees. For this project, a maximum of six flights would occur and during those flights the helicopter would fly relatively low because of the heavy loads it is expected to carry.

Low level flights displace and/or disrupt normal behavior patterns of wildlife along flight paths. There is wide variability in the reaction of wildlife to aircraft based on the degree of habituation to the activity, availability of escape cover, and the type, noise level, altitude, and movements of the aircraft involved. Like other noises, the primary response of wildlife due to approaching helicopters is to run and seek cover. Behaviors of species, such as elk, that travel in herds are difficult to predict because individual response can influence the entire herd. Czech (1991) found that flight responses due to noise disturbance of herds of Roosevelt elk could often be attributed to the decision of one individual to flee for cover. The author also noted that there was less response from elk when forested cover was available nearby.

Noise produced by an excavator would be heard outside of the historic district for an unknown distance. It could probably be heard from the nearby mineral lick. Animals would probably give the district a wider berth during the construction period, though the work should not prevent movements through the valley. Impacts would be to individuals and not entire populations. Some reported physiological changes to animals due to exposure to noise include immune system suppression, increased heart rate, and increased energy expenditure (Larkin et al. 1996). These changes vary considerably among species. Any movements cause wildlife to utilize valuable energy that may be needed for winter survival or for reproductive success in the spring.

The use of the mechanical equipment would allow the work to be completed in one year instead of two. Overall habitat conditions would not change due to actions in this alternative. Improving the sewage system would benefit wildlife by removing a wildlife attractant caused by sewage in the form of gray water that currently reaches the surface. The overall impacts to wildlife and wildlife habitat from the proposed project would be short-term, localized, minor, adverse and beneficial.

Cumulative Effects: Emergency repair activity combined with ongoing maintenance of buildings and trail maintenance and repairs would cumulatively result in long term, localized, minor adverse impacts. Structural improvements to the historic district conducted in 2005 also had noise increases associated with the work, including four helicopter flights. Conducting utility work in 2006 would result in two consecutive summers of increased noise levels which could be detrimental to local wildlife. It also stresses the importance of completing the work in one season instead of finishing in 2007 which would be a third consecutive summer of noise increases. Replacing the tent platforms at the site concurrently with the utility work would create additional noise but construction is only expected to take two days and only hand tools would be used. Cumulative effects of combining the proposed action with other projects would be short-term, localized, minor, and adverse.

Conclusion: The overall impacts on wildlife from the No Action Alternative would be long-term, localized, minor, and adverse due to the continued presence of a wildlife attractant resulting from sewage in the form of gray water. The overall impacts to wildlife from the Preferred Alternative would be short-term, localized, minor, and adverse due to temporary increased noise levels caused by use of a helicopter and an excavator.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Special Status (Federal and State Listed) Species

The Endangered Species Act of 1973 requires examination of impacts on all federally-listed threatened, endangered, and candidate species. Section 7 of the Endangered Species Act requires all federal agencies to consult with the U.S. Fish and Wildlife Service to ensure that any action authorized, funded, or carried out by the agency does not jeopardize the continued existence of listed species or critical habitats. In addition, the 2001 Management Policies (NPS 2000a) and Director’s Order 77 Natural Resources Management Guidelines require the National Park Service to examine the impacts on federally-listed species, as well as state-listed threatened, endangered, candidate, rare, declining, and sensitive species.

Further protection under the Migratory Bird Treaty Act makes it unlawful to pursue, hunt, kill, capture, possess, buy, sell, purchase, or barter any migratory bird, including the feathers or other parts, nests, eggs, or migratory bird products. In addition, this act serves to protect environmental conditions for migratory birds from pollution or other ecosystem degradations. Some migratory birds are present in the vicinity of the historic district, but the area of ground disturbance consists primarily of non-native vegetation.

The following federally-listed species occur within Glacier National Park:

Status Species

Endangered Gray wolf (Canis lupus)

Threatened Bald eagle (Haliaeetus leucocephalus)

Grizzly bear (Ursus arctos horribilis)

Canada lynx (Lynx canadensis)

Bull trout (Salvelinus confluentus)

Spalding’s catchfly (Silene spaldingii)

Water howellia (Howellia aquatillis)

Candidate Slender moonwort (Botrychium lineare)

Grizzly bear sightings are common in the Belly River Valley; the park’s Bear Information Management System (BIMS) observation database contains 13 records of grizzly bear sightings within a mile of the Belly River Ranger Station since 1990. Hundreds of other observations within the Goat Haunt-Belly River Geographic Area are also recorded. Sightings include both grizzly bear family groups (females with young) and individual bears. Riparian areas along the Belly River are highly suitable spring grizzly bear habitat. The Ranger Station is a backcountry facility situated within Situation 1 grizzly bear habitat which requires that maintenance and improvement of grizzly bear habitat and grizzly-human conflict minimization will receive the highest management priority (Grizzly Bear Recovery Plan, USFWS 1993).

Gray wolves, Canada lynx, and other carnivores including wolverines, marten, and mountain lions are known to use the Belly River Valley. Although wolves once denned and ranged throughout the area, no denning or rendezvous sites are known from the American portion of the Belly River Valley since this species started recolonizing the park in the 1970s. Wolf tracks or sightings in the Belly River drainage have been recorded 32 times since 1992. There are no known bald eagle nests or foraging areas near the historic district or along the proposed helicopter flight path. The closest bald eagle nest is located at Cosley Lake approximately 2 miles west of the Ranger Station, but that nest site has been inactive for several years.  Nesting has been suspected but not confirmed at Elizabeth Lake, approximately 3 miles south of the station.  Bald eagles are occasionally observed flying over the station. Bull trout occur in Belly River. In addition to the federally protected wildlife species listed above, the park also contains species that are listed as “species of special concern” by the Montana Natural Heritage Program, “priority species” by Montana Partners in Flight, and “sensitive species” by the Flathead National Forest (Appendix A). Species which could be affected by the project are discussed below.

There are no known locations of Spalding’s catchfly, water howellia, or slender moonwort near the historic district. No other sensitive plant species have been identified in the area to be disturbed for this project.

Impact Analysis – Special Status Species

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: The alternative would affect an individual of a listed species or its critical habitat, but the change would be so small that it would not be of any measurable or perceptible consequence to the protected individual or its population. Negligible effect would equate with a “no effect” determination in U.S. Fish and Wildlife Service terms.

- Minor: An individual(s) of a listed species or its critical habitat would be affected, but the change would be small. Minor effect would equate with a “may affect, not likely to adversely affect” determination for the species in U.S. Fish and Wildlife Service terms and would require informal consultation.

- Moderate: An individual or population of a listed species, or its critical habitat would be noticeably affected. The effect could have some long-term consequence to individuals, populations, or habitat. Moderate effect would equate with a “may affect” determination in U.S. Fish and Wildlife Service terms and would be accompanied by a statement of “likely…” or “not likely to adversely affect” the species and would require either informal or formal consultation.

- Major: An individual or population of a listed species, or its critical habitat, would be noticeably affected with a vital consequence to the individual, population, or habitat. Major effect would equate with a “may affect, likely to adversely affect” or “not likely to adversely affect” determination in U.S. Fish and Wildlife Service terms and would require formal consultation.

- Short-term: After implementation, would recover in less than 1 year.

- Long-term: After implementation, would take more than 1 year to recover or effects would be permanent.

No Action Alternative: This alternative would not change conditions for wildlife at the site. Repairs to water and sewer lines would infrequently occur and digging would be conducted by hand. There would be no change in noise levels. Sewage in the form of gray water would continue to surface at the site creating a wildlife attractant. This attractant could lead to conflicts with grizzly bears possibly leading to adverse impacts on individual bears. If grizzly bears became habituated to the saturated gray water, they may need to be removed from the area. Gray water could also be reaching the Belly River, where bull trout are present. Although the amount of gray water reaching the Belly River is probably quite small, there is the possibility that it is affecting fish. The overall impacts on wildlife from the No Action Alternative would be long-term, localized, minor, and adverse.

Preferred Alternative: The three project components that could impact special status species are: 1) presence and noise of helicopter flights, 2) noise generated during construction including use of an excavator, and 3) elimination of a surfacing gray water source.

1) Klein (1974) studied the effects of low-flying aircraft on large mammals in arctic Alaska and noted that wolves appeared the least disturbed of all species observed. Wolves tracked from low level fixed-wing aircraft have been observed moving quickly away from the aircraft into forest cover (NPS files). Overflights may displace wolves from areas within the flight path; however, any displacement is expected to be temporary. Low-level aircraft flights also have the potential to disturb and displace wolves at important den and rendezvous sites. Wolves in Northwest Montana did not abandon an occupied rendezvous site that was subjected to daily low-level helicopter overflights from a nearby logging operation (Claar et al. 1999). Wolves, even while denning, have demonstrated tolerance to human activities (Thiel et al. 1998). Wolves using the area are probably dispersing from a pack or foraging as no known pack or denning locations are known in the area.

A summary of the literature by the Interagency Grizzly Bear Committee (IGBC 1987) concluded that there is wide variability in the reaction of grizzly bears to aircraft disturbances. Research in Canada and GNP has documented that grizzly bears moved away from helicopters when approached (Harding and Nagy 1980, Kendall 1986). Kendall (1986) documented that 81% of grizzlies observed during low-level helicopter flights in the Apgar Mountains of the park displayed a strong reaction. A “strong” reaction was defined as a bear moving faster than a slow walk, while a “mild” reaction was indicated when a bear did not move at all or slowly walked as the helicopter approached. Bears that move away from a disturbance expend extra energy and possibly enter an area occupied by another bear and experience stress (McLellan and Shackleton 1989). The approximate flight path for this project occurs mostly over forested areas where the potential effects of the helicopter would be minimized. If grizzly bears are in open areas at lower elevations when the flights occur, there are sufficient forested areas throughout the valley for bears to find cover in. Since the flights would occur during the day when bears are less active, the overflights would not significantly affect their ability to find forage.

Impacts from overflights have not been documented for Canada lynx, but they probably react in a similar manner as wolves and bears in that they seek shelter. Other sensitive mammal species such as wolverine and fisher would also react similarly to wolves and bears. There are no known bald eagle nests or foraging areas near the historic district and the flight path chosen for this project does not fly over any nest or foraging sites. Bull trout would not be affected by overflights. Special status bird species in the area could be temporarily flushed from their nests during overflights, but the duration should be short.

2) Noise generated during construction, including that from an excavator, would impact special status species including bird species, but these adverse impacts would be 1) temporary, lasting only as long as construction, and 2) negligible, because suitable habitat for birds is found throughout the vicinity. Noise produced by an excavator would be heard outside of the historic district for an unknown distance. It could probably be heard from the nearby mineral lick. Animals may give the district a wider berth during the construction period, though the work should not prevent wildlife movements through the valley. Impacts would be negligible.

3) Improving the sewage system would remove the attractant currently present that could impact grizzly bears. The possibility that a bear would become habituated to the area would be reduced, as would the chance of bear/human conflicts. Although the amount of sewage in the form of gray water currently reaching the Belly River is probably small (no tests have been conducted), bull trout could benefit from the removal of this source of pollution.

The overall impact to Special Status Species from the Preferred Alternative would range from no effect to minor, short-term, localized, and adverse. Federally-listed plant species would not be affected. The effect to bull trout would be negligible and beneficial. Bald eagles would be negligibly affected by helicopter disturbance because there are no nest sites along the flight path, though they could be temporarily displaced from a foraging site along the flight path. Wolves, grizzly bears, and Canada lynx would be adversely affected at a minor level due to helicopter disturbance and construction noise. The benefits of removing the sewage would slightly offset the impacts caused from increased noise.

Cumulative Effects: Emergency repair activity combined with ongoing maintenance of buildings and trail maintenance and repairs would cumulatively result in long term, localized, minor adverse impacts. The adverse effects of administrative overflights in the park have been discussed and attempts to minimize those impacts on federally listed species are described in the Programmatic Biological Assessment. Construction during 2005 to complete structural improvements in combination with utility improvements in 2006 would result in two consecutive summers of increased noise at the site. As a result, some species may slightly alter movement patterns to avoid the area near the historic district. However, any changes in movements made by individuals would last only during the construction period. Cumulative effects from the Preferred Alternative would be minor, short-term, localized, and adverse.

Conclusion: The overall impacts on Special Status Species from the No Action Alternative would be long-term, localized, minor, and adverse due to the possibility of a grizzly bear becoming habituated to the exposed gray water and organic material. The overall impact to Special Status Species from the Preferred Alternative would be minor, short-term, localized, beneficial and adverse, due mostly to helicopter overflights.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Water Quality

National Park Service policies require protection of water quality consistent with the Clean Water Act. The purpose of the Clean Water Act is to "restore and maintain the chemical, physical, and biological integrity of the Nation's waters". To enact this goal, the U.S. Army Corps of Engineers has been charged with evaluating federal actions that result in potential degradation of waters of the United States and issuing permits for actions consistent with the Clean Water Act. The U.S. Environmental Protection Agency also has responsibility for oversight and review of permits and actions, which affect waters of the United States. The site sits along Gable Creek, which flows year-round from a spring into the Belly River. Gray water waste that occasionally reaches the surface is washed into the creek and some waste below the surface probably reaches the creek. Water quality testing at this site has not been conducted. Gable Creek empties into the Belly River approximately 1400 feet downstream of the District.

Impact Analysis – Water Quality

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: Water quality would not be affected, or changes would be either non-detectable or if detected, would have effects that would be considered slight and local.

- Minor: Changes in water quality would be measurable, although the changes would be small and the effects would be localized.

- Moderate: Changes in water quality would be measurable but would be relatively local.

- Major: Changes in water quality would be readily measurable, would have substantial consequences, and would be noticed on a regional scale.

- Short-term: Effects last less than 1 year.

- Long-term: Effects last more than 1 year.

No Action Alternative: This alternative would not change current conditions. Gray water would continue to reach the creek and affect water quality. The result would be moderate, long-term, localized, and adverse.

Preferred Alternative: Improvements to the sewage system proposed under this alternative would prevent gray water from entering Gable Creek. Installing a new intake system that does not clog as the current system does at the dam in Gable Creek would result in less maintenance having to be conducted in the stream. Measures to prevent soil erosion from reaching Gable Creek during construction would be implemented, if necessary, at the site. Therefore, no adverse impacts to water quality should occur from the proposed project if conducted as described. The overall impact to water quality would be moderate, long-term, localized, and beneficial.

Cumulative Effects: While there is no other known location in the project area of gray water contamination occurring, continued conditions at this site combined with very small amounts of sediment entering creeks in the area from trail repair and maintenance near water would result in minor, long-term, localized adverse cumulative impacts on water quality. The No Action Alternative does not add a new action with which to combine with other projects. Therefore, no cumulative impacts would occur under the No Action Alternative. Structural improvements to the historic district and the tent platforms would not have any impacts on water quality; thus in combination with the proposed project would not change the level of impact. There would be no cumulative impacts from the Preferred Alternative.

Conclusion: The No Action Alternative would result in moderate, long-term, localized, and adverse impacts to water quality as some amount of gray water would continue to reach the creek. The Preferred Alternative would result in moderate, long-term, localized, and beneficial impacts by preventing gray water from entering Gable Creek.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Natural Sounds

In accordance with 2001 Management Policies (NPS 2000a) and Director’s Order 47 Sound Preservation and Noise Management, an important component of the National Park Service’s mission is the preservation of natural soundscapes associated with national park units. Natural soundscapes exist in the absence of human-caused sound. The natural ambient soundscape is the aggregate of all the natural sounds that occur in park units, together with the physical capacity for transmitting natural sounds. The frequencies, magnitudes, and durations of human-caused sound considered acceptable varies among National Park Service units as well as potentially throughout each park unit, being generally greater in developed areas and less in undeveloped areas.

Impact Analysis – Natural Sounds

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: There would be no introduction of artificial noise from the project.

- Minor: A short-term introduction of artificial noise would occur at localized sites. The effect would be readily detectable, but would not adversely affect visitors or wildlife.

- Moderate: A widespread introduction of noise would be readily detectable and would adversely affect nearby visitors and wildlife.

- Major: A long-term introduction of noise would occur that would adversely affect visitors and wildlife.

- Short-term: Occurs only during construction work.

- Long-term: Occurs beyond construction period.

No Action Alternative: Under this alternative, there would be no new introduction of artificial noise to the site. However noise from emergency repairs and ongoing onsite maintenance would continue. There would be continued minor, long-term, site-specific, adverse impacts to natural sounds from this alternative.

Preferred Alternative: During construction, human-caused noise would increase especially during use of an excavator and helicopter. Any sounds generated from construction would be temporary (excavator use is expected to last 4-6 weeks) and would have a moderate, short-term, adverse impact on local soundscapes. The noise generated by a helicopter used for delivery of equipment would fall within that allowed under a Programmatic Biological Assessment for Administrative Flights 2003-2007 (NPS 2004). During the preparation of the Assessment, it was determined that the park typically uses approximately 50 flights annually to conduct administrative work within the park. The flights conducted in support of this project would be counted as part of the total administrative flights flown in 2006.

Cumulative Effects: Noise from emergency repairs and ongoing building and trail repair and maintenance at this site would result in minor, long-term, localized, adverse cumulative impacts from the No Action Alternative. The rebuilding of the tent platforms would not add significantly to this project because of its short duration (i.e. two days). No additional building maintenance would be occurring in 2006 and trail work, if necessary, would be temporary; thus, in conjunction with the proposed project the cumulative effects would be negligible. In conjunction with rehabilitation work conducted in 2004 and 2005, the Preferred Alternative would result in the third consecutive summer of construction at the site. The overall result of these projects would be a moderate, long-term, localized, adverse impact on soundscapes in the area. Return visitors and local wildlife would be affected.

Conclusion: There would be minor, long-term, localized adverse impacts to natural sounds from the No Action Alternative. The Preferred Alternative would result in moderate, short-term, localized, adverse impacts on local soundscapes due to the use of an excavator and helicopter.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Recommended Wilderness

The Belly River Ranger Station Historic District is located within recommended wilderness. It is National Park Service policy to manage recommended wilderness as though it were designated wilderness. The Wilderness Act of 1964 provides the foundation for the management of wilderness and Director’s Order #41 institutes the NPS interpretation of the Act. In regards to cultural resources within wilderness DO #41 states:

“National Park Service managers are responsible for maintaining an affirmative cultural resource management program in wilderness. The cultural resource management tasks within wilderness are the same as those elsewhere, but these sites must additionally be treated in a manner that preserves other wilderness resources and character. Measures to protect and inventory cultural resources in wilderness must comply with the Wilderness Act provisions on access and use of the minimum requirement concept.”

Federal land managers are required to determine if an activity is appropriate for a wilderness area and if the activity utilizes the minimum tool required for project implementation. No motorized travel or mechanized tools are permitted in wilderness areas “. . . except as necessary to meet minimum requirements for the administration of the area for the purpose of this Act.” A Minimum Tool Analysis was completed for the proposed project.

Impact Analysis – Recommended Wilderness

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: Wilderness would not be affected or the effects would not be noticeable.

- Minor: The effect would be detectable, but would not have an appreciable effect on wilderness.

- Moderate: The effects would be readily apparent, and would result in a substantial change to the wilderness landscape that would be noticeable to the public.

- Major: The effects would be highly apparent and would change the character of the wilderness area.

- Short-term: Occurs only during construction work.

- Long-term: Occurs beyond construction period.

No Action Alternative: Under this alternative, the continued exposure of sewage in the form of gray water at the site could constitute an impact on wilderness character. The gray water and organic material act as an unnatural wildlife attractant. Maintenance and repair and emergency repairs would continue at the site resulting ongoing noise effects on the recommended wilderness. The effects of the sewage and ongoing noise would be minor, long-term, site-specific, and adverse.

Preferred Alternative: The soils at the site are extremely hard and have high clay content. It would take a crew of six people approximately 25 weeks to conduct all of the required trenching using hand tools. It was determined that with a small excavator the work could be completed in 15 weeks and allow the work to be completed within one summer season. It has been determined through a Minimum Tool Analysis that using the excavator is the preferred method of construction. This equipment is the smallest necessary to perform the work; however, it is still too large to pack in on horse and thus it would need to be flown in by helicopter. Impacts to the wilderness character of the area would primarily include noise effects associated with the use of mechanical equipment and a helicopter. The containment of sewage in the form of gray water and organic material at the site would somewhat offset the temporary adverse effects caused by the unnatural noise. The overall impacts would be moderate, short-term, localized, and adverse.

Cumulative Effects: Ongoing noise from emergency repair, structure and trail maintenance and repair would result in negligible, long-term, localized and adverse cumulative impacts to recommended wilderness. The No Action Alternative does not add a new action with which to combine with other projects. Therefore, no cumulative impacts would occur under the No Action Alternative. Structural improvements conducted in the District during the summer of 2005 increased noise levels in the area, though no helicopter flights were required. In combination with the proposed actions in the summer of 2006 there could be cumulative effects of ongoing noise through two consecutive summers. However, these impacts would not extend beyond the moderate level though they may be defined as long-term. Under the Preferred Alternative, cumulative effects would be moderate, long-term, localized, and adverse.

Conclusion: The impacts of the No Action Alternative would be minor, long-term, site-specific, and adverse due to the continued exposure of sewage in the form of gray water and organic material at the site. The Preferred Alternative would result in moderate, short-term, localized, adverse impacts to wilderness character due to noise generated by mechanical equipment and the use of a helicopter.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Visitor Experience

The Belly River Ranger Station is located approximately 6 miles from the nearest road and is one of the most remote ranger stations in Glacier National Park. The Belly River drainage is a popular destination for many backcountry visitors especially for stock users. The campground located adjacent to the Ranger Station was closed in 1998 but a new one was opened on Gable Creek approximately 600 feet from the Historic District. Though visitors are allowed to wander the Historic District, the buildings are usually locked unless a ranger is present. It is not a planned destination for most visitors and the trails in the area skirt the historic district (Map 1). Most visitors are on their way to another destination, such as Cosley Lake Campground (2.2 miles from Ranger Station) or Elizabeth Lake Campground (3.4 miles from Ranger Station). From 2000-2004, Cosley Lake campground averaged 879 camper nights (1 individual overnight stay = 1 camper night) annually, the campground at the foot of Elizabeth Lake averaged 1209 camper nights, and Gable Creek Campground averaged 366 camper nights.

Impact Analysis – Visitor Experience

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: Visitors would not be affected or changes in visitor use and/or experience would be below or at the level of detection. The visitor would not likely be aware of the effects associated with the alternative.

- Minor: Changes in visitor use and/or experience would be detectable, although the changes would be slight. The visitor would be aware of the effects associated with the alternative, but the effects would be slight.

- Moderate: Changes in visitor use and/or experience would be readily apparent. The visitor would be aware of the effects associated with the alternative.

- Major: Changes in visitor use and/or experience would be readily apparent and have important consequences. The visitor would be aware of the effects associated with the alternative.

- Short-term: Occurs only during construction work.

- Long-term: Occurs beyond construction period.

No Action Alternative: If the utilities at the Historic District continued to decline it could impact the experience for visitors to the area, especially an exposed gray water site. While visitors to the Ranger Station do not stay long impacts are expected to be moderate, long-term, site-specific, and adverse to the visitor experience.

Preferred Alternative: During the summer when the proposed work would occur, the historic site would remain accessible for visitors to experience. However, they would encounter a small construction crew and some noise disturbance during the day at the Ranger Station. The noise generated by an excavator or a helicopter would be heard at the Gable Creek Campground, and possibly for several miles along trail corridors in the valley. The presence of this construction work could negatively impact the backcountry experience for some visitors. Excavator use is expected to occur sporadically throughout the day for 4-6 weeks, and 4-6 helicopter flights are proposed to deliver materials. Work would be limited to the hours of 8 a.m. to 5 p.m. and overnight campers at Gable Creek would only be affected during the day. A beneficial impact for visitors would be that the improvements would provide for the long-term integrity of the Historic District. The majority of visitors to the Ranger Station vicinity do not stay in the area very long, and impacts to visitors should be short-term to long term, moderate, localized, adverse and beneficial. Adverse noise impacts would decrease as the visitor moved away from the Historic District. The improvements to the utilities of the Historic District would help ensure its integrity for future visitors and thus provide some beneficial impact.

Cumulative Effects: Emergency repairs combined with ongoing building and trail maintenance and repair and continued exposed gray water would result in long-term, minor, localized adverse cumulative impacts to the visitor experience. Only visitors returning to the site in 2006 that encountered construction in 2005 would be cumulatively impacted by the proposed work in 2006. The proposed use of an excavator and helicopter would help ensure additional work was not required at the site in 2007. During the construction of the tent platforms, several additional staff would be present at the site but this would only be for two days.

Conclusion: Visitors do not usually stay long at the Historic District and impacts under the No Action Alternative are expected to be negligible, long-term, site-specific, and adverse due to the continued deterioration of the District’s utilities. The Preferred Alternative would result in short-term, moderate, localized, and adverse impacts due to increased noise levels.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Health and Safety

Public and employee health and safety in the backcountry can often present unique challenges. Medical assistance, if needed, may take several hours to arrive and even minor injuries can create difficult situations. Backcountry rangers receive some training in first aid and fire fighting but are not professionals in either field. The most serious health risks at the Historic District are those related to fire, bear encounters, and unexpected sickness or injury. A portable water pump and hose are set up on Gable Creek during the summer to provide fire protection and a non-freeze hydrant is located on the back of the Ranger Station. Grizzly and black bears inhabit the Belly River Valley and are frequently seen near the Historic District. The closest road to the site is approximately 5 miles away and the closest town with a minimum medical facility is either Cardston, Alberta, Canada or Browning, MT (1 hour).

Impact Analysis – Health and Safety

Intensity Level Definitions: The thresholds for this impact assessment are as follows:

- Negligible: Health and safety would not be affected, or the effects would not be noticeable.

- Minor: The effects would be detectable, but would not have an appreciable impact on health and safety.

- Moderate: The effects would be readily apparent, and would result in a substantial change in health and safety in a manner noticeable to staff and the public.

- Major: The effects would be readily apparent, would result in a substantial change in health and safety in a manner noticeable to staff and the public and be markedly different from existing conditions.

- Short-term: Effects lasting for the duration of the project.

- Long-term: Effects lasting longer than the duration of the project.

No Action Alternative: Exposed sewage in the form of gray water and organic material at the site would continue to create unsafe conditions in several ways. The organic material may attract wildlife such as bears that can pose a danger to individuals at the site. People may also come in contact with the exposed sewage inadvertently, perhaps by touching their shoes after walking in an exposed area. Storing propane bottles within sleeping quarters, as currently occurs, presents the danger of leaking gas that can be inhaled or ignited. The impacts from this alternative would be moderate, long-term, site-specific, and adverse.

Preferred Alternative: The actions proposed under the Preferred Alternative would result in several safety improvements. Containing all sewage produced at the site would remove a wildlife attractant from the site and eliminate the possibility of someone coming in contact with the sewage. Creating a central storage and dispersal location for the propane system would remove individual propane bottles from having to be stored within living quarters. A centralized system would also reduce the possibility of injury caused by the frequent transport of heavy gas tanks to individual buildings. Overall, the impact to health and safety would be moderate, long-term, site-specific, and beneficial.

Cumulative Effects: The No Action Alternative does not add a new action with which to combine with other projects. Therefore, no cumulative impacts would occur under the No Action Alternative. Structural improvements made to the District buildings in 2005 repaired any defects, such as uneven flooring, in the structures that could have caused falls or injuries. The Preferred Alternative in combination with these repairs would be negligible, long-term, site-specific, and beneficial.

Conclusion: Under the No Action Alternative impacts would be moderate, long-term, site-specific, and adverse due to the perpetuation of exposed sewage as gray water and the use of propane bottles within living quarters. The impact to health and safety would be moderate, long-term, site-specific, and beneficial from the Preferred Alternative due to the containment of sewage in the form of gray water and the consolidation of propane tanks separate from living quarters.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

# Summary of Consultation and Compliance with Federal and State Regulations

Preparers and Consultants

Cory Davis, Environmental Compliance Technician, Glacier National Park

Jim Foster, Civil Engineer, Glacier National Park

Steve Gniadek, Wildlife Biologist, Glacier National Park

Lon Johnson, Historical Architect, Cultural Resource Specialist, Glacier National Park

Bill Michels, Air & Water Quality Resource Specialist, Glacier National Park

Jack Polzin, Maintenance Mechanic Supervisor/Historic Structure Specialist

Mary Riddle, Environmental Protection and Compliance Officer, Glacier National Park

John Waller, Wildlife Biologist, Glacier National Park

Del Zimmerli, Facility Manager, Glacier National Park

List of Environmental Assessment Recipients

Norman and Jean Adams

Advisory Council on Historic Preservation

Max Baucus, United States Senate

Conrad Burns, United States Senate

John Case

Burlington Northern Santa Fe Railroad

Chair, Flathead County Board of Commissioners

Coalition for Canyon Preservation

Flathead Basin Commission

Flathead National Forest

Friends of the Wild Swan

Mr. and Mrs. Galvin

Glacier County Commissioners

Glacier Natural History Association

Glacier Raft Company

Great Northern Whitewater Resort

Jack and Reggie Hoag

James K. Johnson

Bill and Bob Lundgren

Ev and Margaret Lundgren

Mayor of Browning Montana

Mayors and City Councils of Kalispell, Columbia Falls, and Whitefish

Montana Department of Environmental Quality, Board of Environmental Review

Montana Department of Environmental Quality, Permitting & Compliance, Helena

Montana Department of Environmental Quality, Water Protection Bureau

Montana Department of Natural Resources and Conservation

Montana Environmental Information Center

Montana Fish, Wildlife, and Parks, Region One Supervisor, Kalispell

Montana Intergovernmental Clearing Office of Budget and Planning

Montana Preservation Alliance

Montana State Clearinghouse

Montana State Historic Preservation Office

Montana Wilderness Association

Pat and Riley McClelland

National Parks Conservation Association

Public Libraries: Kalispell, Whitefish, Columbia Falls, Helena, Butte, Browning, Bozeman, Great Falls, Missoula, Bigfork, and Lethbridge, Alberta, Canada

Dennis Rehberg, United States House of Representatives, Missoula Offices

Brian Schweitzer, Governor of Montana

Mike Snyder, Regional Director, Intermountain Region, National Park Service, Denver

James Steele, Chair, Confederated Salish and Kootenai Tribes of the Flathead Reservation

w/copies to Tribal Council and Confederated Salish and Kootenai Tribal Preservation Department

Pat Thomas, Chair, Blackfeet Tribal Business Council

w/copies to Tribal Council and the Blackfeet Tribal Cultural Liaison

U.S. Army Corps of Engineers

U.S. Environmental Protection Agency

U.S. Fish and Wildlife Service (Helena and Creston)

U.S. Geological Survey, Biological Resources Division

Waterton Lakes National Park

Wilderness Watch

Summary of Compliance with Federal and State Regulations

National Environmental Policy Act (NEPA) and Regulations of the Council on Environmental Quality – The National Environmental Policy Act applies to major federal actions that may significantly affect the quality of the human environment. This generally includes major construction activities that involve the use of federal lands or facilities, federal funding, or federal authorizations. If the environmental effects are undetermined then an Environmental Assessment is prepared to evaluate potential impacts. This Environmental Assessment meets the requirements of the NEPA and regulations on the Council on Environmental Quality in evaluating potential effects associated with activities on federal lands. If no significant effects are identified a finding of no significant impact (FONSI) would be prepared. If significant impacts are identified, then a notice of intent (NOI) would be filed for preparation of an Environmental Impact Statement.

The Endangered Species Act of 1973, as amended (16 U.S.C. 1531 et seq.) – Section 7 of the Endangered Species Act is designed to ensure that any action authorized, funded, or carried out by a federal agency likely would not jeopardize the continued existence of any endangered or threatened plant or animal species. If a federal action may affect threatened or endangered species, then consultation with the U.S. Fish and Wildlife Service is required. Glacier National Park biologists have determined that this project MAY AFFECT, NOT LIKELY TO ADVERSELY AFFECT federally listed species. The reason for a determination of “may affect” is due to the proposed helicopter flights for delivering materials to the site. Consultation has been completed with the US Fish and Wildlife Service.

Clean Water Act (CWA) and State and Local Water Quality and Floodplain Regulations—No altering of streams or floodplains would occur. No permits are required.

Executive Order 11990, Protection of Wetlands-There are no wetlands present in areas where the work would be conducted. The work would occur along Gable Creek which flows into the Belly River. There are wetlands along the nearby Belly River. The only potential impacts to wetlands would be beneficial from the improvements to the sewage system. Therefore, a Statement of Findings for Wetlands will not be completed.

National Historic Preservation Act of 1996, as amended (16 U.S.C. 470, et Seq.) – Section 106 of the National Historic Preservation Act of 1966 (as amended) requires federal agencies to identify cultural resources within the area of potential effect for an undertaking and to consider effects of any federal action on cultural resources eligible for or listed in the National Register of Historic Places (NHRP), prior to initiating such actions. This Environmental Assessment/Finding of Effect will provide documentation of compliance with Section 106 to the State Historic Preservation Office.

# References

Claar, J.J., N. Anderson, D. Boyd, M. Cherry, B.Conard, R. Hompesch, S. Miller, G. Olson, H. Ihsle Pac, J. Waller, T. Wittinger, H. Youmans. 1999. Carnivores. Pages 7.1-7.63 In Joslin, G. and H. Youmans, coordinators. Effects of recreation on Rocky Mountain wildlife: A review for Montana. Committee on Effects of Recreation on Wildlife. Montana Chapter of the Wildlife Society. 307 pp.

Czech, B. 1991. Elk behavior in response to human disturbance at Mount St. Helens National Volcanic Monument. Applied Animal Behavior Science, 29:269-277.

Dutton, B.L., J. Hadlock, M. Arthur, D. Marrett, A. Goldin, and A. Zhu. 2001. Soils of Glacier National Park. Prepared by Land and Water Consulting, Inc. Missoula, MT.

Harding, L. and J.A. Nagy.  1980.  Responses of grizzly bears to hydrocarbon exploration of Richards Island, Northwest Territories, Canada.  In Bears- Their Biology and Management; a selection of papers from the Fourth International Conference on Bear Research and Management (1977), Kalispell, MT. Pages 277-280.

Historical Research Associates (Missoula, Montana). 1985. Multiple Resource Submission for Historically and Architecturally Significant Resources in Glacier National Park, Montana. National Register of Historic Places Inventory -Nomination Form. Listed in the National Register of Historic Places on February 14, 1986.

Interagency Grizzly Bear Committee (IGBC). 1986. Interagency Grizzly Bear Guidelines. U.S. Forest Service, Washington D.C.  100 pp.

Kendall, K.C.  1986.  Grizzly bear population trend studies, Apgar Mountains, Glacier National Park, Progress Report. USDI National Park Service, Glacier National Park, West Glacier, MT.  41pp.

Klein, D.R. 1974. The reaction of some northern mammals to aircraft disturbance. Pages 377-383. In XIth International Congress of Game Biologists. National Swedish Environment Protection Board, Stockholm.

Larkin, R. P., L. L. Pater and D. J. Tazik.  1996.  Effects of military noise on wildlife:  a literature review. USACERL Technical Report 96/21, 83 pp., U.S. Army Construction Engineering Research Laboratory, Champaign, Illinois.

McLellan, B.N. and D.M. Shackleton. 1989. Grizzly bears and resource-extraction industries: habitat displacement in response to seismic exploration, timber harvesting and road maintenance. Journal of Applied Ecology 26:371-380.

National Park Service (NPS). 1999. General Management Plan. Glacier National Park, West Glacier, MT. 340pp.

——. 2000a. Management Policies 2001. NPS D1416. Washington D.C.

——. 2000b. Aviation Management Plan. Glacier National Park, West Glacier, MT.  16pp.

——. 2004. Programmatic Biological Assessment for Administrative Flights 2003-2007. Glacier National Park, West Glacier, MT.

Reeves, Brian, Ph.D.  1996.  Glacier National Park, Archaeological Inventory and Assessment, 1994 Field Season Final Report, Part 1:  Inventory.

——.  2003. Mistakis: The Archaeology of Waterton-Glacier International Peace Park, Archaeological Inventory and Assessment Program, 1993-1996, Final Technical Report, Volumes I and II, eds. Leslie B. Davis, Claire Bourges. In fulfillment of Contract 290847 under Cooperative Agreement No. CA-1268-1-9017 between Montana State University and the National Park Service, 2003.

Riley, L.M. 2005. Belly River Ranger Station Historic District Cultural Resources Survey and Test Excavations. Glacier National Park, West Glacier, MT.

Thiel, R. P., S. Merrill and L. D. Mech. 1998. Tolerance by denning wolves, Canis lupus, to human disturbance. Canadian Field-Naturalist 112(2): 340-342.

USFWS. 1993. Grizzly bear recovery plan. USFWS, Missoula, MT. 181 pp.

# Appendix A: Special Status Wildlife Species in Glacier National Park

| Common Name | Latin Name | Status |
| --- | --- | --- |
| Invertebrates |  |  |
| Rocky Mountain capshell | Acroloxus coloradensis | G3S1 |
| Fish |  |  |
| Bull trout | Salvelinus confluentus | FT, G3S2 |
| Westslope cutthroat trout | Oncorhynchus clarki lewisi | G4T3S2, USFS |
| Spoonhead sculpin | Cottus ricei | G5S3 |
| Trout-perch | Percopsis omiscomaycus | G5S2 |
| Reptiles and Amphibians |  |  |
| Western toad | Bufo boreas | G4S2, USFS |
| Birds |  |  |
| Trumpeter swan | Cygnus buccinator | G4S2, USFS, PIF1 |
| Harlequin duck | Histrionicus histrionicus | G4S2B, USFS, PIF1 |
| Barrow’s goldeneye | Bucephala islandica | PIF2 |
| Hooded merganser | Lophodytes cucullatus | PIF2 |
| Ruffed grouse | Bonasa umbellus | PIF2 |
| White-tailed ptarmigan | Lagopus leucurus | G5S3, PIF3 |
| Common loon | Gavia immer | G5S2B, USFS, PIF1 |
| Horned grebe | Podiceps auritus | PIF2 |
| American white pelican | Pelecanus erythrorhynchos | G3S3B, PIF3 |
| Black-crowned night heron | Nycticorax nycticorax | G5S3B, PIF3 |
| Bald eagle | Haliaeetus leucocephalus | FT, G5S3 |
| Northern goshawk | Accipiter gentilis | G5S3, USFS, PIF2 |
| Ferruginous hawk | Buteo regalis | G4S2B, PIF2 |
| Golden eagle | Aquila chrysaetos | BEPA |
| Peregrine falcon | Falco peregrinus | G5S2B, USFS, PIF2 |
| Long-billed curlew | Numenius americanus | G5S2B, PIF2 |
| Marbled godwit | Limosa fedoa | PIF2 |
| Franklin’s gull | Larus pipixcan | G4G5S3B, PIF2 |
| Caspian tern | Sterna caspia | G5S2B, PIF2 |
| Common tern | Sterna hirundo | G5S3B, PIF2 |
| Forster’s tern | Sterna fosteri | G5S2B, PIF2 |
| Black tern | Chlidonias niger | G4S3B, PIF2 |
| Northern hawk owl | Surnia ulula | G5S1 |
| Great gray owl | Strix nebulosa | G5S3, PIF3 |
| Boreal owl | Aegolius funereus | PIF3 |
| Black swift | Cypseloides niger | G4S3B, PIF2 |
| Vaux’s swift | Chaetura vauxi | PIF2 |
| Calliope hummingbird | Stellula calliope | PIF2 |
| Lewis’s woodpecker | Melanerpes lewis | G4S2B, PIF2 |
| Williamson’s sapsucker | Sphyrapicus thyroideus | PIF2 |
| Three-toed woodpecker | Picoides tridactylus | PIF2 |
| Black-backed woodpecker | Picoides arcticus | G5S2, USFS, PIF1 |
| Pileated woodpecker | Dryocopus pileatus | PIF2 |
| Olive-sided flycatcher | Nattallornis borealis | PIF1 |
| Willow flycatcher | Empidonax traillii | PIF2 |
| Hammond’s flycatcher | Empidonax hammondii | PIF2 |
| Cordilleran flycatcher | Empidonax difficilis | PIF2 |
| Red-eyed vireo | Vireo olivaceus | PIF2 |
| Loggerhead shrike | Lanius ludovicianus | G4S3B, PIF2 |
| Clark’s nutcracker | Nucifraga columbiana | PIF3 |
| Brown creeper | Certhia americana | PIF1 |
| Winter wren | Troglodytes troglodytes | PIF2 |
| Veery | Catharus fuscescens | PIF2 |
| Brewer’s sparrow | Spizella breweri | G5S2B, PIF2 |
| Lark bunting | Calamospiza melanocorys | G5S3B, PIF2 |
| LeConte's sparrow | Ammodramus leconteii | G4S1S2B, PIF3 |
| McCown’s longspur | Calcarius mccownii | G4S2B, PIF2 |
| Chestnut-collared longspur | Calcarius ornatus | G5S3B, PIF2 |
| Lazuli bunting | Passerina amoena | PIF2 |
| Mammals |  |  |
| Northern bog lemming | Synaptomys borealis | G4S2, USFS |
| Swift fox | Vulpes velox | G3S3 |
| Fisher | Martes pennanti | G5S3, USFS |
| Wolverine | Gulo gulo | G4T4, USFS |
| Rocky Mountain bighorn sheep | Ovis canadensis | G4S4 |
| Townsends’ big-eared bat | Corynorhinus townsedii | G4S2, USFS |
| Canada lynx | Lynx canadensis | FT, G5S3 |
| Gray wolf | Canis lupus | FE, G4S3 |
| Grizzly bear | Ursus arctos horribilis | FT, G4T3T4 |
| FE = federally listed as Endangered, FT = federally listed as Threatened; G = rangewide status, S = statewide status, B = rank refers to the breeding population of the species in Montana, T = subspecies status; 3 = Potentially at risk because of limited and/or declining numbers, range, and/or habitat, even though it may be abundant in some areas; 4 = Uncommon but not rare (although it may be rare in parts of its range), and usually widespread, apparently not vulnerable in most of its range, but possibly cause for long-term concern; 5 = Common, widespread, and abundant (although it may be rare in parts of its range); USFS = considered sensitive by US Forest Service; PIF1 = Partners in Flight Priority Level 1 Species, Montana should implement conservation actions, PIF2 = Partners in Flight Priority Level 2 Species, Montana should monitor the status of these species, and/or design conservation actions, PIF3 = Partners in Flight Priority Level 3 Species, species of concern which are not in imminent risk, or which are near-obligates for high priority habitat. |  |  |