- INTRODUCTION

- Purpose and Need

The purpose of the aerial application of herbicide analyzed in this Environmental Assessment (EA) is to contribute to the restoration of natural fire regimes and ecosystem processes in the Kolob burned area inside Zion National Park (ZION). This action would further goals and desired future conditions identified in the General Management Plan (GMP) (USDI NPS 2001c) and Fire Management Plan (FMP) (USDI NPS 2005):

- GMP Mission Goal: Maintain the resource, including plant and animal communities, at healthy and viable levels consistent with natural processes.

- FMP Goal: Prevent and suppress unwanted fires using effective strategies and methods under the decision process of sound risk management.

- FMP Desired future condition: Vegetation succession reflects the natural range of variability under conditions that would occur under historical fire regimes.

- FMP Desired future condition: Native wildlife habitat is maintained, restored, or enhanced through fire management practices that are consistent with natural processes.

The intent of the aerial application of herbicide is to interrupt the grass-fire cycle (Brooks et al. 2004, D’Antonio and Vitousek 1992) that is perpetuated by invasive annual grasses collectively referred to in this document as “cheatgrass,” including Bromus tectorum, B. rubens, B. diandrus, and B. japonicus. Non-native cheatgrass increases in abundance and density after fire, resulting in increased fuel loads and fuel continuity, which in turn create a receptive environment for future fires. As cheatgrass continues to invade and increase after each fire, the time between fires becomes shorter. Since the native shrubs and trees are slower to re-establish after fire and need many years between fire events to complete their lifecycles, the increased fire frequency fueled by cheatgrass eventually eliminates most of the native shrubs and trees from the landscape. Cheatgrass also displaces the native grasses and herbaceous (non-woody) plants because as a winter annual, cheatgrass is able to establish earlier in the growing season than most native grasses and herbaceous plants. In this way, cheatgrass depletes soil moisture and competes against the native species until the native species are eventually crowded out of large areas as the grass-fire cycle continues. Similar to its effects on shrub and tree species, grasses and herbaceous species that are intolerant of frequent fire are eventually eliminated from the landscape by the fires carried by cheatgrass. As the grass-fire cycle is perpetuated, the fire frequency increases, eliminating native species adapted to a longer term fire return interval.

A treatment is needed to interrupt the grass-fire cycle that has already been established, but has not yet eliminated the native seed beds. This interruption should reduce cheatgrass establishment over many growing seasons, thus allowing the native plants to successfully re-establish and persist in the burned area. The re-establishment of native vegetation would then restore habitat needed to support native wildlife and perpetuate natural ecosystem processes.

Action to interrupt the grass-fire cycle is being taken at this time in response to the Kolob Fire. This human-caused fire started on June 24, 2006 and over the next six days burned a total of 17,632 acres, including 10,615 acres in the southwest corner of ZION. This was the largest fire in the park’s history, and almost surpassed the total acres burned in the park since 1950. The vegetation in the burned area primarily consisted of pinyon-juniper woodland, along with shrublands, grasslands, riparian corridors and bare rock formations. While there were still many in-tact native plant communities in the burned area prior to the fire, there were also populations of cheatgrass that served to carry the fire, thus the Kolob Fire represents the initiation of the grass-fire cycle at a landscape level in ZION.

Dense stands of cheatgrass continue to persist immediately adjacent to the burned area, particularly to the north along the Kolob Terrace Road and along the west boundary of the park, as well as in the interior of the burned area where pockets of unburned vegetation persist. The National Park Service (NPS) is concerned that these seed sources, coupled with the cheatgrass seeds that remain in the soil in the burned area, would allow the cheatgrass to quickly re-establish and flourish in the burned area. Such an event would be highly detrimental to the recovery of native plants in the burned area and would result in long-term habitat degradation as the grass-fire cycle would gain in strength and persist for many years to come. The best opportunity to prevent the establishment of cheatgrass is in fall of 2006, when a herbicide could be applied at a landscape level with relatively complete coverage (except for identified no-spray zones) to prevent germination and growth of cheatgrass in the winter and early spring, thus allowing the native plants an opportunity to re-establish themselves in the burned area. For these reasons, ZION feels compelled to take action now to restore and preserve the natural vegetation communities in the Kolob Fire burned area.

## Location and Description

The Kolob Fire project area is located in the southwest corner of ZION. The fire primarily burned lands in the North Creek watershed of the Virgin River, approximately 5 miles northeast of Virgin, Utah in Washington County. The landscape of the burned area is characterized by highly eroded badlands, steep rock land that includes areas of shallow soils and nearly vertical sandstone cliffs, mesas, incised canyons with relatively narrow floodplains and stream terraces, and volcanic cinder cones and basalt lava flows. Elevation ranges from about 3,600 feet near State Route 9 at the southern end of the fire to about 6,900 feet in the northeastern part of the fire.

ZION is 148,024 acres in size and is located on the southwestern edge of the Colorado Plateau (Figure 1). The park lies in portions of three counties in Utah — Washington, Iron, and Kane. The northwest corner of the park is approximately 260 miles southwest of Salt Lake City, Utah. Interstate 15 is located to the west of the park. High plateaus, a maze of narrow, deep sandstone canyons, and striking rock towers and mesas characterize the park. The lowest elevation in the park, 3,666 feet, is found at Coalpits Wash in the southwest corner in the Kolob Fire burned area. The highest elevation, 8,726 feet, is Horse Ranch Mountain in the Kolob Canyons section to the north of the burned area.

- Relationship to Other Plans

This EA is tiered consecutively to three approved plans and the relevance of each is described in more detail below. The three plans are:

- General Management Plan for Zion National Park

- Fire Management Plan for Zion National Park

- Kolob Fire Burned Area Rehabilitation Plan

- Environmental Assessment of Aerial Application of Herbicide

The ZION GMP was completed in 2001 (USDI NPS 2001c) identified zones that define how different areas of the park will be managed to achieve desired resource and social conditions and to serve recreational needs. The park is divided into seven zones: Frontcountry High Development, Frontcountry Low Development, Transition, Primitive, Pristine, Research Natural Areas, and Administrative. The majority of the project area occurred in lands zoned as Pristine. The trail corridors in the burned area are zoned as Primitive and the Kolob Terrace Road corridor is zoned as Frontcountry Low Development. A powerline corridor and the Coalpits helispot in the southern portion of the project area are both zoned as Administrative. These four zones are defined below.

The Pristine Zone (10,433 acres in the project area) encompasses remote expanses of land within the park. The zone emphasizes a natural landscape, free of all signs of people, except for faint routes. Natural conditions and process will largely be undisturbed by people.

The Primitive Zone (72 acres in the project area) includes backcountry trails and popular hiking routes throughout the park. The zone emphasizes a natural landscape, where visitors experience the park on unpaved trails and routes. The zone is a largely undisturbed landscape, with natural processes predominating.

The Frontcountry Low Development Zone (89 acres in the project area) includes the Kolob Terrace Road and trailheads. Visitor experience in this zone is fairly structured, rural, and oriented around motorized sightseeing on secondary roads, camping, picnicking, and short walks. Natural conditions are unmodified in most of the zone.

The Administrative Zone (21 acres in the project area) includes the areas that support park management and administration. These areas are not typically used by visitors. Natural processes and landscapes can be altered in this zone to support park operations.

The ZION FMP (USDI NPS 2005) was completed in 2005 and describes the park’s fire management strategy and operational concerns. It also identifies concerns with the grass-fire cycle and subsequent alteration of natural fire regimes. The environmental assessment prepared for the FMP provides for the use of herbicide to treat non-native species, including the treatment of cheatgrass with imazapic, but it does not fully address potential impacts due to aerial application.

The Kolob Fire Burned Area Rehabilitation (BAR) Plan (USDI NPS 2006) was completed in July 2006 immediately following containment of the Kolob Fire. This plan prescribes rehabilitation recommendations for all lands burned within the Kolob Fire perimeter and downstream impact areas. The primary objectives of the Kolob Fire BAR Plan are to repair or improve lands unlikely to recover naturally from severe wildland fire; restore or establish healthy, stable ecosystems; and repair or replace fire damaged minor operating facilities.

## Laws, Policies, and Authorities

The following regulations and guidance documents relate directly to the completion of this EA.

National Environmental Policy Act (NEPA) – The purpose of NEPA is to encourage productive and enjoyable harmony between humans and the environment; to promote efforts that would prevent or eliminate damage to the environment and stimulate the health and welfare of mankind; and to enrich the understanding of the ecological systems and natural resources important to the nation. NEPA requirements are satisfied by successful completion of an environmental assessment or environmental impact statement, in addition to a decision document.

Director’s Order-12 (DO-12) – Conservation Planning, Environmental Impact Analysis, and Decision Making. DO-12 outlines the NPS guidelines for implementing NEPA according to NPS regulations. DO-12 meets all Council on Environmental Quality (CEQ) regulations for implementing NEPA.

NPS Organic Act 1916 – Congress directed the U.S. Department of the Interior and the NPS to manage units “to conserve the scenery and the natural and historic objects and the wild life therein and to provide for the enjoyment of the same in such manner and by such means as will leave them unimpaired for the enjoyment of future generations.” (16 United States Code (USC) § 1)  Congress reiterated this mandate in the Redwood National Park Expansion Act of 1978 by stating that the NPS must conduct its actions in a manner that will ensure no “derogation of the values and purposes for which these various areas have been established, except as may have been or shall be directly and specifically provided by Congress.” (16 USC § 1 a-1)

Director’s Order-18 (DO-18) – NPS guidance for Wildland Fire Management (USDI NPS 2002b).

Review and Update of the 1995 Federal Wildland Fire Management Policy 2001 – provides guidance and updates for federal fire managers (USDI 2001).

In addition to the regulations and orders listed above, other regulations and policies guide the assessment of impacts. These are listed below:

- NPS Management Policies 2001 – defines how the NPS will meet its park management responsibilities under the 1916 NPS Organic Act (USDI NPS 2001b).

- Utah Water Quality Regulations – conserves waters of the state to protect, maintain, and improve water quality.

- Wild and Scenic Rivers Act – provides for designation and protection of wild, scenic, and recreational rivers.

- Executive Order 11990 – provides for the protection of wetlands.

- Clean Water Act – provides for the protection of waters of the United States.

- Endangered Species Act (ESA)/Section 7 – provides for the listing and protection of endangered and threatened species and their critical habitat; requires consultation under Section 7 if any listed species may be adversely affected.

- Wilderness Act of 1964 –states that wilderness areas shall be administered for the use and enjoyment of all people in a manner that will leave them unimpaired for future use and enjoyment as wilderness. Ninety percent of Zion was proposed to Congress as wilderness in 1974.

- Director’s Order-41 – states that proposed wilderness areas are to be managed to preserve their wilderness character and values.

- National Historic Preservation Act (NHPA)/Section 106 – provides for the identification and protection of historic sites and structures; requires consultation under Section 106 with the Utah State Historic Preservation Officer (SHPO).

## Issues and Impact Topics Analyzed in Detail

An issue describes a relationship between an action and an environmental resource. Issues associated with the proposed aerial application of herbicide were identified through internal and external scoping. Internal scoping included an interdisciplinary team with a member representing each affected division and resource group within the park (refer to the List of Preparers in the Consultation and Coordination section of this document). External scoping was accomplished through a press release inviting concerns and comments from the public and a letter was mailed to interested members of the public also inviting their participation (refer to Consultation and Coordination section of this document). Through issue identification, impact topics were also identified. The impact topics are listed below, followed by an issue statement. Each impact topic is described in the Affected Environment section and is analyzed in the Environmental Consequences section of this document.

Vegetation

- Invasive plants other than cheatgrass also occur in the project area that could be affected if no action is taken or if the proposed treatment is implemented.

- Native plants occur within the project area that could be affected if no action is taken or if the proposed treatment is implemented.

- The grass-fire cycle would likely lead to an increase in fire frequency, fire size, and fire intensity if no action is taken.

- Type conversion from native vegetation communities to exotic grasslands may occur if no action is taken.

- Loss of habitat for many native plants may occur if no action is taken.

- If treatment occurs, it will be essential to monitor the results to determine actual effectiveness and if any unanticipated effects occur.

Threatened, Endangered, and Sensitive Plant Species

- State listed plant species occur in or near the project area that could be affected if no action is taken or if the proposed treatment is implemented.

Wildlife

- Bioaccumulation of toxins derived from the herbicide could be a concern.

- Loss of habitat for native animals may occur if no action is taken.

Threatened, Endangered, and Sensitive Animal Species

- The project area includes critical habitat for the Mexican spotted owl and this federally listed species could be affected if no action is taken or if the proposed treatment is implemented.

- Sensitive fish species occur downstream of the project area and could be affected if no action is taken or if the proposed treatment is implemented.

- Bioaccumulation of toxins derived from the herbicide could be a concern.

Soils

- Potential herbicide mobilization within the soil could have unintended consequences.

Water Resources (Watershed and Streamflow, Water Quality, Sediment Yield, Wetlands)

- Water corridors and riparian zones could be affected if no action is taken or if the proposed treatment is implemented.

- Sediment yield could be affected if no action is taken or if the proposed treatment is implemented.

- Herbicide drift and/or herbicide mobilization within groundwater could have unintended consequences.

Natural Soundscapes

- Noise would be generated by the helicopter during treatment, including the potential for overflights on the Kolob Terrace Road.

Wilderness

- Large-scale herbicide applications in a national park and in a wilderness area require careful consideration to determine appropriateness and consistency with management policies.

- Wilderness values and/or character could be affected if no action is taken or if the proposed treatment is implemented.

Public Health and Safety

- Visitor/public safety would need to be secured during treatment.

- Neighbors and lands bordering the park should be consulted prior to treatment.

- Communication with local communities is needed whether or not treatment is undertaken.

- Aviation safety will need to be planned if treatment is undertaken.

- Interagency coordination is needed to fairly consider potential impacts if no action is taken or if the proposed treatment is implemented.

Visitor Experience

- Visitor experience would be affected short-term by the necessary public use closure during treatment and could also be affected long-term if no action is taken or if the proposed treatment is implemented.

## Issues Considered and Dismissed from Further Consideration

The following issues were eliminated from further analysis for the reasons stated below.

Air Quality

The air quality in ZION is protected under the Clean Air Act as a Class I airshed. However, any deterioration in air quality as a result of the proposed action would occur only during herbicide application and thus would be of very short duration and limited to a very localized areas. There are essentially no potential for measurable impacts to air quality as a result of this project, so this issue is not further addressed.

Cultural Resources

Cultural resources include archeological sites and artifacts; historic structures, sites, and artifacts; ethnographic resources; cultural landscapes; and museum collections. The BAR Plan (USDI NPS 2006) stated that “there were no effects to cultural resources as the result of fire suppression activities” and there were no recommendations for rehabilitation. There are known archeological sites and historic sites in the project area. Since the proposed action does not require any surface disturbing activities and the act of flying a helicopter over these sites would have no effect on the sites and the herbicide would not affect the sites, this issue was dismissed from further consideration. There are no known cultural landscapes or ethnographic resources in the area, so these issues were dismissed from further consideration.

Economic Considerations

Economic impacts are limited to the letting of a contract to conduct the aerial herbicide application. While the contract would be substantial in dollar value, the specialized nature of the contract requirements would essentially limit the potential bidders to contractors from outside the local area, and thus the economic impact would be greatly diluted. The timing and location of the proposed treatment and the subsequent limitations to visitor use during application have been designed to avoid impacts to concessions and local businesses. Due to these circumstances, it was determined that economic impacts should not be further addressed.

Park Administration and Facilities

No park facilities would be impacted by the proposed treatment, except for short-term occupation of the park’s firing range by the helicopter and support vehicles during the application. The funding for the proposed treatment comes from Burned Area Rehabilitation funding and does not affect the park’s base operating budget so it would have no impact to on-going administration of the park. The management of the contract would be handled as part of assigned duties for the park’s contracting officer, aviation officer, and contracting officer’s representative, but would not have a noticeable impact on park administration. For these reasons, this topic is not further addressed.

Ecologically Critical Areas

Impacts related to proposed wild and scenic rivers are covered under the Water Quality section. No other ecologically critical areas are known in the project area. So this issue is not further addressed.

###### Floodplains

No floodplain functions would be affected by activities proposed in this EA. Functions and values related to wetlands or riparian areas located in or near floodplains are addressed under Water Resources.

Prime and Unique Farmlands

No prime and unique farmlands occur in the park or the near vicinity. So this topic was dismissed from further consideration.

Energy Requirements/Depletable Resource Requirements and Conservation Potential

None of the alternatives would affect energy depletable resource requirements or conservation potential to the extent that detailed analysis would be required.

Environmental Justice

None of the alternatives would have a disproportionate adverse health or environmental effect on minorities and/or low-income populations and communities. So this topic was dismissed from further consideration.

Indian Trust Resources

Secretarial Order 3175 requires that any anticipated impacts to Indian trust resources from a proposed project or action by the U.S. Department of the Interior agencies be explicitly addressed in environmental documents. No Indian trust resources occur within the park. The lands comprising Zion are not held in trust by the Secretary of the Interior for the benefit of Indians due to their status as Indians. Therefore, Indian trust resources were dismissed as an impact topic.

- Figure 1 – General Location

- ALTERNATIVES

## Introduction

This section describes the alternatives analyzed in this document – the No Action and the Proposed Action as well as alternatives considered but dismissed from further consideration.

## Alternative A – No Action Alternative

Under the No Action alternative, the aerial application of herbicide would not occur and the park would continue with existing management actions in the burned area. Such actions could include spot treatment of herbicide using ground-based methods, replanting native species, and other actions to alter the vegetation community. However, since no aerial application of herbicide would occur, no large-scale efforts would be made to interrupt the grass-fire cycle at this time.

Over time the no action alternative would most likely lead to perpetuation of the grass-fire cycle. It is expected that cheatgrass would quickly re-invade the burned area, with dense stands of cheatgrass most likely to be established in the next few years along roads, trails, and the western boundary of the park. Interior areas would not be spared cheatgrass invasion, but it would likely be slower to dominate due to fewer cheatgrass seeds in the soil and more competition from native plants. Over the next few decades, cheatgrass could come to dominate much of the burned area. In response to this increasing density of cheatgrass, fire frequency, fire size, and fire intensity would continue to increase, further accelerating the loss of native plant communities. While some native plants would continue to persist, eventually most of the native plant communities and their myriad wildlife habitats would be degraded and those communities that are intolerant of frequent fire would become absent from the landscape. The result would be a permanent vegetation type conversion from native shrublands and woodlands to an invasive grassland. Such conversions have been well documented in northern Great Basin.

The resulting invasive grasslands are both created by increasing fire frequency, fire size, and fire intensity and serve to perpetuate large and frequent fires. The continuous fuels created by the invasive grasses means that more ignition sources (i.e. lightning, cigarettes, vehicle sparks) would strike receptive fuels and start a fire. Furthermore, those continuous fuels also serve to carry the resulting fires over larger areas. Thus fires become larger and tend to spread faster as they carry through the light, flashy fuelbed formed by cured cheatgrass and its thatch layer. The increased frequency and size of fires would make it more difficult to control future fires and protect other values of concern from being burned, such as homes, utility systems, owl nesting sites, riparian corridors, and cultural sites.

## Alternative B – Proposed Action/Preferred Alternative

The proposed action is the aerial application of herbicide on up to 10,280 acres of the Kolob Fire burned area within ZION (Figure 2), for the purpose of controlling cheatgrass which would interrupt the grass-fire cycle and thereby restore native plant communities and wildlife habitat. The operational elements of the proposal are bulleted below for easy reference and are further elaborated in the following text.

- Treatment Targets: The entire burned area within the park would be treated with the exception of 335 acres of designated no-spray zones which include 100 feet on either side of riparian corridors and surface waters and some control plots to be established in the project area for effectiveness monitoring.

- Herbicide: Imazapic, trade name Plateau®, is an herbicide that has been proven effective in cheatgrass control. Methylated seed oil such as MSO® Concentrate and a penetrant and drift control agent such as Liberate® could be added to the herbicide mixture. (Refer to Appendix 1)

- Application Method: A helicopter that is specially equipped for herbicide application and operated by a pilot that is qualified for herbicide application would be used for the application. All applicators would carry required credentials for the State of Utah and the Department of the Interior. The helicopter application equipment and flight patterns are designed to minimize spray drift.

- Application Rate: For Plateau® a maximum rate of 8 ounces per acre would be used throughout the treatment area, with the exception of the no-spray zones. The recommended rate for MSO would be 1-quart per acre and the recommended rate for the drift control agent would be 8-pints per 100-gallons.

- Helicopter Support Area: The existing Coalpits helispot would be used. The helispot is inside the burned area and inside the park, with easy access from Highway 9 for fuel and water support. This helispot would serve as the base of operations where the herbicide would be mixed, and the helicopter would be loaded, fueled, and secured when not in use.

- Timing of application: Ideally the treatment would occur in late October or early November, but any date after October 24, 2006 would be considered based on weather condition and the stage of growth of plants.

- Duration of treatment: The initial treatment is expected to take 2 weeks, depending on weather conditions that may affect actual hours of flight time per day.

- Monitoring: The results of the treatment would be scientifically monitored by researchers at Northern Arizona University and the U.S. Geological Survey to determine response of both cheatgrass and native plants in both treated and untreated areas.

- Associated seeding: Native seed would be applied along the Kolob Terrace Road in the project area to assure that there are adequate seeds to germinate and grow once the cheatgrass is suppressed. A native seed increase program is planned to provide additional seed resources for future seeding needs on this project area.

- Frequency of treatment: The area would be treated once initially, with the potential for follow-up treatments in subsequent years depending on what the monitoring results indicate would be most effective in restoring native plant communities.

Fall aerial herbicide application of imazapic is the best chance the park has of slowing an exponential increase of cheatgrass in the area burned by the Kolob Fire. As most of the burned area had at least trace amounts of cheatgrass before the fire, it is expected with the increase nutrients and bare ground that cheatgrass would quickly come to dominate much of the area post fire. There are native plant species in the burned area that can be expected to recover and flourish after the treatment without the competition of cheatgrass.

Results of a study released in 2002 by BASF and Synergy Resource Solutions Inc. show that fire intensity can be significantly reduced in cheatgrass-infested areas treated by Plateau® (Kury et al. 2002). The study found that the height of flames in treated areas can be reduced by as much as 88 percent and the rate at which the fire spreads can be lowered by as much as 95 percent, compared to untreated areas.

Research initiated by park staff, USGS scientist Matt Brooks and Lake Mead Restoration Biologist, Curt Deuser with funding from Joint Fire Science examined the effects of fire, seed and Plateau® (Louie et al 2005). The treatments were initiated in the fall of 2005. Preliminary results show that fire followed by a fall season Plateau® application was effective in reducing cheatgrass and allowing seed naturally found in the soil and seeded native perennials to occupy the site.

Plateau® is non-restricted use herbicide that attacks a specific enzyme found only in plants to control growth (BASF 2004, BASF 2006). The active ingredient in Plateau®, imazapic, is not mutagenic or teratatogenic and would not be expected to have any adverse effect on big game and non-game species when used as labeled (BASF 2004, BASF 2006). It is considered to be nontoxic to mammals, birds, fish, and aquatic invertebrates (BASF 2006, BASF 2005). If ingested by mammals, imazapic is rapidly excreted in the urine and feces and does not bioaccumulate in animals. In addition to the acute toxicity and irritation studies conducted with Plateau® show this product to be a nontoxic and nonirritating. The potential exposure to wildlife following a labeled application of imazapic would not be expected to have any adverse effects. Imazapic is nontoxic to fish and aquatic vertebrates with a 96 hour LD50 value greater than 100 mg/L (comparable to the toxicity of caffeine).

Imazapic has limited mobility in soil and soil binding is a complex function of soil pH, texture and organic matter content (BASF 2006). The binding of imazapic to soil has been observed to increase with time. Imazapic has been shown to have little lateral movement in the soil. The major route of imazapic loss from the soil is through microbial degradation. From a total of nine soil dissipation studies conducted with imazapic, no residues were found below the 18-24 inch soil layer. After an application of imazapic, there is little potential for movement off the treated area. Imazapic is not volatile and binds moderately to most soil types once applied. Physical movement of the treated soil would be most common way for significant quantities of imazapic to move outside the project area.

As described above, Plateau® can work as a pre-emergent herbicide prohibiting the germination of cheatgrass seeds. Plateau® can also work as a post-emergent herbicide killing cheatgrass after it has germinated. If Plateau® is used as a post-emergent (applied after the cheatgrass has begun to grow) an additive must be included in the herbicide mixture to ensure that the herbicide penetrates the target plants. Methylated seed oils and Liberate® are compounds formulated to improve the penetrating ability of the herbicide and to reduce potential herbicide drift (Loveland 2004a-d). These compounds are made from vegetable oils, fatty acids, and alcohol ethoxylate. Alcohol ethoxylates can be found in domestic detergents and personal care products such as shampoo.

Prior to treatment, no-spray zones would be designated that would prevent chemical application within 100 feet on either side of streams and riparian wetlands. Additionally, no-spray control plots would be established as part of the effectiveness monitoring protocol. Exact size and configuration of the control plots have not yet been determined, but would be scientifically valid with consideration of vegetation type, soil, burn severity, slope, and other environmental factors. The locations of these no-spray zones, both the stream corridors as well as the control plots, would be loaded into the helicopters computer system and the pilot would be able to navigate to avoid those areas during application. In addition, there are obvious visible changes in topography and vegetation along stream corridors that the applicator can use as a guide.

During treatment, all aspects of the operation would be managed in compliance with all State laws and the chemical label requirements, including as worker and environmental safety precautions for chemical storage, mixing, and loading. The actual application rate would be measured and calibrated as needed to assure that the appropriate amount of chemical is applied per unit area of ground. The NPS would provide an Implementation Coordinator to oversee all aspects of the operation.

During treatment, the project area would be closed to all users. This would include periodic closures of the Kolob Terrace Road and overlooks for short periods of time when the helicopter is operating in that corridor. Closures would be announced through normal channels, including press releases to local media outlets and bulletin boards in the park. Additionally, roadside signs would be posted along Kolob Terrace Road and in Virgin to announce the closures. Within the project area no permits for any backcountry use, including the Subway Trail, would be issued during the treatment period. Once the chemical is dried and the helicopter has left the area, the project area would be re-opened to all users.

Natural revegetation of native species not sensitive to imazapic, which includes the vast majority of species occurring in the park, would be allowed to occur. Locally collected native seed would be applied to identified project areas in the spring of 2009. A nucleus of seed has been collected, and this would be increased in an agricultural setting for two seasons, at which time it is anticipated that the resulting quantity of seed would be sufficient for the treatment area.

- Figure 2 – Proposed Action

- Mitigation Measures

Mitigation is defined in the Code of Federal Regulations (40 CFR 1508.20) as:

- Avoiding the impact altogether by not taking a certain action or parts of an action.

- Minimizing impacts by limiting the degree or magnitude of the action and its implementation.

- Rectifying the impact by repairing, rehabilitating, or restoring the affected environment.

- Reducing or eliminating the impact over time by preservation and maintenance operations during the life of the action.

- Compensating for the impact by replacing or providing substitute resources or environments.

Mitigation Measures for Alternative A – No Action Alternative

Vegetation

- Spot treat as feasible to reduce cheatgrass in very small targeted areas using ground based herbicide application methods.

- Protect native plant species to the extent feasible during fire suppression.

Threatened, Endangered, and Sensitive Plant Species

- Spot treat as feasible to reduce cheatgrass in very small targeted areas near rare plant populations using ground based herbicide application methods or mechanical controls.

- Protect known rare plant populations to the extent feasible during fire suppression.

Wildlife

- Protect wildlife habitat to the extent feasible during fire suppression.

Threatened, Endangered, and Sensitive Animal Species

- Protect rare animals and their habitat to the extent feasible during fire suppression.

Natural Soundscapes

- Use non-motorized fire suppression resources and tools to the extent feasible during fire suppression.

- Continue ambient sound monitoring in and near the project area.

Wilderness

- Use minimum impact suppression tactics as feasible during future fire events.

Public Health and Safety

- Enact temporary public use closures as needed to protect people from fires, fire suppression activities, and smoke during fire events.

Visitor Use and Experience

- Provide educational information to help the visiting public understand the grass-fire cycle and the need for public use restrictions during fire events.

- Enact campfire and smoking restrictions during periods of high fire danger to reduce possible ignition sources.

Mitigation Measures for Alternative B – Proposed Action/Preferred Alternative

General

- A pre-project meeting and orientation would be conducted with the herbicide applicator prior to beginning field application including:

  - An aerial reconnaissance of area with pilot/applicator to ensure that s/he is familiar with topography and vegetation indicators of no-spray zones.

  - Test application with observers to determine the extent of drift. This information will be used to modify buffers, or change application parameters (such as droplet size, or air speed) as needed to protect water resources.

  - Determine application patterns (grid vs. parallel to slopes and streams) best suited to avoiding no-spray zones.

  - An orientation to hazards to aircraft in the area.

Vegetation

- Use a smart herbicide with a well documented effectiveness record to maximize control of cheatgrass while minimizing non-target impacts to native species.

- Use an application rate of 8 ounces per acre to maximize control of cheatgrass while minimizing non-target impacts to native species.

- Use a fall application rather than a spring application to maximize control of cheatgrass while minimizing non-target impacts to native species.

- Protect riparian plants from herbicide injury by designation of no-spray zones.

- Establish control plots to monitor the effectiveness of the aerial herbicide treatment, and take follow-up action as appropriate based on lessons learned. Share findings with others.

- Conduct spot-treatments using ground-based herbicide application methods around roadsides and other areas to improve effectiveness of the aerial treatment.

- Complete connected actions to improve opportunities for native plant restoration, including seeding and outplanting of native species in the treatment area.

Threatened, Endangered, and Sensitive Plant Species

- Monitor response of rare plant species in the project area.

Wildlife

- Protect riparian habitats from herbicide injury by designation of no-spray zones

- Monitor the effectiveness of the aerial herbicide treatment using control plots, and take follow-up action as appropriate based on lessons learned. Share findings with others.

Threatened, Endangered, and Sensitive Animal Species

- Continue monitoring of Mexican spotted owls and peregrine falcons in the project area.

- Provide a treatment summary to the Utah Division of Wildlife Resources so that they can consider this treatment in their monitoring program for the flannelmouth sucker and the Virgin spinedace.

Natural Soundscapes

- Continue ambient sound monitoring in and near the project area.

Public Health and Safety

- Enact temporary public use closures during herbicide application treatment to protect people from overhead hazards and herbicide exposure.

- Restrict public access to the Coalpits helispot for the 2 weeks needed to complete the herbicide application.

- Follow standard aviation safety practices, such as flight following, air to ground communication, and identification of aviation hazards.

- Follow all herbicide label requirements and material safety data sheet recommendations for safe storage, handling, and application.

Visitor Use and Experience

- Provide educational information to help the visiting public understand why the temporary public use closure is in effect during herbicide application. Provide information on alternative recreational opportunities in the park.

- Road closure would be minimized as much as possible and would last up to 30 minutes at a time.

- Plan the application period to be after October 24, 2006 to avoid impacts to hunters on nearby non-park lands, and after the visitation to the project area declines with the onset of cold weather.

## Environmentally Preferred Alternative

As stated in Section 2.7.D of DO-12 and Handbook (USDI NPS 2001a), the environmentally preferred alternative is the alternative that will promote the national environmental policy expressed in the NEPA (Sec. 101(b)). This includes alternatives that:

- Fulfill the responsibilities of each generation as trustee of the environment for succeeding generations.

- Ensure for all Americans safe, healthful, productive, and esthetically and culturally pleasing surroundings.

- Attain the widest range of beneficial uses of the environment without degradation, risk of health or safety, or other undesirable and unintended consequences.

- Preserve important historic, cultural, and natural aspects of our national heritage and maintain, wherever possible, an environment that supports diversity and variety of individual choice.

- Achieve a balance between population and resource use that will permit high standards of living and a wide sharing of life’s amenities.

- Enhance the quality of renewable resources and approach the maximum attainable recycling of depletable resources.

Simply put, “this means the alternative that causes the least damage to the biological and physical environment; it also means the alternative which best protects, preserves, and enhances historic, cultural, and natural resources” (Question 6a in CEQ 1981). In the NPS, the No Action Alternative may also be considered in identifying the environmentally preferred alternative.

Alternative A represents no action to interrupt the grass-fire cycle. As a result, cheatgrass would continue to invade and fires would become more frequent. Under this scenario, many native plant communities would be greatly reduced and habitat value would be degraded. This type of event would result in adverse affects to many of the park’s resources and values.

The Proposed Action/Preferred Alternative, Alternative B, would use aerial application of herbicide to suppress cheatgrass germination and invasion into the burned area, thus allowing the native plants to successfully re-establish and persist in the burned area. The re-establishment of native vegetation would then restore habitat needed to support native wildlife and perpetuate natural ecosystem processes The Preferred Alternative as compared to current management/No Action Alternative would:

- Provide an environment dominated by native plant communities functioning within their natural fire regime.

- Reduce the risk to human health and safety and other undesirable consequences of frequent wildland fire.

- Improve the safety, healthfulness, and esthetics of the surroundings.

- Provide better protection of historic, cultural, and natural resources for succeeding generations.

Therefore, Alternative B, the Proposed Action/Preferred Alternative, also would be the environmentally preferred alternative.

## Alternatives Considered but Dismissed From Further Analysis

The following alternatives were identified through scoping. For the reasons stated below they will not be further analyzed in this document.

Spray only a ¼ mile buffer along park boundary, trails and roads in the project area. This alternative would treat about 4,500 acres, representing about 42 percent of the 10,615 acre burned area. Application along the west boundary would help keep fires from spreading from the park to adjacent lands and vice-versa. Application along the east boundary of the fire would serve to keep cheatgrass from invading deeper into the park and to limit its extent to the lower elevation vegetation types drawing a ‘line in the sand’ to keep most of the higher elevation vegetation communities intact. This treatment would also buffer ¼ mile on either side of all roads, trails, and routes to limit the spread of cheatgrass along these major invasion corridors. From the fire behavior perspective, this may prevent human starts from these areas from moving further into the park and provide defensible space for fighting fire. However, this alternative would still leave large islands of untreated areas into which cheatgrass would readily invade and thrive. Since the majority of the burned area, 58 percent, would not be treated and those areas that would be treated are primarily the most disturbed lands with the least amount of ecological value, the restoration of native plant communities and associated wildlife habitats would not be realized under this alternative. After a great deal of consideration, this alternative was rejected because it does not fully meet the purpose and need of the project.

Spray ¼ mile buffer along trails and roads only. This alternative is a subset of the alternative described above. Under this alternative, only the roads and trails would be sprayed and the west park boundary and eastern perimeter of the burned area would not be treated. The treated areas would total about 2,400 acres, representing about 23 percent of the 10,615 acre burned area. These treated roads and trails could serve as fuel breaks for fires that are carried by cheatgrass and would reduce, but not prevent, seed spread from travel corridors into interior areas. Furthermore, the vast majority of the burned area, 77 percent, would not be treated and those areas that would be treated would be the most disturbed lands with the least amount of ecological value, the restoration of native plant communities and associated wildlife habitats would not be realized under this alternative. This alternative was rejected because it does not meet the purpose and need of the project.

Ground-based spot application only. The 2005 ZION FMP EA allows for herbicide use by hand (backpack), ATV and truck. It also says that aerial applications of herbicides can also be used where large invasive patches occur or where other application treatments are impractical or inefficient. But, any aerial use of herbicides would require additional NEPA compliance. We analyzed an alternative for herbicide use that is covered by the current EA, but using an application rate of 2 acres per-day-per-person. With a 2 person crew it would take approximately 2-weeks to treat the roadsides, trails and routes, 29 acres. That treatment would leave 99.7 percent of the burned area open for cheatgrass to invade. For these reasons, it was determined that this alternative would not meet the purpose and need of the project and thus it was rejected as a viable alternative.

Ground-based widespread application only. Aerial application by helicopter is more accurate and precise in application rate than hand spraying. It is estimated that a helicopter can spray 50 acres per hour or about 600 acres per day (6 hours of flight time). The helicopter is recommended due to the ability to fly uneven, difficult terrain; ability to fly slower allowing for prescribed water volume, increased application and local landing and refilling ability. To treat the entire 10,280 acre project area using ground based methods, it would take a 10 person crew more than 450 days under optimal field conditions. Such a prolonged treatment period would greatly reduce the effectiveness of the treatment and would increase impacts to resources due to the continual presence of human activity in the project area. For these reasons, it was determined that this alternative would not meet the purpose and need of the project and thus it was rejected as a viable alternative.

Use of other herbicides. There are very few chemicals that are effective in cheatgrass control that do not also have a great deal of impact on native plants. As the purpose of the project is to restore native plant communities, no bare-ground herbicides or broad-spectrum herbicides were considered. One chemical, Journey®, was considered. Journey® is a mixture of imazapic (the same chemical proposed for use) and glyphosate.

Field trials with Journey® in Zion Canyon have shown that it is effective in cheatgrass control in spring application, but still not as effective as a fall Plateau® application (personal comm., Cheryl Decker). The alternative herbicide treatment would be to use Journey® in the spring, but because of the glyphosate used in Journey® to kill the post emergence cheatgrass; this could cause damage to early emerging native perennial plants. As widespread application of this chemical would potentially eliminate many desirable native plants, it would fall short of restoring native plant communities in the burned area. For these reasons it was determined that this alternative would not meet the purpose and need of the project and thus it was rejected as a viable alternative.

Seeding only – no herbicide. We considered an alternative where only seed would be spread across the burned area and no herbicide would be applied to control cheatgrass. Where cheatgrass is abundant or likely to become abundant, native plant seeds often fail to germinate or establish, and seeding alone does not necessarily decrease invasive species cover or may even reduce native perennial plant cover (Brooks 2005). Similarly, cheatgrass control is only effective when combined with treatments that establish perennial species (Harris and Goebel 1976; Klemmedson and Smith 1964; Mosley et al. 1999); or, in areas where there already is a significant component of native perennial plants present, chemicals can control cheatgrass (Mosley et al. 1999) and allow the native plants to grow. The plant composition of the burned area was carefully analyzed. As many of the interior areas of the burned area have plants that are known to recover from fire either by sprouting or germination of seed, the park favors allowing the natural re-establishment of native plant communities throughout most of the burned area coupled with some seeding along roadsides where the native seedbeds may have been depleted by long-existing and abundant cheatgrass found in these areas. Furthermore, widespread seeding is very difficult due to the unavailability of large amounts of native seeds and that seed introduced from elsewhere, even if native species, may not reflect the genetic makeup of the plants found in the local area. With the potential for seed failure due to the ability of cheatgrass to outcompete it, the infeasibility of finding adequate amounts of seed, and the potential for contamination of local genetics of native plant species, this alternative would most likely be unsuccessful in restoring native plant communities at a landscape level. For these reasons it was determined that this alternative would not meet the purpose and need of the project and thus is was rejected as a viable alternative.

| Table 1: Comparison of the Achievement of Purpose and Need by Alternative |  |  |
| --- | --- | --- |
| Purpose and Need | Alternative A: No Action | Alternative B: Proposed Action/Preferred Alternative |
| Contribute to the restoration of natural fire regimes and ecosystem processes. | Does not meet the purpose and need, because without treatment cheatgrass will become dominant which will increase the frequency, duration, and severity of fire. This will decrease native plant cover and wildlife habitat and interrupts natural process. | Meets the purpose and need by actively reducing cheatgrass cover which interrupts the grass-fire cycle which allows native plant communities to regenerate and persist. These native plant communities provide habitat for wildlife and contribute to functioning ecosystems. |
| Interrupt the grass-fire cycle caused by cheatgrass. | Does not meet the purpose and need, because without treatment cheatgrass will become dominant which will increase the frequency, duration, and severity of fire – perpetuating the grass-fire cycle. | Meets the purpose and need by actively reducing cheatgrass cover which interrupts the grass-fire cycle which allows native plant communities to regenerate and persist. In these healthy plant communities natural fire regimes can be maintained. |

| Table 2: Comparative Summary of Impacts |  |  |
| --- | --- | --- |
| Impact Topic | Alternative A – No Action | Alternative B – Proposed Action/Preferred Alternative |
| Vegetation | Implementation of the no action alternative would result in no short-term impacts to vegetation, but would result in long-term, moderate negative impacts to vegetation due to perpetuation of the grass-fire cycle which ultimately would lead to widespread loss of native shrubland and woodland communities and extirpation of many native plant species. | The proposed action would result in short-term, minor negative impacts to some plants due to herbicide exposure and long-term, moderate positive impacts due to perpetuation of native shrublands and woodlands. |
| Threatened, Endangered, and Sensitive Plant Species | Implementation of the no action alternative would result in no short-term impacts to threatened, endangered, and sensitive plant species, but would result in long-term, moderate negative impacts to these plants due to perpetuation of the grass-fire cycle. The landscape level conversion of native woodlands and shrublands to invasive grasslands would ultimately lead to extirpation of threatened, endangered and sensitive plant species within the project area and could contribute to their losses in adjacent lands. Effects Determination. Implementation of the no action alternative with increased fire effects and fire suppression effects may affect, not likely to adversely affect Shivwits milkvetch populations or habitats. | The proposed action would result in short-term, minor to moderate negative impacts to some state listed plants due to herbicide exposure and long-term, moderate positive impacts due to perpetuation of native plant communities and suitable habitat for listed plant species. Effects Determination. Implementation of the proposed action may affect, not likely adversely affect Shivwits milkvetch populations or habitats. |
| Wildlife | Implementation of the no action alternative would result in no short-term impacts to wildlife, but would result in long-term, moderate negative impacts to wildlife habitat and increase the potential for mortality of animals due to perpetuation of the grass-fire cycle. The landscape level conversion of native woodlands and shrublands to invasive grasslands would alter wildlife habitat and reduce the species diversity and population health of many wildlife species. | The proposed action would result in short-term, minor negative impacts to some animals due to herbicide exposure and response to the helicopter. There would be long-term, moderate positive impacts due to perpetuation of native shrublands and woodlands which provide wildlife habitat. |
| Threatened, Endangered, and Sensitive Animal Species | Implementation of the no action alternative would result in no short-term impacts to threatened, endangered, and sensitive animal species, but would result in long-term, moderate negative impacts to these animals due to perpetuation of the grass-fire cycle. The landscape level conversion of native woodlands and shrublands to invasive grasslands would potentially reduce the suitability of the habitat that currently supports Mexican spotted owl and peregrine falcon. It could also contribute to the loss of these bird species in adjacent lands and could have negative impacts to the habitat of the fish species downstream of the project area. Effects Determination. Implementation of the no action alternative would have no effect on Mexican spotted owls, but could adversely affect critical habitat for this species. | The proposed action would result in short-term, negligible negative impacts to some birds due to the noise generated by the helicopter and there would be no short-term impacts to fish species. There would be minor long-term positive impacts to threatened, endangered, and sensitive animal species due to perpetuation of suitable habitat. Effects Determination. Implementation of the proposed action would have no effect on Mexican spotted owls because the proposed action would occur outside the critical breeding season and would occur outside of any protected activity centers. The implementation of the proposed action may affect, not likely to adversely affect critical habitat for this species. |
| Soils | Implementation of the no action alternative would result in no short-term impacts to soils, but would result in long-term, minor negative impacts to soils due to increased potential for post-fire erosion and subsequent localized loss of soil productivity and fertility. | The proposed action would result in short-term, minor negative impacts to soil due to decreased soil productivity as a result of herbicide and long-term, moderate positive impacts due to perpetuation of natural fire regimes that decrease the frequency and magnitude of post-fire erosion events. |
| Water Resources | Implementation of the no action alternative would result in no short-term impacts to water resources, but would result in long-term, moderate negative impacts to water resources due to increased potential for post-fire erosion and subsequent alteration of sediment yield, water quality, and stream morphology. | The proposed action would result in short-term, minor negative impacts to water resources and long-term, moderate positive impacts to water resources due to perpetuation of natural fire regimes that decrease the frequency and magnitude of post-fire watershed responses. |
| Natural Soundscapes | Implementation of the no action alternative would result in no short-term impacts to soundscapes, but would result in long-term, negligible negative impacts to soundscapes due to noise from increased fire suppression activity. | The proposed action would result in short-term, negligible negative impacts to soundscapes in frontcountry settings during daylight hours and short-term, moderate negative impacts to soundscapes in primitive or pristine settings during daylight hours as a result of helicopter noise. There would be long-term minor positive impacts to soundscapes due to restoration of natural fire regimes and reduced frequency of noise generated by fire suppression activities. |
| Wilderness | Implementation of the no action alternative would result in no short-term impacts to wilderness, but would result in long-term, moderate negative impacts to wilderness due to loss of naturalness resulting from increased fire suppression activity as well as the loss of native plant communities and wildlife habitat. | The proposed action would result in short-term, moderate negative impacts to wilderness due to the introduction of herbicide and the intrusion of the helicopter in wilderness. There would be long-term, moderate positive impacts to wilderness due to perpetuation of native plant communities, wildlife habitats, and natural fire regimes. |
| Public Health & Safety | Implementation of the no action alternative would result in no short-term impacts to public health and safety, but would result in long-term, minor negative impacts to public health and safety due to increased exposure to fire and fire suppression hazards resulting from increased fire frequency, fire size, and fire intensity. | All short-term, negative impacts to public health and safety can be mitigated. There would be long-term, minor positive impacts to public health and safety due to restoration of the natural fire regime and reduced exposure of the public, park neighbors, and firefighters to hazards associated with fire and fire suppression activities. |
| Visitor Use & Experience | Implementation of the no action alternative would result in no short-term impacts to visitor use and experience, but would result in long-term, minor negative impacts to visitor use and experience due to public use closures associated with increased fires and fire suppression activities. | There would be short-term, minor negative impacts to visitor use and experience due to public use closures and long-term, minor positive impacts to visitor experience due to perpetuation of native plant communities and wildlife that visitors can enjoy. |

# AFFECTED ENVIRONMENT & ENVIRONMENTAL CONSEQUENCES

## Methodology for Assessing Impacts

In order to analyze the environmental consequences of the alternatives proposed in this document, three factors must be examined for each resource: type of impact, duration of impact, and intensity of impact. After the environmental consequences of the alternatives are examined by separate topic, the impact of implementing the alternative is considered along with the impacts of other relevant actions in the area.

The type of impact describes a relative measure of beneficial or adverse effects on biological or physical systems, cultural resources, or the social environment. Because impacts could have short-term, adverse impacts while having long-term, beneficial impacts, it is important to look at the duration of the effect of an impact.

However, examining only the type and duration of an impact is not enough because an impact could cover a large area or a large portion of a population, or could be highly noticeable or even irreversible. Impacts can vary in intensity, from small and imperceptible to large and substantial. Measures of intensity consider whether an impact would be negligible, minor, moderate, or major. These measures are used to describe both beneficial and adverse impacts.

## Cumulative Impacts

Cumulative impacts are also considered in this analysis. A cumulative impact is described in the CEQ regulations (1508.7) as “the impact on the environment which results from the incremental impact of the action when added to other past, present, and reasonably foreseeable future actions regardless of what agency (federal or non-federal) or person undertakes such other actions.”  Cumulative impacts can result from individually minor but collectively major actions taking place over a period of time.

Cumulative impacts of each alternative were addressed by considering the effects of the alternative, combined with the effects of the following past, present, and reasonably foreseeable future actions that were identified in and around the project area. There are several actions that were identified in the Kolob Fire BAR Plan that are considered cumulative actions to the proposed action:

Native Seed Increase. Zion initiated a seed increase program for local grass species through an interagency agreement with the Natural Resource Conservation Service (NRCS). The NRCS Plant Material Centers are currently or will be growing out native grass species propagated from local seed sources. The seed increase focuses on bottlebrush squirreltail (Elymus elymoides) and sand dropseed (Sporobolus cryptandrus); two native species that do well in broadcast seeding and compete with cheatgrass. The seed will later be harvested and broadcast on 924 acres in the direct of vicinity of Crater Hill in June of 2009.

Aerial Seeding of Roadsides. The purpose of this treatment is to re-establish native plants in areas that have in the last 2-3 years been heavily invaded by cheatgrass. Without such treatment in association with herbicide application, the post-fire scenario is likely to be one where cheatgrass and other undesirable exotics would out-compete at the expense of natives. The area around the Kolob Terrace and Smith Mesa Road had understories that were totally dominated by cheatgrass before the Kolob Fire. Examining the area just up the road, near the Subway Trailhead there is little visible evidence of other herbaceous native plants. There is concern that this area does not have enough residual native seed to compete with cheatgrass after the effect of the herbicide treatment fades. Therefore, a broadcast seeding on 481 acres covering ¼ mile buffers around these roads is planned for implementation in conjunction with the herbicide treatment. This action was analyzed in the ZION FMP completed in 2005.

Exotic Plant Monitoring and Control. The purpose of this treatment is to conduct early detection and eradication of targeted invasive plants along burned road sections, known areas with noxious and invasive plants, retardant drops and other areas of special concern. Control measures will be applied in conjunction with monitoring and assessment in areas where invasive plants are found and are likely to spread into burned or disturbed areas. Detection of new occurrences of noxious and invasive plant species in areas burned or disturbed by suppression activity, and monitoring of known populations will be conducted to assess if further actions are required to control the spread of these plants into burned and disturbed areas. Any control will be conducted using Integrated Pest Management practices.

Sensitive Plant Monitoring and Spot Control. The purpose of this project is to determine if any of the known sensitive plant populations are still extant within the burned area; and, if so describe their abundance, condition, and any competitive threats that may exist from non-native plants that may be addressed through spot applications of herbicide. Three known populations of sensitive plants were within the burn perimeter: Zion daisy (Erigeron sionis), Religious daisy (Erigeron religiosus) and Zion penstemon (Penstemon humilis var. obtusifolius).

Boundary Fence Replacement. Replace barbed wire boundary fence along the west park boundary in the Kolob Fire burned area. The Kolob Fire incident resulted in approximately 4 miles of park boundary fence line that sustained damage. This included 1,175 burned posts, 54 damaged stays and 4 miles of damaged barb wire. The task is to replace burned wooden posts with metal T-posts, the damaged stays, and wire.

Completely separate from the projects identified in the BAR Plan, the Tunnel East Construction Project on Zion-Mt. Carmel Highway (EA signed April 2006) is planned in a different part of the park during fall 2006, the same time period as the proposed herbicide application. The Tunnel East Construction project is expected to cause delays due to intermittent lane closures which would impact park visitors traveling on the east side of the park. In association with the Tunnel East Construction project, Pine Creek canyoneering route could be closed for up to 1-day plus the entrance into the canyon will be rerouted and the Canyon Overlook Trail would be closed for the duration of the project. Because the two projects would occur in different locations that are very dissimilar, the Tunnel East Construction Project would not impact any of the same resources as the proposed herbicide application, but it could impact the same people.

## Impairment Analysis Method

The NPS Management Policies (USDI NPS 2001b) requires analysis of potential effects to determine whether or not actions would impair park resources or values.

The fundamental purpose of the National Park System, established by the Organic Act and reaffirmed by the General Authorities Act, as amended, begins with a mandate to conserve park resources and values. NPS managers must always seek ways to avoid, or to minimize to the greatest degree practicable, actions that would adversely affect park resources and values.

These laws give the NPS the management discretion to allow impacts to park resources and values when necessary and appropriate to fulfill the purposes of the park, as long as the impact does not constitute impairment of the affected resources and values. Although Congress has given the NPS the management discretion to allow certain impacts within parks, that discretion is limited by the statutory requirements that the NPS must leave park resources and values unimpaired, unless a particular law directly and specifically provides otherwise.

The prohibited impairment is an impact that, in the professional judgment of the responsible NPS manager, would harm the integrity of park resources or values, including the opportunities that otherwise would be present for the enjoyment of those resources and values. An impact to any park resource or value may constitute impairment. Impairment may result from NPS management activities, visitor activities, or activities undertaken by concessionaires, contractors, and others operating in the park. An impact would be more likely to constitute impairment to the extent that it has a major or severe adverse effect upon a resource or value whose conservation is:

- necessary to fulfill specific purposes identified in the established legislation or proclamation of the park;

- key to the natural or cultural integrity of the park or to opportunities for enjoyment of the park; or

- identified as a goal in the park’s general management plan or other relevant NPS planning documents.

A determination on impairment is included in the impact analysis section for all impact topics relating to park resources and values.

## Vegetation

The vegetation of ZION and the surrounding area was mapped through a project with the U.S. Bureau of Reclamation, The Nature Conservancy (Nature Serve), and the NPS (Cogan et al. 2004). This information was further refined and analyzed to develop the park’s FMP (USDI NPS 2005) to highlight those vegetation attributes most relevant to fire. After the Kolob Fire, the burned area was intensively investigated by Kara Paintner, a plant and fire ecologist with the NPS, and members of the park staff, primarily Vegetation Manager Cheryl Decker and Fire Ecologist Kelly Fuhrmann. The results of their investigation were summarized in the Kolob Fire BAR Plan (USDI NPS 2006). The site specific information presented here comes primarily from these sources.

Affected Environment

An elevation range from 3,666 feet to 8,726 feet, coupled with topographic complexity and unique geologic substrates creates a diverse flora in ZION. The park includes four life zones: Lower Sonoran, Upper Sonoran, Transition, and Canadian. These life zones consist of low elevation desert shrubland communities with Mojave Desert elements, mid-elevation shrublands and pinyon-juniper woodlands typical of the Colorado Plateau and Great Basin, and montane forests and oak brush shrublands at the park’s highest elevations. Within the park, the Kolob Fire occurred at 3,666 to 6,900 feet in elevation and burned in the Lower Sonoran, Upper Sonoran, and Transition Zones.

Vegetation in the lower to mid-elevations of the park, including the Kolob Fire burned area, is generally sparse and low in stature due to lack of moisture. Semi-arid desert species are common, such as blackbrush (Coleogyne ramosissima), four-wing saltbush (Atriplex canescens) and mesquite (Prosopis glandulosa). Sandy slopes upland from waterways support mostly pinyon pines (Pinus edulis, P. monophylla), juniper (Juniperus osteosperma), sand and big sagebrush (Artemesia filifolia and A. tridentata), and rubber rabbitbrush (Chrysothamnus nauseosa). Interspersed within these species are pockets of grasses, mainly sand dropseed (Sporobolus cryptandrus), mutton grass (Poa fendleriana), and the invading non-native red brome (Bromus rubens) and cheatgrass (Bromus tectorum).

The vegetation types across the fire have a wide variety of fire adaptations and fire return intervals (Figure 3 and Table 3). Over three quarters of the vegetation burned in the fire was in the pinyon-juniper community. Historically this vegetation type has a highly variable fire return interval. Earlier work in the park estimated that this type has not burned in at least 160 years (West and Loope 1977). Much of the pinyon-juniper in the fire has an herbaceous understory. These herbaceous natives should naturally re-establish after fire, if they aren’t out competed by cheatgrass.

There is one vegetation association within the pinyon-juniper vegetation type of particular concern for post-fire recovery, the Utah juniper / big sagebrush woodland. This juniper and sagebrush woodland mainly occurs in the Crater Hill area with fine sandy to cindery soils. The herbaceous layer is absent or very sparse (Cogan et al. 2004). Fires in this association are thought to be infrequent because smaller Utah juniper and big sagebrush are easily killed by fire and do not re-sprout (Barney and Frischknecht 1974, Everett 1987). Big sagebrush will re-establish relatively quickly (about 10-20 years) if a seed source is nearby (Barney and Frischknecht 1974, Bunting 1987). Additionally, Utah Juniper is relatively slow to recover following fire, and sagebrush may dominate the sites for decades (Jameson et al. 1962). If fire-return intervals are more frequent than 10 years, as expected if cheatgrass becomes dominant on these sites, then big sagebrush has difficulty recovering (Bunting 1987, Everett 1987).

| Table 3: Plant Communities from the Zion FMP, Acreage and Percentage within the Kolob Fire |  |  |  |
| --- | --- | --- | --- |
| Plant Communities | Acres | Area (percent) | Fire Return Interval (years) |
| Juniper-Pinyon | 7,882 | 75 | 50-150 |
| Shrublands | 723 | 7 | 40-60 |
| Bare Soil/Stone Formations | 530 | 5 | NA - Unburnable |
| Desert Shrublands | 440 | 4 | 100+ |
| Mountain Shrub | 394 | 4 | 20-40 |
| Grass-Herbaceous Lands | 231 | 2 | 25-50 |
| Wetland/Riparian | 191 | 2 | 100+ |
| Ponderosa Pine | 67 | 1 | 5-25 |
| Exotic Grasses | 26 | 0 | 1-10 |
| Slickrock | 17 | 0 | NA - Unburnable |
| Exotic Riparian | 3 | 0 | Unknown |
| Douglas Fir | 2 | 0 | 50-80 |
| Return Intervals from Zouhar (2003) and Campbell and others (2003) |  |  |  |

Figure 3 – Pre-Fire Vegetation

Most of the major trees and shrubs in the juniper-big sagebrush woodland and through the rest of the pinyon-juniper vegetation type regenerate from seed. Both pinyon and juniper have large seeds with short dispersal distances, except when eaten by birds. Pinyon pine is fairly susceptible to both crown scorch and cambial damage. Junipers can sustain much more crown scorch and survive. All of the woody sagebrushes in the park are not sprouting species and need a local seed source to re-occupy the site. A few of the common shrubs and their ability to re-sprout from the crown are listed in Table 4. This ability will be critical if cheatgrass invades and the fire frequency increases. Evidence from the northern Great Basin has shown that non-sprouting shrub species are the most easily displaced from the system by shortened fire return intervals, although with very repeated frequent fire sometimes even the sprouting species are taken out of the system (Barney 1974; Bunting 1987).

| Table 4: Common shrubs & Their Sprouting Capability in the Lower Elevation Communities |  |  |
| --- | --- | --- |
| Common Name | Scientific Name | Re-sprout? |
| Big Sagebrush | Artemisia tridentata | No |
| Little leaf manzanita | Acrotstaphylos patula | Yes |
| Sand Sagebrush | Artemisia filifolia | No |
| Rabbitbrush | Chrysothamnus spp. | Yes |
| Blackbrush | Coleogyne ramosissima | No |
| Broom snakeweed | Gutierrezia sarothrae | Yes |
| Four-wing saltbush | Atriplex canescens | No |
| Mormon tea | Ephedra nevadensis | No |
| Gamble oak | Quercus gambellii | Yes |
| Shrub live oak | Quercus turbinella | No |
| Utah serviceberry | Amelanchier utahensis | Yes |

The fire also burned through and around riparian areas on 193 acres. Canyons in the park are an important desert oasis, with streams, seeps, wetlands, and hanging gardens. Perennial and ephemeral streams converge into the East and North Forks of the Virgin River, hosting riparian tree species such as the Fremont cottonwood (Populus fremontii), Goodings willow (Salix goodingii), boxelder (Acer negundo), and velvet ash (Fraxinus velutina). Seepwillow (Baccharis emoryi) and coyote willow (Salix exigua) are common riparian shrubs. While wetland/riparian areas represent a small percentage of the burned area, these communities provide many important habitat functions for a number of wildlife species. In the wetland and riparian category, a small percentage of the area (2 percent) shows high burn severity (Table 5). The majority of this community is either unburned (26 percent) or low burn severity (49 percent). There could be some concern for the cottonwoods in these systems due to crown scorch. A substantial threat to native plant communities within floodplains and increasingly in the uplands is the invasion and dominance of the exotic annual grasses, collectively referred to in this document as cheatgrass, that include downy brome (Bromus tectorum), red brome (Bromus rubens) and ripgut brome (Bromus diandrus) that would serve to carry fire into these riparian corridors.

| Table 5: Acres and Percentages by Plant Community and Burn Severity Class |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Plant Community | Burn Severity Class |  |  |  |  |  |  |  |  |  |  |  |
|  | Unburned |  |  | Low |  |  | Moderate |  |  | High |  |  |
|  | Acres | % by plant community | % by burn severity | Acres | % by plant community | % by burn severity | Acres | % by plant community | % by burn severity | Acres | % by plant community | % by burn severity |
| Juniper P-J | 816 | 10 | 66 | 2428 | 31 | 69 | 4561 | 58 | 81 | 76 | 1 | 52 |
| Shrublands | 42 | 6 | 3 | 273 | 38 | 8 | 393 | 54 | 7 | 14 | 2 | 10 |
| Bare Soil - Stone Formations | 186 | 35 | 0 | 212 | 40 | 6 | 124 | 23 | 2 | 7 | 1 | 5 |
| Desert Shrublands | 33 | 8 | 3 | 259 | 59 | 7 | 148 | 34 | 3 | 0 | 0 | 0 |
| Mountain Shrub | 85 | 21 | 7 | 68 | 17 | 2 | 196 | 50 | 3 | 46 | 12 | 31 |
| Grass - Herbaceous | 5 | 2 | 0 | 125 | 54 | 4 | 101 | 44 | 2 | 0 | 0 | 0 |
| Wetland - Riparian | 26 | 14 | 2 | 93 | 49 | 3 | 69 | 36 | 1 | 3 | 2 | 2 |
| Ponderosa Pine | 20 | 29 | 2 | 22 | 33 | 1 | 24 | 37 | 0 | 1 | 1 | 1 |
| Exotic Grass | 7 | 29 | 1 | 11 | 44 | 0 | 7 | 27 | 0 | 0 | 0 | 0 |
| Slickrock | 10 | 62 | 1 | 6 | 34 | 0 | 1 | 4 | 0 | 0 | 0 | 0 |
| Exotic Riparian | 1 | 31 | 0 | 2 | 64 | 0 | 0 | 5 | 0 | 0 | 0 | 0 |
| Douglas Fir | 1 | 35 | 0 | 1 | 44 | 0 | 0 | 20 | 0 | 0 | 1 | 0 |
| TOTAL | 1234 | 12 |  | 3588 | 33 |  | 5625 | 54 |  | 147 | 1 |  |

Within the burned area over 70 percent of the plots visited by the vegetation mapping effort in 2003-04 showed at least trace amounts cheatgrass (Cogan et al. 2004). Field visits to the areas along the Kolob Terrace Road show the understory almost totally dominated by cheatgrass with very little native component. Before the fire, the area around the Kolob Terrace and Smith Mesa Road had understory vegetation that were totally dominated by cheatgrass. Examining the area just up the road, near the Subway Trailhead, there is little visible evidence of other herbaceous native plants. Areas in the Crater Hill area had cheatgrass as well. In the heavier clays soils in the area galleta grass (Pleuraphis jamesii) was already greening up from a recent rain. To the east and north of the burned area, cheatgrass occurrence was much less at about 40 percent. Additional survey of the West Rim Trail showed only trace amounts at high elevations. Within the burned area it can be expected that cheatgrass populations will explode post fire.

Impact Threshold Definitions

| Negligible | No native vegetation would be affected, or some individual native plants could be affected as a result of the alternative, but there would be no effect on native species populations. The effects would be on a small scale. Non-native vegetation would not be affected. |
| --- | --- |
| Minor | The alternative would temporarily affect some individual native plants and would also affect a relatively minor portion of that species’ population. Mitigation to offset adverse effects could be required and would be effective. Some non-native plants would be affected. |
| Moderate | The alternative would affect some individual native plants and would also affect a sizeable segment of the species’ population over a relatively large area. Mitigation to offset adverse effects could be extensive, but would likely be successful. There would be a sizable affect on non-native plants. |
| Major | The alternative would have a considerable affect on native plant populations and would affect a relatively large area in and outside of the park. Mitigation measures to offset the adverse effects would be required and would be extensive; success of the mitigation measures would not be guaranteed. |
| Duration | Short-term – recovers in less than three years |
|  | Long-term – requires more than three years to recover |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary |

Effects of Alternative A – No Action Alternative

Under the no action alternative, no aerial herbicide application would occur. There would be no non-target effects of herbicide on the native or non-native plant species within the project area. Plants would re-sprout or germinate, grow, reproduce, and die in response to other biological and environmental factors. There would be no post-emergent or pre-emergent effect on plants within the project area. As there are no other known methods to effectively interrupt the grass-fire cycle in a wildland landscape, it is anticipated that cheatgrass would quickly re-establish.

Short-term uncontrolled cheatgrass invasion in the burned area would most likely result in localized displacement of native species. Such displacement occurs by two primary methods. First, the cheatgrass germinates earlier than native species and thus has a chance to occupy growing space and use soil water and nutrient resources before the native species have an opportunity to grow, thus depriving native species of the materials needed for life which reduces germination rates and recruitment success of native species (Brooks et al. 2004). Secondly, cheatgrass alters the growing environment by creating a dense thatch layer composed of the dead plant materials from previous growing seasons that covers the soil surface. This thatch layer serves to deter seed set, germination, and recruitment of some native plants, particularly annual species and biological soil crust communities. The first winter following the fire, the existing cheatgrass seedbanks would germinate and grow vigorously in response to the increased soil water and nutrient conditions that exist as a result of the fire and the thatch would begin forming at the end of the first growing season and become thicker in subsequent years. As the cheatgrass seedbanks are most abundant along the western boundary of the park and the Kolob Terrace Road, as well as the trailheads and hiking trails that originate from these areas, it is expected that these areas would see a dense and fairly continuous cover of cheatgrass within one or two growing seasons. In the interior sections of the project area, cheatgrass would also germinate and grow, but would likely be more sparse and patchy the first few years after the fire due to the smaller amount of cheatgrass seed available and because the native plant populations are likely to be healthier and therefore more competitive initially. Each successive growing season would increase the continuity between interior cheatgrass patches.

Long-term, the project area is considered at high risk for cheatgrass invasion (Pellent 2003). Uncontrolled cheatgrass invasion is expected to alter natural ecosystem processes (Brooks et al. 2004), particularly fire regimes. Over the next several years, it is expected that cheatgrass would become increasingly more dense and continuous throughout the burned area. Each successive growing season increases the cheatgrass seedbank in the soil and leaves an increasingly deep layer of thatch on the soil surface (Brooks et al. 2004). Within 5-10 years, it is expected that cheatgrass would form a continuous fine fuel layer across most of the burned area. As cheatgrass invasion is known to shorten fire return intervals to 3-5 years (BASF 2003a), and 240 acres within the project area burned in 2005 and again in the Kolob Fire of 2006, the project area would likely experience at least one other large-scale fire event during this 5-10 year time period. This grass-fire cycle would likely continue and fires would continue to increase in frequency, size, and intensity with each fire. With each successive fire event, many native perennial plant species would be less likely to recover and re-establish either through sprouting or seed germination. Most notably, sagebrush species, blackbrush, and saltbush do not easily regenerate after fire (Table 4) and riparian species are sometimes slow to regenerate, so these species can be expected to be extirpated from the project area if fires become more frequent. Under a frequent fire scenario, the seedbanks of many native annual and perennial species would be reduced with each fire event as seeds succumb to heat mortality, diminished viability over time resulting from years of inability to germinate due to cheatgrass competition, or are removed from suitable growing locations in soil that erodes as a result of normal post-fire watershed responses. If the time between fires does not allow for sufficient seed production, the seedbank continues to deplete until there are virtually no viable seeds left to restore native plant communities. Species that are fire intolerant would be the first to be extirpated from the area and the species composition of most plant communities would be altered. As discussed in the soils section of this document, frequent large fires would likely preclude the recolonization or succession of biological soil crusts, which in turn would impact the ability of some vascular plants to re-establish on burned areas. Eventually, native woodlands and shrublands would be converted into invasive grasslands.

Cumulative Impacts. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. The resulting increase in fires would likely result in wide-spread losses of native plant populations and degradation of native plant communities. Overall, impacts of no action added to the impacts of other actions affecting vegetation, would result in long-term, moderate cumulative negative impacts to native plant communities and populations.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to vegetation, but would result in long-term, moderate negative impacts to vegetation due to perpetuation of the grass-fire cycle which ultimately would lead to widespread loss of native shrubland and woodland communities and extirpation of many native plant species.

Impairment. Because there would be no major, adverse impacts to vegetation whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s vegetation resources from the implementation of Alternative A.

Effects of Alternative B – Proposed Action/Preferred Alternative

Imazapic-based herbicide would be applied during the fall season by helicopter to the entire project area, except for the designated no-spray zones that include streams, riparian corridors, and wetlands as well as treatment effectiveness monitoring control plots. Plateau®, which includes imazapic as the active ingredient, is a pre-emergent and post-emergent herbicide that controls weeds by inhibiting the plant-specific enzyme, acetohydroxyacid synthase, which is involved in the synthesis of three specific amino acids: isoleucine, leucine and valine. This inhibition disrupts protein synthesis and subsequently interferes with DNA synthesis and cell growth (BASF 2003b). Imazapic is readily absorbed through leaves, stems, and roots and is translocated rapidly throughout the plant, with accumulation in the meristematic regions which is where growth originates. Treated plants stop growing soon after spray application. Chlorosis appears first in the newest leaves, and necrosis spreads from this point. In perennial species, the herbicide is translocated into the underground storage organs which prevents regrowth (BASF 2006). As a post-emergent herbicide, complete kill of susceptible plants may not occur for several weeks after application. As a pre-emergent herbicide, seeds susceptible to the herbicide fail to germinate and/or seedlings fail to establish.

Plant response to Plateau® varies by species, season, and exposure to the chemical. Generally, warm season species that germinate and grow in late spring and summer are tolerant of fall imazapic application, while cool season species that germinate and grow in winter or early spring are more commonly intolerant of fall herbicide application. Based on field trials (BASF 2004, Monaco et al. 2005) and experimental treatments within ZION (Louie et al. 2005), some native grass species that occur in the project area and are known to be tolerant of fall application of imazapic at a rate of 8 ounces per acre or more include: big bluestem (Andropogon gerardii), little bluestem (Schizachyrium scoparium), indiangrass (Sorghastrum nutans), needlegrass (Stipa spp), needleandthread (Hesperostipa comata), purple threeawn (Aristida oligantha), Sandberg’s bluegrass (Poa sandbergii), western wheatgrass (Pascopyrum smithii), bottlebrush squirreltail (Sitanian hystrix), sideoats grama (Bouteloua curtipendula), galleta grass (Pleuraphis jamesii), and blue grama (Bouteloua gracilis). While most of the project area was burned and the majority of the above ground plants within the project area were either consumed or scorched by the fire, there are approximately 1,234 acres of unburned lands within the project area that include some living plants. Also some plants have germinated or re-sprouted since the fire, most notably galleta grass (Pleuraphis jamesii). Most plants within the project area, with the exception of riparian vegetation, would be sprayed with Plateau®. But because the herbicide is highly selective, a minimum of native vegetation would be affected. Fortunately, there are other non-native species in the project area that are suppressed or not tolerant of the proposed herbicide treatment, including orchardgrass (Dactylis glomerata), crested wheatgrass (Agropyron cristatum), Kentucky bluegrass (Poa pratensis), ripgut brome (Bromus diandrus), medusahead (Taeniatherum caput-medusae), field bindweed (Convolvulus arvensis), puncture vine (Tribulus terrestris), mustards (Brassica spp.), perennial pepperweed (Lepidium latifolium), Russian thistle (Salsola iberica), and whitetop (Cardaria spp.). The response of herbaceous wildflower species to the proposed treatment is more variable and there are few local species that have been tested. Generally, some composite and legume species are known to be intolerant or grow with reduced vigor after treatment while others appear to be tolerant. Many native tree and shrub species are generally tolerant of fall application, including sagebrush (Artemisia spp.), juniper (Juniperus spp.), oak (Quercus spp.), serviceberry (Amelanchier alnifolia), and chokecherry (Prunus virginiana). Other native tree and shrub species are less tolerant, particularly of foliar application, including: ash (Fraxinus spp.), boxelder (Acer negudo), cottonwood (Populus spp.), currant (Ribes spp.), and western snowberry (Symphoricarpos occidentalis). Pinyon pine has not been evaluated and the response of other pine species is variable, with some species being tolerant and others being susceptible to damage by foliar application. Taken together, most native species are unlikely to be affected by the herbicide and established perennial species that persist in unburned pockets are very unlikely to be affected because the riparian species would be avoided and the upland species are generally tolerant.

Research conducted in many areas throughout the Great Basin and Intermountain West found that cheatgrass can be reduced by more than 90 percent the first year after treatment (BASF 2003b) with 12 ounces per acre fall application rates, but there are more non-target impacts to desirable plants at these higher application rates as well as with spring season application. Conditions within the project area are ideal for treatment because the Kolob Fire removed most of the overstory and ground cover which exposes mineral soil, and post-fire rainfall in July washed away most of the surface ash. Realistically, it is expected that an 8 ounces per acre fall application in the project area would result in about 80-90 percent control of cheatgrass the first year with declining levels of control the following two growing seasons, and no residual herbicide control after 3 years. Ideally, the release of the native plant species from cheatgrass competition coupled with the availability of suitable germination sites and increased soil fertility soil that result from the fire, would allow the native species to become established and increase their competitive capacity for subsequent growing seasons. As cheatgrass seedbanks are relatively short lived, and most cheatgrass seed either germinates and grows or is not viable after 1 year (Meyer 2003), the suppression of cheatgrass germination for 3 years as a result of the herbicide treatment should provide adequate time for the native plant communities to re-establish. Once that native plant community is firmly re-established, it would be more resistant to wholesale cheatgrass invasion, although careful monitoring and follow-up spot treatments along invasion corridors (i.e. roads and trails) would still be needed.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would allow native plant communities to regenerate and persist. This would preserve the fullest complement of native plant species, communities, and ecosystem processes. Before the fire, there were still intact native plant communities throughout the burned area so it is expected that the seedbanks of the native species are viable enough to re-establish and perpetuate native communities in the project area if cheatgrass is controlled and the grass-fire cycle interrupted at this early stage. These healthy native plant communities would be more resistant to future cheatgrass invasion from surrounding lands. In the absence of cheatgrass, future fires in the project area would be within the natural fire regime and therefore would be less frequent, smaller in size, and lower in intensity than fires that burn in cheatgrass environments. The primary reasons for this difference are due to later green-up and die-back of native species—providing less available dry fuels, and  to the discontinuous spacing of fuel and the percent bare ground that naturally exists in the Great Basin and Colorado Plateau shrubland and woodland environments. These intershrub spaces are also important for the colonization or succession of biological soil crusts.

Cumulative Impacts. The proposed action as well as the native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control would all maintain native plant communities and species. The replacement of the boundary fence would indirectly help protect recovering plants in the burned area from impacts caused by illegal off-road vehicles or trespass livestock grazing. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting vegetation, would result in short-term, minor cumulative negative impacts to some plants and long-term moderate or cumulative positive impacts to native plant communities and populations.

Conclusion. The proposed action would result in short-term, minor negative impacts to some native plants due to herbicide exposure and long-term, moderate positive impacts due to perpetuation of native shrublands and woodlands.

Impairment. Because there would be no major, adverse impacts to vegetation whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s vegetation resources from the implementation of Alternative B.

## Threatened, Endangered, and Sensitive Plant Species

Affected Environment

One federally-listed endangered plant species, the Shivwits milkvetch (Astragalus ampullarioides), occurs in Zion. This species was listed in 2001 by the U.S. Fish and Wildlife Service (USFWS) because of its extremely limited range on the Chinle Formation and its rapidly vanishing habitat due to development (USDI USFWS 2001a). Informal consultation with the USFWS specific to the Kolob Fire has indicated that there are no populations of this plant within the project area and it is unlikely that any suitable habitat for this species exists within the project area.

Zion also hosts 22 plant species considered “sensitive” by the park and the state of Utah because of their limited distribution (endemism) or are disjunct from more abundant population centers. Most of these species are specialized to sandstone crevices and derived soils or hanging gardens. Many of these habitats are not susceptible to fire, and therefore fires are infrequent or rare in areas where these plants may occur. Three known populations were within the burned area: Zion daisy (Erigeron sionis), Religious daisy (Erigeron religiosus), and Zion penstemon (Penstemon humilus var. obtusifolia).  Table 6 lists the affected plant species by habitat and sensitivity of that habitat to fire.

| Table 6: Sensitive Plant Species by Habitat |  |  |  |  |
| --- | --- | --- | --- | --- |
| Common Name | Scientific Name | Family Name | Fire-prone Habitat | Habitat |
| Zion penstemon | Penstemon humilis var. obtusifolius | Scrophulariaceae | Yes | Ponderosa pine forest understory or pinyon pine understory |
| Religious daisy | Erigeron religiosus | Asteraceae | Yes | Dry meadows |
| Shivwits milkvetch | Astragalus ampullarioides | Fabaceae | Rare | Chinle Formation1 |
| Zion daisy | Erigeron sionis | Asteraceae | Rare | Hanging garden or wetland |
| 1 Habitat not fire-prone, but with invasion of red brome fire frequency can dramatically increase |  |  |  |  |

Impact Threshold Definitions

| Negligible | No federally listed species or sensitive species would be affected or the alternative would affect an individual of a listed species, its critical habitat, or a sensitive species, but the change would be so small that it would not be of any measurable or perceptible consequence to the protected individual or its population. |
| --- | --- |
| Minor | The alternative would affect an individual(s) of a listed species, its critical habitat, or a sensitive species, but the change would be small. |
| Moderate | An individual or population of a listed species, its critical habitat, or a sensitive species would be noticeably affected. The effect would have some consequence to the individual, population, or habitat. |
| Major | An individual or population of a listed species, its critical habitat, or a sensitive species would be noticeably affected with a vital consequence to the individual, population, or habitat. |
| Duration | Short-term - recovers in less than one year |
|  | Long-term – requires more than one year to recover |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary |

Effects of Alternative A – No Action Alternative

Federally Listed Species. Shivwits mikvetch is not known to occur within the project area. However, there could be undiscovered populations within the project area.

Perpetuation of the grass-fire cycle in the project area could lead to increased fuel loads and cheatgrass invasion would increase the flammability of the landscape in general which may impact Shivwits milkvetch populations or suitable habitats outside of the project area. These changes could result in long-term negative impacts to this species due to increased fire frequency, fire size, and fire intensity and the resulting fire suppression effort.

State-Listed or Other Sensitive Species. Since aerial application of herbicide would not be undertaken in the no action alternative, there would be no short-term impacts to state-listed or other sensitive plant species in the project area.

Long-term, the lack of treatment would allow the grass-fire cycle to be perpetuated and overtime the native woodlands and shrublands would be replaced by invasive grasslands. The perpetuation of the grass-fire cycle in the project area would lead to increased fuel loads and cheatgrass invasion would alter the suitability of the habitat for these species. These changes could result in long-term negative impacts to this species due to increased fire frequency, fire size, and fire intensity and the resulting fire suppression effort. As the Zion penstemon and religious daisy occur in flammable habitats, they would be most directly impacted by the increased flammability of the landscape. Together these increased fire effects and fire suppression effects are likely to negatively impact virtually all state-listed or other sensitive plant species both within the project area and potentially into surrounding areas.

Cumulative Impacts. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. The resulting increase in flammability of the landscape and degradation of native plant communities would likely reduce the sustainability of and potentially extirpate threatened, endangered, and sensitive plant populations in the project area. Overall, impacts of no action added to the impacts of other actions affecting vegetation, would result in long-term, moderate or cumulative negative impacts to threatened, endangered, and sensitive plant species.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to threatened, endangered, and sensitive plant species, but would result in long-term, moderate negative impacts to these plants due to perpetuation of the grass-fire cycle. The landscape level conversion of native woodlands and shrublands to invasive grasslands could ultimately lead to extirpation of threatened, endangered and sensitive plant species within the project area and could contribute to their losses in adjacent lands.

Effects Determination. Implementation of the no action alternative with increased fire effects and fire suppression effects may affect, not likely to adversely affect Shivwits milkvetch populations or habitats.

Impairment. Because there would be no major, adverse impacts to threatened and endangered plant species or sensitive plant species whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s threatened and endangered plant species or other sensitive plants from the implementation of Alternative A.

Effects of Alternative B – Proposed Action/Preferred Alternative

Federally Listed Species. Known Shivwits mikvetch populations are not within the project area. However, there could be undiscovered populations within the project area. If plants exist within the project area is unlikely they would be affected by the proposed action. This is because the proposed action would occur in the fall when Shivwits milkvetch is dormant. Also, the relatively small amount of herbicide that would remain in the soil by April is not likely to affect milkvetch emergence or germination. The herbicide selectively targets annuals. The use of Plateau® to control annual bromes in Zion Canyon has resulted in improved performance and release of other milkvetch species, in part due to the reduced competition with those annual brome grasses.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would reduce the flammability of the landscape in general which may help preserve Shivwits milkvetch populations or suitable habitats outside of the project area. By restoring natural fire regimes and not allowing for the cheatgrass-driven increased fire frequency, fire size, and fire intensity and the resulting fire suppression effort, the populations of this species outside the project area would likely benefit.

State-Listed or Other Sensitive Species. The Zion daisy occurs in wetland habitats, which would not be treated with herbicide because all wetlands are included in the no-spray zones. So there would be no short-term direct effects on this species.

The religious daisy occurs in dry meadow habitats that were probably affected by the fire and any populations within the project area would likely be exposed to herbicide. This species is an annual or short-lived perennial that flowers May through September, so might not be exposed to foliar application of herbicide applied in the fall unless plants germinated and grew after the fire. It is possible that the seedbanks of this species could be affected by the pre-emergent properties of the herbicide. This species has not been tested for it’s tolerance to this herbicide, and there is no published research that addresses imazapic tolerance of the genera Erigeron. Reported tolerances for the family Asteraceae are widely variable, with some species demonstrating full tolerance, while others show some level of suppression or negative affect, and others are intolerant. As this species is widespread in ZION and there are many extant populations, any loss of plants in the project area would be unlikely to affect the conservation status of this species or noticeably reduce its distribution or genetic diversity.

The Zion penstemon occurs in the understory of pinyon pine woodlands, and such habitats were burned in the Kolob Fire. This species is a perennial that flowers May through September, so plants might not be exposed to foliar application of herbicide applied in the fall unless plants germinated and grew after the fire. Where plants or seedbanks occur under the canopy of trees that were unburned or incompletely consumed by fire, these populations might be shielded from direct exposure to herbicide. However, where the overstory vegetation has been removed due to fire, it is likely that the plants and seedbanks would be exposed to herbicide. This species has not been tested for it’s tolerance to this herbicide, and there is no published research that addresses imazapic tolerance of the genera Penstemon or the family Scrophulariaceae. As this species is widespread in ZION and there are many extant populations, any loss of plants in the project area would be unlikely to affect the conservation status of this species or noticeably reduce its distribution or genetic diversity.

Long-term, the herbicide treatment would interrupt the grass-fire cycle. By restoring natural fire regimes and not allowing for the cheatgrass-driven increased fire frequency, fire size, and fire intensity and the resulting fire suppression effort, the populations of these species would likely benefit.

Cumulative Impacts. The proposed action as well as the native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control would all maintain native plant communities which include the threatened, endangered, and sensitive plant species. The replacement of the boundary fence would indirectly help protect recovering plants, including the state-listed rare plants, in the burned area from impacts caused by illegal off-road vehicles or trespass livestock grazing. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting threatened, endangered, and sensitive plant species, would result in short-term, minor cumulative negative impacts to some individual plants and long-term, moderate cumulative positive impacts to threatened, endangered, and sensitive plant populations.

Conclusion. The proposed action would result in short-term, minor to moderate negative impacts to some state listed plants due to herbicide exposure and long-term, moderate positive impacts due to perpetuation of native plant communities and suitable habitat for listed plant species.

Effects Determination. Implementation of the proposed action may affect, not likely adversely affect Shivwits milkvetch populations or habitats since the species is not know to occur in the project area.

Impairment. Because there would be no major, adverse impacts to threatened and endangered plant species or sensitive plant species whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s threatened and endangered plant species or other sensitive plants from the implementation of Alternative B.

- Wildlife

Affected Environment

The diverse vegetation communities within ZION support a variety of wildlife species. ZION is home to 6 species of amphibians, 28 species of reptiles, 79 mammal species, 289 bird species, and 7 fish species. Nevertheless, the inventory of wildlife in the park is incomplete. Particularly understudied are biologically rare species, nocturnal species, and the many invertebrate species.

Many species of birds and some mammal species, such as bats, are migratory. Consequently, the number of species and the size of populations vary considerably from season to season and place to place. Prior to the Kolob Fire, some of the most commonly sighted animals in the area that burned included mule deer, coyote, mountain lion, raptors, lizards, and rodents. Generally, animals that are highly mobile, such as large mammals and adult birds, are able to flee from the fire while burrowing animals escape the fire by going underground where the heat of the fire does not penetrate. Other animals, particularly rabbits and reptiles, lack these escape strategies and commonly perish in fires. After the fire, animals that use the area must cope with the burned landscape. In some cases, animals move into nearby unburned areas. Others find short-term shelter and wait for the vegetation to recover. Some animals, specifically ungulates and raptors, are known to seek out newly burned areas because of the food availability and, in the case of ungulates, the mineral nutrition that can be found in ash. Since the Kolob Fire, fresh sign of mountain lion as well as raptors and lizards have been found in the burned area. Long-term recovery of the burned area and the quality and variety of habitats it may provide in the future is probably the most important factor in sustaining native wildlife populations in the area.

Seed-eating animals, called granivores, play vital roles in desert ecosystems. The most common granivores in the desert are rodents, ants, and, to a lesser extent, birds. Granivores have been studied in both the Mojave and Sonoran Deserts and have been found to be very influential in the re-establishment of vegetation after fire (Esque et al 2004a). The “caching” of seed by these animals is a primary means of distribution of seed from nearby unburned areas into burned landscapes. However, rodents and other herbivores are also known to greatly reduce the viability of perennial woody plants that re-sprout after fire because these re-sprouts often represent the only food sources available in the burned area and they are sometimes eaten as quickly as they grow (Esque et al. 2004b). The ability to withstand this increased herbivory varies by species and is also affected by the growing conditions of the site. Thus the relationship between animals, especially granivores and herbivores, and the recovery of burned lands is complicated.

ZION has its full complement of four native fish species and few invasive exotic fish. This is because park streams retain natural flow regimes, there have been few introductions of exotic fish, and the native species are well adapted to the sediment laden floods that frequently occur while the exotic species are not.

Native fish occurring in the project area are the desert sucker (Catostomus clarkia) and speckled dace (Rhinichthys osculus) both common in North Creek and other streams in the park. Exotic rainbow trout (Oncorhynchus mykiss) have also established a breeding population in the park in North Creek and the Left and Right Forks due to past stocking by the NPS. Just outside of the park downstream of the project area in North Creek, and in the Virgin River downstream of Coalpits Wash, the Virgin spinedace (Lepidomeda mollispinis mollispinis) and flannelmouth sucker (Catostomus latipinnis) are found. Both the spinedace and the flannelmouth sucker are managed under conservation agreements in lieu of listing, so are considered special status species and discussed in the section of this document titled Threatened, Endangered, and Sensitive Animal Species.

Impact Threshold Definitions

| Negligible | Wildlife would not be affected or the effects would be at or below the level of detection and the changes would be so slight that they would not be of any measurable or perceptible consequence to the wildlife species’ population. |
| --- | --- |
| Minor | Effects to wildlife would be detectable, although the effects would be localized, small, and of little consequence to the species’ population. Mitigation measures, if needed to offset adverse effects, would be simple and successful. |
| Moderate | Effects to wildlife would be readily detectable, localized, and with consequences at the population level. Mitigation measures, if needed to offset adverse effects, would be extensive and likely successful. |
| Major | Effects to wildlife would be obvious and would have substantial consequences to wildlife populations in the region. Extensive mitigation measures would be needed to offset any adverse effects and their success would not be guaranteed. |
| Duration | Short-term – recovers in less than one year |
|  | Long-term – requires more than one year to recover |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary |

Effects of Alternative A – No Action Alternative

Since aerial application of herbicide would not be undertaken in the no action alternative, there would be no short-term impacts to animals or wildlife habitat in the project area.

Long-term, the lack of treatment would allow the grass-fire cycle to be perpetuated and overtime the wildlife habitat would be degraded due to cheatgrass invasion. Cheatgrass provides little forage value and is not palatable during most of the summer months due to their sharp awns (Mosley 1999). As cheatgrass invasion increases fire frequency, fire size, and fire intensity, animals would suffer increased mortality due to fire or fire suppression effects. Furthermore, the grass-fire cycle would eventually degrade the native plant communities which results in habitat alteration and even the loss of some habitats. The most noticeable change would be alteration of the habitat structure as woodlands and shrublands are eventually converted into grasslands. Granivores would be especially affected because the variety of seed would be greatly reduced and the ground surface, where ants and rodents tend to live, would be covered in thatch. The conversion to grassland would also cause displacement of animals into surrounding areas, however, cheatgrass invasion has already altered tens of millions of acres in the Intermountain West, so the availability of suitable habitat may be insufficient to support the current population sizes and species composition that exist now. Overtime, less mobile and less resilient species would be extirpated and those populations remaining in a cheatgrass dominated landscape would likely be reduced in size and vigor.

Cumulative Impacts. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. The resulting increase in flammability of the landscape and degradation of native plant communities would also degrade wildlife habitat in the project area. Overall, impacts of no action added to the impacts of other actions affecting wildlife, would result in long-term, moderate cumulative negative impacts to wildlife habitat.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to wildlife, but would result in long-term, moderate negative impacts to wildlife habitat and increase the potential for mortality of animals due to perpetuation of the grass-fire cycle. The landscape level conversion of native woodlands and shrublands to invasive grasslands would alter wildlife habitat and reduce the species diversity and population health of many wildlife species.

Impairment. Because there would be no major, adverse impacts to wildlife whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s wildlife by the implementation of Alternative A.

Effects of Alternative B – Proposed Action/Preferred Alternative

Under this alternative, a helicopter would be working in the project area for approximately 2 weeks to complete the herbicide application. All helicopter work would take place during daylight hours. Animals in the treatment area would likely respond to the noise from the helicopter and, while unlikely, some animals may be directly contacted with the herbicide mix as it is released from the helicopter. The nature of the response would vary by species and where the animal is in relation to the helicopter, but common responses might include immobilizing, fleeing to a burrow or crevice, or fleeing across the ground surface to vegetative cover. In all cases, the response would be very short in duration, probably lasting several minutes, and the extent of the response would be limited only to those animals near the helicopter. Fall season is an important time for animals to put away food reserves for the oncoming winter, but it is not a particularly sensitive time for most species because the young of the year have generally had several months to mature and are highly mobile at this time of year. The exception would be animals that breed and birth continuously, such as some rodent and insect species. For these species, there could be pregnant females present and/or very young animals present. The potential for impact to these animals would be higher than for more mobile animals.

Those species that immobilize in response to an intrusion in their environment, such as the helicopter, are most likely to be directly exposed to the herbicide mix. Imazapic is neither an acute or chronic toxicant to mammals, birds, fish, aquatic invertebrates, or honey bees (BASF 2006). Other groups of animals were not specifically reported, but effects on herpetofauna and invertebrates (other than bees and aquatic) are likely to be similar. Studies have found that ingested imazapic is readily excreted unaltered in urine and feces so it does not bioaccumulate in the food chain (BASF 2006). This means that predators are not dosed with the chemical as a result of the prey that they consume. Imazapic does not persist in water because it binds to soil and any free chemical in water solution quickly photo degrades within hours (BASF 2006), so potential for wildlife consumption of contaminated water is very low.

In studies, Methylated seed oils have been found to be minimal irritant to eyes and skin (Loveland 2004c-d). Liberate® has been to be found to be a nonirritant to eyes and a moderate irritant to skin (Loveland 2004a-b). Neither compound has been listed as having carcinogenic potential.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would allow native plant communities to regenerate and provide habitat for native wildlife. This would prevent cheatgrass from dominating the landscape, thus preserving native woodland and shrubland habitats that provide high quality forage and habitat structure. Granivores would still have access to a wide variety of seeds. While the habitat requirements vary by species, sustainable native wildlife populations require native habitats. This would assure that native wildlife populations continue to thrive in the park.

Cumulative Impacts. The proposed action as well as the native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control would all maintain native plant communities which serve as wildlife habitat. The replacement of the boundary fence would indirectly help protect recovering habitats and animals present in the burned area from impacts caused by illegal off-road vehicles or trespass livestock grazing. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting wildlife, would result in short-term, minor cumulative negative impacts to some individual animals and long-term moderate cumulative positive impacts to wildlife habitat.

Conclusion. The proposed action would result in short-term, minor negative impacts to some animals due to herbicide exposure and response to the helicopter. There would be long-term, moderate positive impacts due to perpetuation of native shrublands and woodlands which provide wildlife habitat.

Impairment. Because there would be no major, adverse impacts to wildlife whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s wildlife by the implementation of Alternative B.

## Threatened, Endangered, and Sensitive Animal Species

Affected Environment

An endangered species is defined as a species that is in danger of extinction throughout all or a significant portion of its range. A threatened species is defined as a species that is likely to become endangered within the foreseeable future throughout all or a significant portion of its range.

ZION is within the Colorado Plateau Recovery Unit for the Mexican spotted owl (Strix occidentalis lucida) which is federally listed as a threatened species. The Mexican spotted owl reaches the northwestern limits of its range in this recovery unit (USDI USFWS 1995), and all of ZION is designated as critical habitat for this species (USDI USFWS 2001b). ZION has 20 known Mexican spotted owl territories, which are widely distributed (USDI NPS 2005). A spotted owl monitoring program for the park was initiated in 1995 and continues today.

One known and occupied nesting site is located near, but outside of the eastern edge of the project area. Using a 0.5 mile buffer around the nesting site, approximately 40 acres of the buffer zone would fall within the project area. As the buffer represents important foraging habitat, impacts to this area are an important consideration. After the fire, wildlife biologists surveyed the canyon and found the pair had not been affected by the fire.

The following background information comes from the Biological Opinion issued for the FMP by the Utah Field Office in February 2005 (USDI USFWS 2005).

The entirety of Zion National Park is designated critical habitat for Mexican spotted owl. Primary constituent elements related to critical habitat in Utah include one or more of the following: (1) presence of water (often providing cooler temperatures and higher humidity than the surrounding areas); (2) clumps or stringers of mixed conifer, pine-oak, pinyon-juniper, and/or riparian vegetation; (3) canyon wall containing crevices, ledges, or caves; and (4) high percent of ground litter and woody debris. The primary constituent elements provide a qualitative description of those physical and biological features necessary to ensure the conservation of the owl in Utah (50 FR 53182).

The park initiated informal consultation with the USFWS regarding implementation of the BAR Plan, including the proposed action to apply herbicide aerially over the burned area.

Two other protected animal populations occurred in the burn perimeter, although neither is federally listed at this time.

Although the peregrine falcon (Falco peregrinus anatum) was removed from the federal list of endangered and threatened species in 1999, ZION has continued to monitor territories associated with climbing routes. ZION is known to have 15 historic falcon territories. A subset of those territories and the climbing route territories are monitored each year (USDI NPS 2001d). Two peregrine territories were within the burned area. In one of the territories, the young had fledged before the fire. The other territory was not surveyed this year, so impacts to those birds are unknown. Peregrine falcons prey on other bird species, usually capturing their prey in mid-flight. In the park, they nest on rock ledges and cliff faces.

Just outside of the park downstream of the project area in North Creek, and in the Virgin River downstream of Coalpits Wash, the Virgin spinedace (Lepidomeda mollispinis mollispinis) and flannelmouth sucker (Catostomus latipinnis) are found. The spinedace are found in abundance in both streams, even though the population in North Creek is occasionally lost when drought and agricultural uses dry up the lower portions of the creek, as happened in 2002. Flannelmouth sucker prefer habitats in larger rivers, so they are less common in North Creek. Spinedace and flannelmouths are blocked from moving further up North Creek by a barrier waterfall at the park boundary, and from moving into Coalpits Wash by the small flow and seasonal drying. While neither species is found in the project area, they are both downstream and could be affected by actions taken or not taken in the project area. Both the Virgin spinedace and flannelmouth sucker are managed under a Conservation Agreement to avoid listing as threatened. As such they are afforded protections similar to listed species.

Two fish species listed as endangered, the Virgin River chub (Gila seminude) and woundfin (Plagopterus argentissimus) are found in the Virgin River several miles downstream of the project area. Though early records are limited, it appears that these two species never occurred much upstream of the Timpoweap Canyon and Pah Temp Hot Spring, both well downstream of the park.

Impact Threshold Definitions

| Negligible | No federally listed species or sensitive species would be affected, or the alternative would affect an individual of a listed species, its critical habitat, or sensitive species, but the change would be so small that it would not be of any measurable or perceptible consequence to the protected individual or its population. |
| --- | --- |
| Minor | The alternative would affect an individual(s) of a listed species, its critical habitat, or sensitive species, but the change would be small. |
| Moderate | An individual or population of a listed species, its critical habitat, or sensitive species would be noticeably affected. The effect would have some consequence to the individual, population, or habitat. |
| Major | An individual or population of a listed species, its critical habitat, or sensitive species would be noticeably affected with a vital consequence to the individual, population, or habitat. |
| Duration | Short-term - recovers in less than one year |
|  | Long-term – requires more than one year to recover |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary |

Effects of Alternative A – No Action Alternative

Federally Listed Species. Since aerial application of herbicide would not be undertaken in the no action alternative, there would be no short-term impacts to the Mexican spotted owl or its habitat.

Long-term, the lack of treatment would allow the grass-fire cycle to be perpetuated and overtime the owl’s habitat would be degraded due to cheatgrass invasion. The most direct effect would be an alteration of the prey base as some rodent populations would probably be reduced in the cheatgrass dominated landscape. Secondarily, the ability of the owls to detect and capture rodents would probably be diminished due to the thick ground cover and thatch that exist in cheatgrass grasslands. Additionally, as cheatgrass invasion would result in increased fire frequency, fire size, and fire intensity, trees available for roosting and cover would eventually be reduced or even eliminated, from the landscape. These factors would permanently reduce the distribution of two of the four constituent elements of the critical habitat: (1) clumps or stringers of mixed conifer, pine-oak, pinyon-juniper, and/or riparian vegetation; and (2) high percent of ground litter and woody debris.

State-Listed or Other Sensitive Species. Since aerial application of herbicide would not be undertaken in the no action alternative, there would be no short-term impacts to the peregrine falcon or its habitat, or the Virgin spinedace and flannelmouth sucker or their habitat.

Long-term, the lack of treatment would allow the grass-fire cycle to be perpetuated and overtime the habitat for these species would be degraded due to cheatgrass invasion. As cheatgrass invasion would result in increased fire frequency, fire size, and fire intensity, there would be a loss of the woodlands and shrublands that provide habitat for the bird species that serve as prey for the peregrine falcons. This reduction in prey base would reduce the suitability of the habitat for peregrine falcons and would likely decrease the number of birds the area could support. Increased fire also means increased post-fire watershed effects which could reduce the habitat quality of the Virgin spinedace and flannelmouth sucker downstream of the project area. The primary influence would be changes in water chemistry and turbidity that result from increased sediment and ash transport into streams from burned areas. The degree to which these watershed events would cause direct mortality of fish or reduce the suitability of their habitat would vary based on the magnitude and duration of the watershed event and the life stage of the fish at the time of the event.

Cumulative Impacts. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. The resulting increase in flammability of the landscape and degradation of native plant communities would likely reduce the capacity of the habitat for threatened, endangered, and sensitive animal species. Overall, impacts of no action added to the impacts of other actions affecting critical wildlife habitat, would result in long-term, moderate cumulative negative impacts to threatened, endangered, and sensitive animal species through habitat alteration.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to threatened, endangered, and sensitive animal species, but would result in long-term, moderate negative impacts to these animals due to perpetuation of the grass-fire cycle. The landscape level conversion of native woodlands and shrublands to invasive grasslands would potentially reduce the suitability of the habitat that currently supports Mexican spotted owl and peregrine falcon. It could also contribute to the loss of these bird species in adjacent lands and could have negative impacts to the habitat of the fish species downstream of the project area.

Effects Determination. Implementation of the no action alternative would have no effect on Mexican spotted owls, but could adversely affect critical habitat for this species.

Impairment. Because there would be no major, adverse impacts to threatened and endangered animal species or sensitive animals whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s threatened and endangered animal species or other sensitive animals by implementation of  Alternative A.

Effects of Alternative B – Proposed Action/Preferred Alternative

Federally Listed Species. Under this alternative, a helicopter would be working in the project area for approximately 2 weeks to complete the herbicide application. All helicopter work would take place during daylight hours. As the owl nest is located well east of the project area and owls normally roost in their nesting area during the daylight hours, it is unlikely that owls would be directly exposed to herbicide or directly affected by the helicopter operations. The noise generated by the helicopter could temporarily disrupt their diurnal rest for short periods of time during the one or two days that the helicopter would be working on the eastern edge of the project area. The work would be completed outside critical breeding periods for spotted owl.

It is unlikely that owls would be indirectly exposed to herbicide because studies have found that ingested imazapic is readily excreted unaltered in urine and feces so it does not bioaccumulate in the food chain (BASF 2006). This means that predators, like owls, are not dosed with the chemical as a result of the prey that they consume. Imazapic does not persist in water because it binds to soil and any free chemical in water solution quickly photo degrades within hours (BASF 2006), so potential for consumption of contaminated water is very low.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would allow native plant communities to regenerate and provide habitat for native wildlife, including Mexican spotted owl. This would prevent cheatgrass from dominating the landscape, thus preserving native woodland and shrubland habitats that provide two of the four constituent elements of critical habitat for the species: (1) clumps or stringers of mixed conifer, pine-oak, pinyon-juniper, and/or riparian vegetation; and (2) high percent of ground litter and woody debris.

State-Listed or Other Sensitive Species. Under this alternative, a helicopter would be working in the project area for approximately 2 weeks to complete the herbicide application. During this time, peregrine falcons in the project area would likely be affected by the helicopter presence. Because adult birds are highly mobile, both the peregrine falcons and their prey birds would be expected to avoid the area where the helicopter is working at the time. For this reason, it is highly unlikely that either the falcons or their prey birds would be directly exposed to herbicide during application. As imazapic does not bioaccumulate, there would be no trophic effects on peregrine falcons. There are no Virgin spinedace in the project area, so there would be no potential for direct herbicide exposure. Furthermore, the no-spray zones are designed to reduce the likelihood that herbicide would enter the streams. See the water resource section for more information on pathways for herbicide to enter streams and the impacts and fate of herbicide in water.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would allow native plant communities to regenerate and provide habitat for native wildlife, including the peregrine falcon, Virgin spinedace, and flannelmouth sucker. This would prevent cheatgrass from dominating the landscape, thus preserving native woodland and shrubland habitats that provide habitat for the prey base of the peregrine falcon and preserve the water conditions and riparian vegetation needed to support Virgin spinedace and flannelmouth sucker.

Cumulative Impacts. The proposed action as well as the native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control would all maintain native plant communities which serve as habitat for threatened, endangered, and sensitive animal species. The replacement of the boundary fence would indirectly help protect recovering habitats and animals present in the burned area from impacts caused by illegal off-road vehicles or trespass livestock grazing. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting threatened, endangered, and sensitive animal species, would result in minor cumulative positive impacts to threatened, endangered, and sensitive animal species.

Conclusion. The proposed action would result in short-term, negligible negative impacts to some birds due to the noise generated by the helicopter and there would be no short-term impacts on fish species. There would be minor positive impacts to threatened, endangered, and sensitive animal species due to perpetuation of suitable habitat.

Effects Determination. Implementation of the proposed action would have no effect on Mexican spotted owls because the action would occur outside the critical breeding season and would occur outside any protected activity centers. The implementation of the proposed action may affect, not likely to adversely affect critical habitat for this species.

Impairment. Because there would be no major, adverse impacts to threatened and endangered animal species or sensitive animals whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s threatened and endangered animal species or other sensitive animals by implementation of Alternative B.

- Soils

Affected Environment

The geologic formations within the burned area were created during the formation of the Colorado Plateau during the Mesozioc era. Originally created from ancient sediments, the major formations that occur within the burned area from oldest to youngest are: Moenkopi, Chinle, Moenave, Kayenta, Navajo, Temple Cap, Quaternary basalt flows and cinder cones, and Quaternary slide deposits. The Quaternary slide deposits include fragmented rock fall debris and talus materials cemented with calcite. Holocene (recent) alluvium deposits occur in channels, floodplains and on stream terraces. These formations and surface materials are the parent materials for the soils in the area.

The Washington County Area, Utah soil survey was used to obtain information on soils (USDA 1977). There are 22 soil map units that occur within the burned area (Table 7).

| Table 7: Soil Map Units Within the Kolob Fire Burned Area |  |  |  |
| --- | --- | --- | --- |
| Soil Type | Percent of Soil Within Burned Area | Percent Slope | Pre-fire Water Erosion Hazard |
| Rock land, stony | 20 | 30-70 | moderate |
| Mathis-Rock outcrop complex | 18 | 20-50 | severe |
| Rock outcrop | 10 | variable | moderate |
| Clovis-Pastura complex | 9 | 1-10 | moderate |
| Stony colluvial land | 9 | 30-70 | moderate |
| Bond sandy loam | 8 | 1-10 | moderate |
| Clovis fine sandy loam | 5 | 1-5 | moderate |
| Veyo-Pastura complex | 5 | 1-10 | slight |
| Badland, very steep | 4 | >30 | very high |
| Naplene silt loam | 2 | 2-6 | moderate |
| Cinder land | 2 | variable | moderate |
| Magotsu-Pastura complex | 2 | 2-20 | moderate |
| Badland | 1 | variable | very high |
| Rock land | 1 | variable | moderate |
| Spenlo very fine sandy loam | 1 | 2-10 | moderate |
| Pastura-Esplin complex | 1 | 0-10 | moderate |
| Fluvaquents & Torrifluvents, sandy | <1 | variable | severe |
| Collbran very cobbly clay loam | <1 | 2-30 | moderate |
| Gravel pits | <1 | variable | slight |
| Gullied land | <1 | variable | severe |
| Palma fine sandy loam | <1 | 1-5 | moderate |
| Mespun fine sand | <1 | 0-10 | moderate |

Below are brief descriptions of the major soil map units that occur over more than 5 percent of the Kolob Fire burned area:

Rock land, stony and Rock outcrop: Consists of stony and bouldery soils with sandstone outcrops and cliffs. Stones and boulders are commonly underlain by sand, shale, or siltstone and weathered rock. Soil development is minor due to continual deposits of material from higher lying slopes. Rock outcrops are mostly sandstone, limestone, conglomerate, or basalt.

Mathis-Rock outcrop complex: Consists of somewhat excessively drained soils on severely eroded, dissected mountain side slopes and mesa remnants. Surface soil is typically very stony loamy fine sand with sandstone bedrock at a depth of about 30 inches.

Clovis-Pastura complex: Occurs mainly on old basalt mesas with the Clovis soil on gently sloping swales and Pastura soil on ridges and steeper slopes.

Stony colluvial land: Consists of unconsolidated colluvial land covered with stones and rock fragments. Shale bedrock is generally at a depth of less than 12 inches. There are a few small areas of shallow soils.

Bond sandy loam: Consists of shallow, well-drained soils on high, broad mesa tops. Soils formed in residuum weathered from conglomerate and sandstone.

ZION also contains notable amounts of biological soil crusts where the soil surface is bound together by a community of algae, fungi, lichen, and other microorganisms. This soft crust greatly increases the soils ability to capture and hold water, fix nitrogen from the atmosphere, and resist erosion from wind, raindrop impact and flowing water (Belnap et al. 2001). ZION does not have detailed field surveys to determine the distribution of biological soil crusts. However, these crusts are typically associated with open canopies and sandy soil usually found in pinyon/juniper woodlands and desertshrub communities. Such conditions characterize most of the burned area. Biological crusts are usually killed by hot surface fires, which generally correspond to the moderate to high soil burn severity. The amount of damage and the potential for post-fire recovery of biological crusts depends on the pre-fire vegetation, fire intensity, and fire frequency. One of the biggest threats to the recovery of biological crusts after fire is the potential for cheatgrass invasion (Belnap et al. 2001), which decreases suitable growing surfaces and increases the risk of repeated fire.

Impact Threshold Definitions

| Negligible | Soils would not be affected or the effects to soils would be below or at the lower levels of detection. Any effects to soil productivity or fertility would be slight. |
| --- | --- |
| Minor | The effects to soils would be detectable. Effects to soil productivity or fertility would be small, as would the area affected. If mitigation were needed to offset adverse effects, it would be relatively simple to implement and likely successful. |
| Moderate | The effect on soil productivity or fertility would be readily apparent and result in a change to the soil character over a relatively wide area. Mitigation measures would probably be necessary to offset adverse effects and would likely be successful. |
| Major | The effect on soil productivity or fertility would be readily apparent and would substantially change the character of the soils over a large area in and outside of the park. Mitigation measures to offset adverse effects would be needed and would be extensive; their success could not be guaranteed. |
| Duration | Short-term - recovers in less than three years |
|  | Long-term - requires more than three years to recover |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary |

Effects of Alternative A – No Action Alternative

Since aerial application of herbicide would not be undertaken in the no action alternative, there would be no direct short-term impacts to soils. Because cheatgrass would likely invade and create a relatively consistent ground cover, the soil erosion would be reduced in the project area in the short-term. Until the next fire, more soil would remain in place, supporting plant growth.

Long-term, the lack of treatment would allow the grass-fire cycle to be perpetuated and overtime the landcover would be altered as woodlands and shrublands would be replaced by invasive grasslands. As cheatgrass invasion would result in increased fire frequency, fire size, and fire intensity, there would eventually be an increase in soil loss from the project area as each fire event would leave exposed soil vulnerable to erosion until plants become re-established. In a cheatgrass dominated landscape, the time period in which soil may be exposed after fire is relatively short, but is likely to correspond with the late summer monsoon thunderstorms that have the most capacity to carry soil away. Top soil would be removed in sheet erosion, and both top soil and subsoil would be transported by rilling and gullying. Soil would be redeposited at the toe of the slope, or more likely, carried into streamchannels with the stormflow. This persistent loss of soil, particularly top soil, would eventually reduce the productivity and soil fertility of the project area. As the grass-fire cycle is known to preclude the recolonization or succession of biological soil crusts, it is likely that the distribution of biological crusts would be reduced and there could be changes in species composition within the crustal communities to favor fire tolerant species or those species that can quickly re-establish after fire.

Cumulative Impacts. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. The resulting increase in flammability of the landscape and degradation of native plant communities would likely increase soil erosion. Increased soil erosion after fires would result in localized loss of soil productivity and fertility. Overall, impacts of no action added to the impacts of other actions affecting soil, would result in long-term, minor cumulative negative impacts to soils.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to soils, but would result in long-term, minor negative impacts to soils due to increased potential for post-fire erosion and subsequent localized loss of soil productivity and fertility.

Impairment. Because there would be no major, adverse impacts to threatened and endangered plant species or sensitive plant species whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s soils by implementation of Alternative A.

Effects of Alternative B – Proposed Action/Preferred Alternative

Herbicide application would have no affect on the soil parent material or soil formation processes. The primary potential for impact is through chemical changes in soil and/or changes in the movement of soil in the landscape.

Based on field dissipation studies, imazapic is moderately persistent in soil with a loss of 50 percent of initial concentration in 7 to 150 days depending upon soil type and climatic conditions (BASF 2006). Observations of test applications of imazapic in Zion Canyon have shown effective control of annual brome grasses for two growing seasons (Louie et al. 2005), indicating that persistence in this area is near the maximum value. Using a maximum persistence of 150 days to lose 50 percent of initial concentration, and carrying out that loss over time, in 3 years there would be less than 1 percent of the initial concentration of imazapic remaining in the soil.

Imazapic has limited mobility in soil due to soil binding, a complex function of soil pH, texture, and organic matter content. The binding of imazapic to soil increases with time. From a total of nine soil dissipation studies conducted with imazapic, no residues were found below the 18-24 inch soil layer. Imazapic does not volatilize from the soil surface and the ability of imazapic to decompose on the soil surface due to sunlight is negligible. The major route of imazapic loss from soil is through microbial degradation. The fate of herbicide in soil that is transported from the treatment area in rainfall events is discussed in the water resources section. As soil productivity is the capacity of a soil to support plant growth and the purpose of this treatment is to suppress growth of specific plant, namely cheatgrass, the application of herbicide would initially decrease soil productivity and fertility.

Since there would be less cheatgrass covering the soil surface, it would be more vulnerable to erosion during the winter. However, the emergence of native plants in spring would quickly reduce that vulnerability. The potential for soil movement during the winter is lessened by the fact that winter precipitation is normally relatively low intensity with minimal erosive capacity. As such, the increase in short-term erosion rates due to the increased exposure of mineral soil in the winter and early spring that would result from cheatgrass suppression would be negligible to minor, depending on the occurrence of heavy winter precipitation.

It is likely that the period of increased vulnerability to soil erosion would extend into the following summer due to the lack of litter cover that would have been contributed by the cheatgrass. The risk of erosion is moderated by the relatively high infiltration capacity of many of the sandy soils and the presence of abundant coarse fragments armoring the soil surface on steeper slopes, the effectiveness of which was demonstrated by the relative lack of soil movement during the heavy rainfall event of July 28, 2006. The impact to soils would be negligible to minor and dependent on rainfall patterns.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would allow native plant communities to regenerate. This would reduce the frequency of fire-associated soil loss because vegetation dominated by native species has lower fire frequency, intensity and fire size when compared to vegetation communities with cheatgrass as a major component. Maintaining the natural fire regime would result in a moderate improvement in long-term soil productivity and fertility, when compared to the no action alternative.

Cumulative Impacts. The proposed action as well as the native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control would all maintain native plant communities which in turn help maintain soil productivity and fertility. The replacement of the boundary fence would indirectly help protect soils, particularly biological soil crusts, from impacts caused by illegal off-road vehicles or trespass livestock grazing. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting soil, would result in short-term, minor cumulative negative impacts to soil and long-term, moderate cumulative positive impacts to soil.

Conclusion. The proposed action would result in short-term, minor negative impacts to soil due to decreased soil productivity as a result of herbicide and long-term, moderate positive impacts due to perpetuation of natural fire regimes that decrease the frequency and magnitude of post-fire erosion events.

Impairment. Because there would be no major, adverse impacts to soils whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s soils by implementation of

Alternative B.

- Water Resources

Affected Environment

Most of the park is characterized by an arid climate. This is true of the project area which is located at lower elevations in the southwest portion of the park. Based on elevation, nearby climate stations and vegetation, the average annual precipitation in the project area is estimated to be 11 inches in the southern portions, increasing with elevation to about 15 inches along the northern and eastern margins. Climate data from the park station located 8 miles east of the project area is representative of the wetter portions of the project area.

Through the year, precipitation patterns include:

- A winter to early spring wet season that peaks from January through March,

- A reliable late spring and early summer dry season in May and June, that is also the period of most frequent fires,

- A summer rainy season (or monsoon) typically occurring between mid July through September, and

- Fall seasons that are moderately dry.

About 60 percent of the precipitation falls during the winter months (October through April) typically as frontal storm systems that produce rainfall or snow over large areas. About 40 percent of the precipitation falls during the summer monsoon when a southerly air flow brings moist air that produces scattered thunder storms to some degree most days. These sometime produce intense local rainfall and large runoff events. Precipitation is highly variable from year to year, and the summer storms are notably variable from place-to-place. Winter precipitation typically shows a marked increase during “El Niño” events, while the summer monsoon shows little if any response to the El Niño Southern Oscillation. Droughts are common and often occur over several years while wet years tend to occur as isolated events.

Soil erosion and floods occur as a result of high-intensity rainfall events, or rarely at these elevations, from rain-on-snow events. To understand the frequency and magnitude of these events the National Oceanic and Atmospheric Administration has developed a model of high intensity rainfall events based on the climate records from the park climate station. Table 8 presents the amount of precipitation that has occurred in a given time interval based on how frequently that magnitude of storm has occurred (or its return interval). For example, a rainfall accumulation of 1.02 inches in one hour has occurred, on average, once every 10 years at the park climate station.

| Table 8: Precipitation Frequency |  |  |  |
| --- | --- | --- | --- |
| Estimates From Park Weather Station (inches) |  |  |  |
| Average Return Intervals (years) | Duration of Storm |  |  |
|  | 15 minutes | 60 minutes | 6 hours |
| 1 | 0.28 | 0.46 | 0.79 |
| 2 | 0.36 | 0.60 | 0.98 |
| 5 | 0.49 | 0.82 | 1.24 |
| 10 | 0.61 | 1.02 | 1.46 |
| 25 | 0.80 | 1.33 | 1.78 |
| 50 | 0.96 | 1.60 | 2.04 |
| 100 | 1.15 | 1.92 | 2.34 |
| 500 | 1.72 | 2.87 | 3.32 |
| 1000 | 2.03 | 3.38 | 3.85 |
| Excerpted from Precipitation-Frequency Atlas of the United States, NOAA Atlas 14, Volume 1, Version 3, 2003 |  |  |  |

Watersheds and Streamflow. The project area drains to the Virgin River that is located at the southern edge of the fire. One perennial stream, North Creek, drains the higher terrain in ZION and flows for about 18 miles to the Virgin River at the town of Virgin, Utah. It crosses the northern part of the project area in a northeast to southwest direction. Two major tributaries of North Creek, the Left Fork and Right Fork join in the project area. North Creek and is tributaries flow within deep canyons that are several hundred to almost 2,000 feet deep. The streams have built narrow, relatively flat terraces and floodplains in the canyon bottoms that are typically 40 to 80 feet wide and covered with a mixture of upland pinyon-juniper vegetation on the higher terraces and riparian cottonwood-willow vegetation nearer the stream channel. Outside of these terraces, the terrain is dominated by steep slopes covered with rubble from the cliffs above and some areas of exposed rock outcrop. The numerous short steep rocky channels that drain these slopes flow only in immediate response to heavy precipitation.

Summer and fall base flow in North Creek is about 3 cubic feet per second (cfs). Somewhat higher flows in the winter combine with spring runoff and other flood events to raise the average discharge to 5.6 cfs. Spring runoff from snowmelt in the higher headwaters may reach 60 to 100 cfs in wet years, while only about 10 cfs after dry winters.

Approximately 18 percent of the North Creek watershed burned, and about 11 percent of the watershed is in the project area, with the remaining 7 percent of the burned watershed located outside the park boundary.

The other major drainage in the project area is Coalpits Wash draining the southern half of project south to the Virgin River. Coalpits Wash has a small perennial flow upstream and east of the burn area, but surface flow becomes seasonal by the time it reaches the project area and continuing on to the confluence with the Virgin River downstream from Rockville. During the winter, when evapotranspiration from riparian plants is minimal, streamflow in Coalpits Wash is about 1 cfs. With summer heating surface stream flow dries up entirely. Spring runoff is less significant in Coalpits when compared to North Creek and flows may rise above 10 cfs for only a few days. Two small springs, with a combined flow of less than 2 gallons per minute, discharge from the Shinarump Conglomerate near where the large dry tributary meets Coalpits Wash east of Crater Hill.

Even though the Coalpits Wash watershed in the project area is somewhat less steep and rugged than the North Creek drainage, there remains several hundred feet of relief and the area is incised by many dry channels that have cut into the softer bedrock. Outcrops of Chinle Formation form colorful badlands. The south slopes of Cougar Mountain are steep and rubble covered. In the center of the watershed several channels cut into Pleistocene lakebed sediments that were deposited behind the Crater Hill Lava dam. South of there, where Coalpits Wash has cut below the Basalt and Shinarump Conglomerate, it flows through a steep gorge cut into the soft Moenkopi Formation. On the west side of this gorge, which is in the project area, the steep slopes are covered with basalt talus derived from the Crater Hill Basalts.

The slopes of Crater Hill, a cinder cone deposited about 300,000 years ago, are steep but the soil surface is well armored by cinders and infiltration rates are high. Near the base of Crater Hill are some relatively flat benches with deeper soils that are more prone to erosion, and have, in fact, been dissected by several ephemeral channels that have cut to bedrock.

About 40 percent for the watershed of Coalpits Wash burned in the Kolob Fire. Almost all of this land is included in the project area.

Two other small portions of the project area drain into other watersheds. About 1 square mile of land west of the Kolob Terrace Road drains into Black Wash then into North Creek. Two small areas totaling about 3.5 square miles drain south and west to Dalton Wash. These are the southwest slopes of Crater Hill and west of Cougar Mountain.

Flash flooding occurs on all channels in the project area. These are most common with summer monsoon storms, which often produce the largest flood events of the year, but can occur when heavy precipitation occurs during any season. Based on the streamflow record for the North Fork of the Virgin River in Zion Canyon, a very large flood event in North Creek would be about 6,000 to 7,000 cfs. A similar estimate of a large flood event in Coalpits Wash would be 4,000 to 5,000 cfs.

Water Quality. According to Standards of Quality for Waters of the State, R317-2, Utah Administrative Code, waters in North Creek and Coalpits Wash are protected for domestic water supply(category 1C), secondary contact recreation (category 2B), nongame fish and aquatic life (category 3C), and agriculture uses (category 4). The numeric values associated with these protected uses are the standards referred to in the following description of water quality.

Sediment and turbidity are the most significant water quality characteristics of the rivers and streams in the park. While most streams are relatively clear during periods of low flow (<15 Nephelometric Turbidity Units (NTU)), high flows are accompanied by large increases in sediment transport and turbidity exceeding 1,000 NTU for brief periods.

Of the water sources in the project area, only North Creek has had water quality analysis, and most of that was conducted well downstream of the park. North Creek has waters dominated by calcium, magnesium and sulfate ions with a well buffered average pH of 8.2 and an average concentration of total dissolved solids (TDS) of 1,150 milligrams/liter. There have been frequent exceedances of the State of Utah standard of TDS for agricultural use of water (1,200 mg/l). A State investigation of this problem concluded that the source of the dissolved solids was probably a combination of natural sources (primarily Moenkopi Formation downstream of the park) and irrigation return flows. Very limited sampling inside the park indicates that TDS levels are less than half those found downstream. Sampling downstream also found occasional exceedances of standards for selenium and exceedances of recommended values for total phosphorus.

Monthly water quality monitoring began in North Creek in the project area in the fall of 2005 and will continue. Field data is collected for discharge, water and air temperature, turbidity, specific conductance, dissolved oxygen and pH. Water samples are collected and analyzed for major ions, metals and nutrients. Preliminary measurements in the month following the fire found an increase in pH to 9.2 and an increase in algal growth indicating a flush of nutrients from the recently burned lands, which is a common occurrence after fire.

Little is known of water quality in Coalpits Wash. A single measurement found the TDS concentration to be 1,520 mg/l. The small spring adjacent to Coalpits Wash east of Crater Hill had a TDS of 6,800 mg/l. There are no human induced sources of contamination in Coalpits Wash watershed beyond recreational campers.

Sediment Yield. Streams draining from ZION are prodigious producers of sediment due to the phenomenal rate of canyon cutting that is occurring. Using estimates of sediment yields developed for the North Fork of the Virgin River (800,000 to 1,000,000 tons of sediment per year) the smaller watershed of North Creek is estimated to yield on average 250,000 to 300,000 tons per year. Coalpits Wash is estimated to yield 50,000 to 80,000 tons per year. The vast majority of this sediment moves during flood events, so there is a high degree of variability from day-to-day and year-to-year depending on the number, magnitude and duration of flood events. The predominant size class of sediment in the Virgin River Basin is sand due to the abundance of eroding sandstone. Streams in the project area probably produce more cobbles and boulders than other park streams due to the abundance of durable basalt from the lava flows.

An exceptional type of sediment laden flow called a “debris flow” occurs in the steepest intermittent tributary channels. A debris flow occurs when a flood gains sufficient sediment, typically from a slope failure, so that it flows as a thick slurry with a consistency of wet concrete. While relatively slow moving, debris flows carry a large amount of energy and are capable of considerable damage when they encounter man-made structures. Debris flows from these steep channels typically loose energy and drop their sediment when they encounter the more gradual slopes of the larger channels of North Creek and Coalpits Wash.

Wetlands. Wetlands occur in the project area as narrow strips along perennial and seasonal streams and at a few spring discharges. Specifically, riparian wetlands occur along the Left and Right Forks of North Creek, North Creek proper, Trail Canyon and Coalpits Wash. These were formed, watered and maintained by the stream channels. In this area they are indicated by a Fremont cottonwood tree canopy with components of velvet ash, Goodings willow and box elder. Understory vegetation includes coyote willow, seep willow, sedges (Carex spp.), horsetail (Equisetum spp.), cattail, bullrush (Juncus spp.), and grasses. Since inundation is only occasional, several upland plant species are also found. This vegetation type is mapped in the National Wetland Inventory (USDI USFWS 2000) as Rp1FO6 (Riparian, flowing water, forested, deciduous (cottonwood)). The actual extent of these linear wetlands is relatively small, totaling about 70 acres in the project area.

Under natural conditions fires are rare in these wetlands, which are not generally adapted to fire. Where human activity has introduced fire or non-native plants that promote fire have invaded, the result can be an increase in mortality among trees of all age classes. Plants that grow in the wettest areas and have growing points below ground (such as horsetail, sedge, bullrush, and willow) will often survive fire and sprout vigorously. Mortality among riparian trees from the Kolob Fire appears to be about 50 percent, though this impression may change over time as plants that have been stressed succumb, and species with the ability to resprout quickly recover.

Wetlands near springs in this area are generally very small. The largest is at Grapevine Spring located adjacent to the Left Fork of North Creek at the northern margin of the project area. It is mapped at about 7 acres of Rp1FO6 (Riparian, flowing water, forested, deciduous (cottonwood)) wetland. This spring has been the subject of several studies and is an example of a very diverse wetland. The other spring wetlands are located along steep tributaries of Trail Canyon north and east of Cougar Mountain. They originate with spring discharges at the base of the Navajo Sandstone and extend as very narrow corridors down to the bottom of Trail Canyon. They are not identified in the NWI, but are mapped as Fremont Cottonwood – Velvet Ash complex in the Zion National Park Vegetation Maps.

NPS Director’s Order 77-1 provides guidelines for the protection of wetlands within NPS units. It states a policy of no net loss of wetlands and provides a process for evaluating actions that have a potential to have adverse effects on wetlands. When proposed actions have an adverse impact on wetlands, a Wetlands Statement of Findings is required to describe why the action is necessary and how impacts to wetlands are to be mitigated. In this project a Statement of Findings is not needed because wetlands would specifically be avoided by designating no-spray zones and buffer strips along all perennial and seasonal streams and springs (Figure 2).

Impact Threshold Definitions

| Negligible | Water quality, hydrology, and wetlands would not be affected, or changes would be either non-detectable or, if detected, would have effects that would be considered slight and local. |
| --- | --- |
| Minor | Changes in water quality, hydrology, and wetlands would be measurable, although the changes would be small and the effects would be localized. No mitigation measure associated with water quality or hydrology would be necessary. |
| Moderate | Changes in water quality, hydrology, and wetlands would be measurable, but would be relatively local. Mitigation measures associated with water quality or hydrology would be necessary and the measures would likely succeed. |
| Major | Changes in water quality, hydrology, and wetlands would be readily measurable, would have substantial consequences, and would be noticed on a regional scale. Mitigation measures would be necessary and their success would not be guaranteed. |
| Duration | Short-term – would occur within the first year following treatment |
|  | Long-term – would continue more than one year following treatment |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary as well as the downstream drainages |

Effects of Alternative A – No Action Alternative

Since aerial application of herbicide would not be undertaken in the no action alternative, there would be no potential for herbicide to enter surface or ground water.

Long-term, the lack of treatment would allow the grass-fire cycle to be perpetuated and overtime the land cover would be altered as woodlands and shrublands would be replaced by invasive grasslands. A complete conversion to shallow rooted grasses would likely increase the amount of groundwater recharge that would occur in the project area. The effect of this on stream flow is uncertain and likely negligible since the geologic strata of the project area are almost entirely below the Navajo Sandstone, the major groundwater aquifer, and contribute very little to overall streamflow. The period during which there is a short-term increase in flood peaks following fires would be shortened by the rapid growth of cheatgrass, however these periods would be more frequent. Stream water quality would experience short-term increases in sediment loading, nutrients, ash and pH with each major fire typically lasting 1 to 2 years.

Frequent fire would likely remove the fire intolerant riparian vegetation along stream channels, resulting in a moderate increase in water temperature, and minor alterations of stream channel morphology, water chemistry, and wetland functions. Also, more frequent fires would result in increases in turbidity, sediment and ash transport from hillslopes to channels, as well as the loss of sediment retention and filtration functions that would have been performed by the riparian buffers along streams. Given the already high level of erosion and sediment transport from the landscape, the magnitude of impacts from an increased fire frequency due to the increased presence of cheatgrass is minor.

Cumulative Effects. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. The resulting increase in flammability of the landscape and degradation of native plant communities would likely increase the frequency and intensity of post-fire watershed response, affecting sediment yield, water quality, and stream morphology in and downstream of the project area. Overall, impacts of no action added to the impacts of other actions affecting water resources, would result in long-term, moderate cumulative negative impacts to water resources.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to water resources, but would result in long-term, moderate negative impacts to water resources due to increased potential for post-fire erosion and subsequent alteration of sediment yield, water quality, and stream morphology.

Impairment. Because there would be no major, adverse impacts to water resources whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s soils by implementation of Alternative A.

Effects of Alternative B – Proposed Action/Preferred Alternative

There are two primary pathways for herbicide to enter water resources. First, is direct application of herbicide during treatment and second is transport of soil containing herbicide into water resources.

An approximation of the degree of risk to fish from incidental contamination during spraying can be gained by estimating the concentration of herbicide in the water if the streams were sprayed at the same concentration as the surrounding lands. North Creek and the Left and Right Forks have about 3.5 miles of channel and about 3.49 acres of water surface in the project area. The volume of water in these channels at base flow is estimated at 73,000 cubic feet or 2 million liters. Were the entire water surface of 3.49 acres sprayed with 8 ounces per acre, a total of 27.92 ounces (825 milligrams) of herbicide would be applied to the water surface, resulting in a roughly estimated water concentration of 0.0004 mg/l, or several orders of magnitude less then the lethal dose of >100 mg/l for bluegill sunfish.

Nevertheless, the NPS considers the application of herbicide to streams as undesirable and contrary to the pesticide label. Part of the proposal is to designate no-spray zones along all perennial and seasonal flow channels, springs, and wetlands that are likely to have water at or near the surface. This would make the potential for any herbicide to get into the water very unlikely. All such features would be buffered by 100 feet on either side to assure that there would be essentially no opportunity for spray, directly or in drift, to enter those waters. Additionally, the application equipment (boom, nozzles, droplet size, etc.) and technique (elevation, air speed, direction of travel) would be designed to reduce drift. With the mitigation measures identified the expected width of the transition zone between fully sprayed and no-spray along the direction of travel would be 20-40 feet. The no-spray zones would be loaded into an on-board computer system that would allow the pilot to accurately navigate around those features.

In consideration of the possible movement of herbicide from sprayed soil into streams, both the persistence of the chemical in the soil and sediment transport within the system are examined. While winter precipitation is greater, it is generally of low intensity and therefore rarely results in large scale mobilization and transport of the soil surface. Since the herbicide application would probably take place in the fall, the first time that sediment transport is likely to occur in the treated watersheds would be in July 2007 with the onset of the summer monsoon season and its intense thunderstorms. Imazapic loses 50 percent of initial concentration in 7 to 150 days depending upon soil type and climatic conditions as a result of plant uptake and microbial degradation (BASF 2006). Using a maximum reported persistence of 150 days to lose 50 percent of initial concentration, and carrying out that loss over time, only about 30 percent of the initial concentration would still be present in the soil in July, the rest would have been lost through plant uptake or microbial degradation. Imazapic has limited mobility in soil due to soil binding, which increases with time (BASF 2006), so it is progressively less likely to leach out of the soil into surface water. The preservation of the riparian corridors in the stream buffers would capture some of the contaminated soil before it enters the stream channels, but some amount would potentially be transported into stream channels. Once soil containing herbicide enters stream channels, much would remain with the soil particles and would eventually be re-deposited as soil within the floodplain. The concentration of any herbicide that did go into solution would be very low due to the limited mobility of the herbicide and the large volume of water and sediment moving through the system during such run-off events.

Furthermore, imazapic is reported to rapidly photo degraded by sunlight with a half-life in water of less than 8 hours (BASF 2006), so any herbicide that would go into solution would be very short lived. Because imazapic binds to soil and dissipation studies have not found any chemical residues below 18-24 inch soil layer, it is highly unlikely that imazapic would get into groundwater.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would allow native plant communities to regenerate. This would reduce the watershed impacts of burned lands (increased surface flows, erosion, ash flows, and nutrient flushes) that result from increased fire frequency, fire size, and fire intensity. The wetlands and riparian vegetation would not be removed by fire and would continue to provide a host of ecological benefits.

Cumulative Effects. The proposed action as well as the native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control would all maintain native plant communities which in turn help maintain natural watershed characteristics, including streamflows, sediment transport, water quality, and wetlands. The replacement of the boundary fence would indirectly help protect stream channels from impacts caused by illegal off-road vehicles or trespass livestock grazing. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting water resources, would result in short-term, minor cumulative negative impacts to water resources and long-term, moderate cumulative positive impacts to water resources.

Conclusion. The proposed action would result in short-term, minor negative impacts to water resources and long-term, moderate positive impacts to water resources due to perpetuation of natural fire regimes that decrease the frequency and magnitude of post-fire watershed responses.

Impairment. Because there would be no major, adverse impacts to water resources whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s soils by implementation of the Alternative B.

- Natural Soundscapes

Affected Environment

Natural soundscapes are comprised of the natural sound conditions that exist in the absence of any human-produced noises. These conditions are actually composed of many natural sounds, near and far, which often are heard as a composite, not individually. Natural sound conditions include the sounds of running water, blowing wind, chirping birds, and many other sounds found in nature. The opportunity to experience ZION’s natural soundscape unimpaired by the sounds of human civilization is an important part of the overall visitor experience, especially as it contributes to the solitude and wilderness experience that is integral to much of the park.

Acoustic data has been collected in ZION over the years. The most recent and most comprehensive data collection effort was by Wyle Laboratories (Hobbs and Downing 2003), which collected acoustic data from October 2000 to November 2001 at 13 sites throughout the park. The North Creek site is within the area burned. The Tabernacle Dome site is just north of the burned area. The data was collected during spring, summer, and fall at 12 sites and during all four seasons at one site. The data suggests that ZION is a quiet soundscape. Little variation in the soundscape was observed across the park, during the day, and throughout the year. The recorded ambient noise level in the park in backcountry settings is approximately 20 A-weighted decibels (dBA), which is the noise level equivalent to a quiet house at midnight. The recorded ambient noise level in the park in frontcountry settings is approximately 40 dBA.

Human-generated noise in the park is predominantly from vehicle traffic, aircraft overflights, and maintenance and administrative activities. Frontcountry areas, such as near roads, often have higher levels of noise. Mechanical noises, such as those produced by aircraft, can drown out these natural sounds on a temporary basis.

Impact Threshold Definitions

| Negligible | Frontcountry Low Development – Noise created by human activities associated with the alternative may be present during the daylight hours, but would rarely be audible between sunset and sunrise. |
| --- | --- |
|  | Primitive and Pristine – Natural sounds predominate. Noise created by human activities associated with the alternative is rarely audible. When noise is present, it is at low levels and occurs for only short durations in a small geographic area. Visitors almost always have the opportunity to experience the natural soundscape free from noise. |
| Minor | Frontcountry Low Development – Noise created by human activities associated with the alternative may predominate during the daylight hours, but the majority of the time the noise is at low levels, and is only rarely at greater than medium levels. Noise created by human activities associated with the alternative is rarely audible between sunset and sunrise. |
|  | Primitive and Pristine – Natural sounds usually predominate. Noise created by human activities associated with the alternative is infrequent, and occurs for only short durations in most of the area. Visitors almost always have the opportunity to experience the natural soundscape free from noise created by human activities associated with the alternative most of the time in the majority of the area. |
| Moderate | Frontcountry Low Development – Noise created by human activities associated with the alternative predominates during the daylight hours, but only at medium or lower levels a majority of the time. Localized areas may experience noise at medium to high levels half of the daylight hours. Noise created by human activities associated with the alternative is occasionally audible between sunset and sunrise. |
|  | Primitive and Pristine – Noise created by human activities associated with the alternative is present infrequently too occasionally, at low to medium levels and durations. Portions of these zones within 0.5-mile of the Frontcountry Low Development Zone often experience noise at low or medium levels and durations. Noise created by human activities associated with the alternative is occasionally audible between sunset and sunrise. |
| Major | Frontcountry Low Development – Noise created by human activities associated with the alternative predominates during daylight hours, and is at greater than medium levels a majority of the time that noise is present. Large areas may experience noise at medium to high levels during the majority of the daylight hours. This noise is often audible between sunset and sunrise. |
|  | Primitive and Pristine – Natural sounds are commonly masked by noise created by human activities associated with the alternative at low or greater levels for extended periods of time. Portions of the zones within 0.5-mile of the Frontcountry Low Development Zone often experience noise at medium levels and durations, and noise levels in these areas occasionally are high. Noise created by human activities associated with the alternative is frequently audible between sunset and sunrise. |
| Duration | Short-term – effects extend only through the duration of the proposed project |
|  | Long-term – effects extend beyond the period of the proposed project |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary |

Effects of Alternative A – No Action Alternative

Since aerial application of herbicide would not be undertaken in the no action alternative, there would be no short-term increase in mechanical noise generated by treatment activities, such as the drone of helicopter operating in the project area for a 2 week period and the occasional sounds of pumps and equipment needed to fill and service the aircraft at the helispot.

Long-term, the lack of treatment would allow the grass-fire cycle to be perpetuated and overtime there would be an increase in fires and fire suppression activities. Fire suppression activities generate noise due to the use of mechanical equipment, such as helicopters, pumps, and chainsaws. Noise levels for these pieces of equipment vary by model and modification, but for reference chainsaws generate 125dBA and helicopters operating 200 feet above the ground generate 99 dBA. This fire suppression equipment would be used for short durations and in local areas so sound would dissipate quickly and variations in vegetation and topography would also minimize sound impacts with distance. However, the use of fire suppression equipment in the project area is expected to increase with the increased fire frequency, fire size, and fire intensity, so such noise intrusions would become increasingly common during the summer months. Also, fires burning in continuous fuels created by cheatgrass are notoriously difficult to control so firefighting resources are likely to be more numerous and effective fire control would be more reliant upon heavy equipment and aerial suppression tactics.

Cumulative Effects. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. As no action means there would be no helicopter, there would be no short-term, cumulative impacts to soundscapes. There are no long-term cumulative impacts to soundscapes.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to soundscapes, but would result in long-term, negligible negative impacts to soundscapes due to noise from increased fire suppression activity.

Impairment. Because there would be no major, adverse impacts to natural soundscapes whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s natural soundscapes from the implementation of Alternative A.

Effects of Alternative B – Proposed Action/Preferred Alternative

Under this alternative, a helicopter would be working in the project area for approximately 2 weeks to complete the initial herbicide application. All helicopter work would take place during daylight hours. The helicopter would generally be flying about 200 feet above the ground, depending on the terrain and target, so the noise generated would be about 99 dBA. This increase in noise is five times what the ambient noise is in the backcountry settings of the project area and about double what the ambient noise is in the frontcountry settings in or near the project area, specifically Kolob Terrace Road. The noise would be loudest closest to the helicopter and would diminish with distance. As the area actively being treated would be closed to the public during treatment, it is unlikely that people other than those working on the project would experience the maximum noise level.

The helispot near Coalpits Wash would experience the most frequent noise and the most variation in noise levels due to the re-filling and servicing of the helicopter that would take place at that location. Each helicopter landing and take-off would temporarily increase noise at the helispot. The helispot would also experience localized, short-term noise generated by the intermittent use of the fuel pump used to dispense fuel into the helicopter as well as the mechanical agitator and pump used to mix the herbicide and fill the tank of the helicopter. Generators may be used to power the pumps, but it is most likely that the contractor would have a self-contained fuel truck and mix truck that would handle all mechanical needs associated with the operation. Some of the noise of the helispot would be noticeable from Highway 9 corridor, but would likely be low key due to the distance from the highway and the fact that the terrain. The helispot is approximately 100 yards north of Highway 9, behind a large earthen berm. The Coalpits Trailhead near the helispot would be closed to use during operations, so the soundscape experience of visitors would not be impacted.

Long-term, the herbicide application would interrupt the grass-fire cycle so fires would be less frequent than if no action were taken. Fewer fires would require fewer fire suppression operations and less noise would be generated. Furthermore, any fires that would start after successful herbicide treatment would likely be smaller and more easily handled with minimum impact suppression tactics which usually generate less noise than other firefighting tactics.

Cumulative Effects. The proposed action as well as the proposed aerial seeding of roadsides and the East Tunnel construction project would all use helicopters, heavy equipment, or rock blasting that would increase ambient noise in two different areas of the park during daylight hours. Road corridors in the project areas are zoned as Frontcountry Low Development and the remaining lands are generally zoned as either Primitive or Pristine. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting soundscapes, would result in short-term, negligible cumulative negative impacts to soundscapes in frontcountry settings during daylight hours and short-term, moderate cumulative negative impacts to soundscapes in primitive or pristine settings during daylight hours.

Conclusion. The proposed action would result in short-term, negligible negative impacts to soundscapes in frontcountry settings during daylight hours and short-term, moderate negative impacts to soundscapes in primitive or pristine settings during daylight hours as a result of helicopter noise. There would be long-term minor positive impacts to soundscapes due to restoration of natural fire regimes and reduced frequency of noise generated by fire suppression activities.

Impairment. Because there would be no major, adverse impacts to natural soundscapes whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s natural soundscapes from the implementation of Alternative B.

## Wilderness

Affected Environment

In 1974, approximately 131,000 acres of ZION were recommended to Congress for formal wilderness designation (Figure 4); these lands include most of the Kolob Fire burned area with the exception of the Kolob Terrace Road corridor. While not yet legislatively designated, this recommended wilderness is managed as wilderness in accordance with NPS Management Polices (USDI NPS 2001b). These areas provide visitors an opportunity to experience Zion’s natural soundscape unimpaired by the sounds of human civilization.

The 1964 Wilderness Act defined wilderness as “an area where the earth and its community of life are untrammeled by man.” In addition, the act states that “except as necessary to meet the minimum requirements for the administration of the area for the purposes of this act, there shall be no temporary road, no use of motor vehicles, motorized equipment or motorboats, no landing of aircraft, no other form of mechanical transport, and any structure or installation within any such area.”

In this environmental assessment, wilderness includes both the biophysical resources of wilderness as well as wilderness character, which can be thought of as the human experience of wilderness. Two commonly used terms to describe wilderness resources and character are “naturalness” and “wildness.” While the two terms are similar, they each describe a different value of wilderness. Naturalness encompasses the ecosystem components and processes that belong in the wilderness, such as native plant communities and native wildlife species. Wildness encompasses the lack of direct human control, such as vast roadless landscapes and free-flowing rivers. Some things are both natural and wild (i.e. a bear wandering through a mountain meadow) while others are either natural or wild (i.e. flood that results from a ruptured dam is wild but not natural; while the release of a captive bred condor is natural but not wild).

All activities affecting wilderness must be considered under the minimum requirement concept. This concept is a documented process used to determine if administrative activities affecting wilderness resources or the visitor experience are necessary and how to minimize impacts. The park Wilderness Committee met to review the proposed action. The Committee determined through the minimum requirement analysis process that the action could be completed within the recommended wilderness area without long-term impairment of wilderness character and that the use of a helicopter to apply the herbicide was the minimum tool to use to produce the long-term benefits of the proposed action.

Impact Threshold Definitions

| Negligible | A change in the wilderness character could occur, but it would be so small that it would not be of any measurable or perceptible consequence. |
| --- | --- |
| Minor | A change in the wilderness character and associated values would occur, but it would be small and, if measurable, would be highly localized. |
| Moderate | A change in the wilderness character and associated values would occur. It would be measurable but localized. |
| Major | A noticeable change in the wilderness character and associated values would occur. It would be measurable and would have a substantial or possibly permanent consequence. |
| Duration | Short-term – effects would extend only through the duration of the proposed project |
|  | Long-term – effects would extend beyond the period of the proposed project |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary |

Figure 4 – Recommended Wilderness and Visitor Use

Effects of Alternative A – No Action Alternative

Under this alternative, aerial herbicide application would not occur so there would be no use of aircraft over wilderness and a foreign substance, herbicide, would not be broadcast into the ecosystem. In this way, wildness would not be directly impacted. The visitor experience of a wild landscape free from human intervention would not be impacted.

However, without the aerial application of herbicide, the grass-fire cycle would continue uninterrupted. As described in the vegetation section of this document, non-native cheatgrass would eventually come to dominate the project area and spread into the surrounding areas, including the more remote interior areas of the park. Fueled by the cheatgrass, fire frequency, fire size, and fire intensity would increase. Both cheatgrass and the fires fueled by cheatgrass are unnatural in the ZION wilderness. Overtime, natural components of the wilderness, such as native vegetation communities and the native wildlife species they provide habitat for, would be reduced and possibly even locally extirpated due to the continuation of the grass-fire cycle. This would result in long-term impacts to naturalness. Furthermore, the increase in fire frequency, fire size, and fire intensity as well as the resulting fire suppression activities would negatively impact the visitor experience of wilderness with increasing regularity.

Cumulative Effects. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. The increased flammability of the landscape would result in loss of native plant communities and wildlife habitat would reduce the naturalness of the wilderness. It would also result in more fire suppression efforts due to increased fire frequency, which would temporarily reduce the wildness of the wilderness. Overall, impacts of no action added to the impacts of other actions affecting wilderness, would result in long-term, moderate cumulative negative impacts to wilderness.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to wilderness, but would result in long-term, moderate negative impacts to wilderness due to loss of naturalness resulting from increased fire suppression activity as well as the loss of native plant communities and wildlife habitat.

Impairment. Because there would be no major, adverse impacts to wilderness whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s wilderness form the implementation of Alternative A.

Effects of Alternative B – Proposed Action/Preferred Alternative

Under this alternative, a helicopter would be working in the project area for approximately 2 weeks to complete the herbicide application. All helicopter work would take place during daylight hours. There would be temporary visitor use restrictions in various sections of the project area to assure that there are no visitors where herbicide is actively being applied. Short-term, such restrictions would greatly reduce the sense of wildness and freedom for which wilderness is sought by visitors. Furthermore, areas adjacent to the closures would still be open to visitor use but the sight and sound of the helicopter would be apparent to visitors using those areas and would diminish their sense of wildness. Long-term, visitors who know that the area had been treated with herbicide might feel that the land was less wild and value their experience less.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would allow native plant communities to regenerate and provide habitat for native wildlife. As the native plant communities and the native wildlife species as well as the related ecosystem processes (e.g. nutrient cycling) are all natural features of the wilderness, their perpetuation would serve to preserve the naturalness of the wilderness. Furthermore, interruption of the grass-fire cycle would restore natural fire regimes, characterized by infrequent and small fires. The restoration of natural fire regimes would enhance naturalness of the wilderness.

Cumulative Effects. The proposed action as well as the proposed aerial seeding of roadsides and the East Tunnel construction project would use helicopters, heavy equipment, or rock blasting that would increase ambient noise in two different areas of the park during daylight hours. Such noise would be noticeable in some wilderness areas and would negatively impact the wilderness experience of some users in those surrounding areas. Long-term, the proposed action as well as the native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control would all maintain native plant communities and wildlife habitat, thus enhancing the naturalness of the wilderness in the project area. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting wilderness, would result in short-term, moderate cumulative negative impacts to wilderness and long-term, moderate cumulative positive impacts to wilderness.

Conclusion. The proposed action would result in short-term, moderate negative impacts to wilderness due to the introduction of herbicide and the intrusion of the helicopter in wilderness. There would be long-term, moderate positive impacts to wilderness due to perpetuation of native plant communities, wildlife habitats, and natural fire regimes.

Impairment. Because there would be no major, adverse impacts to wilderness whose conservation is (1) necessary to fulfill specific purposes identified in the established legislation of Zion; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s GMP or other relevant NPS planning documents, there would be no impairment to the park’s wilderness form the implementation of Alternative B.

## Public Health and Safety

Affected Environment

The health and safety of visitors, park staff, and adjacent landowners are of the utmost importance to ZION and the NPS. The park has identified two primary concerns related to health and safety from the proposal to use aerial application of herbicide to restore native plant communities. The first concerns hazards directly related to the helicopter and herbicide application operations that could affect the public, contractors, and agency personnel involved in the treatment. The second concerns hazards posed by the result of undertaking the treatment or not undertaking the treatment, and include the fate of herbicides in the environment and the impacts of treatment or non-treatment on future fire frequency and intensity.

The north and east sides of the project area are park lands and are mostly used for dispersed recreation. A backcountry permitting system for overnight and canyon use allows the park to have some control over that use and provides an opportunity to inform visitors of risks inherent to the burned landscape. The south side of the project area is very narrow, about 0.5 mile, and is bordered by the right-of-way of State Highway 9 which serves as a major travel route for local communities as well as the primary entrance to the park. Across the highway to the south, Bureau of Land Management (BLM) and private lands abut the Virgin River. The west side of the project area borders private land for 2.1 miles while the remaining 5.7 miles are managed by the BLM. Both the private and BLM managed lands along the west boundary of the project area are primarily used for ranching, although grazing has recently been deferred due to the lack of forage created by the fire. Additionally, the BLM managed lands are used for hunting with deer season scheduled for October 21 through 24, 2006.

Impact Threshold Definitions

| Negligible | Public health and safety would not be affected, or the effects would be at low levels of detection and would not have an appreciable effect on the public health or safety. |
| --- | --- |
| Minor | The effect would be detectable, but would not have an appreciable effect on public health and safety. If mitigation was needed, it would be relatively simple and likely successful. |
| Moderate | The effects would be readily apparent and would result in substantial, noticeable effects to public health and safety on a local scale. Mitigation measures would probably be necessary and would likely be successful. |
| Major | The effects would be readily apparent and would result in substantial, noticeable effects to public health and safety on a regional scale. Extensive mitigation measures would be needed, and their success would not be guaranteed. |
| Duration | Short-term - effects lasting for the duration of the treatment action |
|  | Long-term - effects lasting longer than the duration of the treatment action |
| Area of Analysis | Park and surrounding communities |

Effects of Alternative A – No Action Alternative

Under this alternative, aerial herbicide application would not occur so there would be no potential for impacts to human health and safety due to helicopter operations or herbicide in the environment.

However, without the aerial application of herbicide, the grass-fire cycle would continue uninterrupted. As described in the vegetation section of this document, non-native cheatgrass would eventually come to dominate the project area and spread into the surrounding areas, including the more remote interior areas of the park. Fueled by the cheatgrass, fire frequency, fire size, and fire intensity would increase. Fires fueled by cheatgrass are more difficult to suppress due to long flame lengths and rapid rates of spread (BASF 2003a). These conditions increase the risk to firefighters and reduce the effectiveness of many firefighting tactics (BASF 2003a). Increased fire frequency, fire size, and fire intensity coupled with increased fire intensity also increase the likelihood that future fires could burn structures, utility corridors, and road corridors that may impact the life and safety of surrounding landowners and the visiting public. The extent to which firefighter and public life and safety is compromised in future fires would depend on many factors, such as location, weather conditions, communication systems, and timing; however, it would be more difficult to protect people from wildland fire if cheatgrass invasion continues uninterrupted. Increased fire would also mean increased smoke production, which is a known inhalation hazard as well as reduces visibility which results in an increased likelihood of car accidents on smoky roadways.

Cumulative Effects. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. As no action means there would be no helicopter or herbicide, there would be no short-term, cumulative impacts to public health and safety. The increased flammability of the landscape would result in increased potential for fire and fire suppression related hazards for the visiting public, park neighbors, and firefighters. This would result in long-term, minor cumulative negative impacts to public health and safety.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to public health and safety, but would result in long-term, minor negative impacts to public health and safety due to increased exposure to fire and fire suppression hazards resulting from increased fire frequency, fire size, and fire intensity.

Effects of Alternative B – Proposed Action/Preferred Alternative

Under this alternative, a helicopter would be working in the project area for approximately 2 weeks to complete the herbicide application. All of the direct hazards associated with this operation would be mitigated. All helicopter work would take place during daylight hours by qualified aircraft and pilot and work/rest requirements would be observed. There would be temporary visitor use restrictions put in place in various sections of the project area to assure that there are no visitors where herbicide is actively being applied or wet chemical exists. The helispot would be off limits to the public for the duration of the project and all equipment and chemicals would be secured at all times. All label restrictions would be followed, including the proper handling, storage, and mixing of the chemical as well as the use of personal protective equipment.

Indirectly, there are risks that result from the broadcast application of herbicide into the environment. Since visitor use restrictions would be in place to prevent direct exposure to visitors and personal protective equipment would be used to prevent direct exposure to workers, any chemical exposure would most likely be indirect. The most likely route for human exposure to the herbicide is via ingestion of contaminated food or drinking water or body contact with contaminated water used for swimming. The water resources and soils sections of this document already described the pathways by which chemical might enter water bodies, so these pathways are only briefly described in this section. The most likely scenario is for a small percentage of chemical bound to soil particles to be mobilized during summer monsoon rainfall events and washed into main stream channels. Because such events also greatly increase the volume of water in streams, any chemical would be diluted and would move quickly through the river system into larger and larger streams and bodies of water increasing the dilution and decreasing the concentration of the chemical. Imazapic that remains bound to soil would be deposited in with that soil overbank deposits and eventually the chemical would be lost through microbial degradation (BASF 2006). During flood events that mobilize treated soils into waterways, any imazapic that is not bound to soil would dissolve in water where it would rapidly be photo degraded by sunlight with a half-life in water of less than 8 hours (BASF 2006). As a result of these factors, it is highly unlikely that the chemical would enter drinking water supplies and, because swimming during floods in inherently unsafe and unpopular, it is unlikely that people would be exposed due to body contact. However unlikely there remains the potential for unforeseen direct or indirect exposure to the chemical. Imazapic attacks specific plant enzymes, so its effects on mammals are almost non-existent. Toxicological studies have found that acute oral, dermal, and inhalation toxicity values are Environmental Protection Acency toxicity category IV (lowest toxicity rating), it is non-irritating to the eye and skin and is not a skin sensitizer (BASF 2005, 2006). Under chronic exposure, imazapic is non-carcinogenic and non-mutagenic (BASF 2005, 2006). It does not bioaccumulate (BASF 2006), so there is no potential for human exposure due to consumption of animals that were exposed to the chemical or animals that ate other animals that were exposed to the chemical.

In studies, Methylated seed oils have been found to be minimal irritant to eyes and skin (Loveland 2004c-d). Liberate® has been to be found to be a nonirritant to eyes and a moderate irritant to skin (Loveland 2004a-b). Neither compound has been listed as having carcinogenic potential.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would prevent the escalation of fire frequency, fire size, and fire intensity. Studies have found that suppression of cheatgrass with Plateau® herbicide reduces flame height by as much as 88 percent and minimizes fire spread by as much as 95 percent (BASF 2003a). By restoring the natural fire regime, firefighters, nearby landowners, and the public would not be exposed to the hazards associated with increased fire frequency, fire size, and fire intensity.

Cumulative Effects. The proposed action as well as the proposed aerial seeding of roadsides and the East Tunnel construction project would involve use of helicopters or heavy equipment. To mitigate potential risks, temporary public use restrictions would be enacted to protect public health and safety and all worker safety procedures would be followed. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting public health and safety, would result in short-term, negligible cumulative negative impacts to public health and safety.

Conclusion. All short-term, negative impacts to public health and safety can be mitigated. There would be long-term, minor positive impacts to public health and safety due to restoration of the natural fire regime and reduced exposure of the public, park neighbors, and firefighters to hazards associated with fire and fire suppression activities.

- Visitor Use and Experience

Affected Environment

In 2005, over 2.6 million people visited ZION. Visitors participate in a wide range of activities, including lodging and camping (both within the park and in the gateway towns), hiking, canyoneering, rock climbing, attending ranger guided programs, and nature observation. An increasing number of visitors are using ZION’s backcountry — in 2004, 7,292 backcountry permits were issued. Overall, backcountry visitors seek varying degrees of solitude and visitors enjoy natural sounds during most of their experiences.

Several day and overnight use trails and routes are accessed from the Kolob Terrace Road including: The Subway canyoneering route, Right Fork of North Creek route, Northgate Peaks Trail, West Rim Trail, Wildcat Canyon Trail, and Hop Valley Trail. There is also a small campground at Lava Point. These areas are used heaviest from June through October. The Subway is the most popular route in the area and is regulated by a permit system. A maximum of 50 people per day can enter the canyon. In 2004 this capacity was met on 50 days – in mid-summer, on weekends, and holidays. Use in the area drops off as air temperatures cool. In 2005, there were 182 backcountry permits issued for 561 people to use the Subway Trail in October and 83 backcountry permits issued for 197 people to use the Subway Trail in November.

The Coalpits Wash area is used from November through May when air temperatures are cooler. This area is one of the least used areas in the park, with most use occurring during February and March. In 2004, 92 overnight permits were issued for the area. The Coalpits and Dalton Wash area get some day use, but the use is very small. In 2005, there were 2 backcountry permits issued for 3 people to use the Coalpits area in October and 9 backcountry permits issued for 30 people to use the Coalpits area in November.

Impact Threshold Definitions

| Negligible | Visitors would not be affected, or changes in visitor use and/or experience would be below or at the level of detection. The visitor would not likely be aware of the effects associated with the alternative. |
| --- | --- |
| Minor | Changes in visitor use and/or experience would be detectable, although the changes would be slight. The visitor would be aware of the effects associated with the alternative, but the effects would be slight. |
| Moderate | Changes in visitor use and/or experience would be readily apparent. The visitor would be aware of the effects associated with the alternative and would likely be able to express an opinion about the changes. |
| Major | Changes in visitor use and/or experience would be readily apparent and would have important consequences. The visitor would be aware of the effects associated with the alternative and would likely express a strong opinion about the changes. |
| Duration | Short-term - occurs only during the treatment effect |
|  | Long-term - occurs after the treatment effect |
| Area of Analysis | Within and immediately adjacent to the Kolob Fire burned area inside the park boundary |

Effects of Alternative A – No Action Alternative

Under this alternative, aerial herbicide application would not occur so there would be no temporary public use restriction in the project area and there would be no short-term impact to visitor experience.

However, without the aerial application of herbicide, the grass-fire cycle would continue uninterrupted. As described in the vegetation section of this document, non-native cheatgrass would eventually come to dominate the project area and spread into the surrounding areas, including the more remote interior areas of the park. Fueled by the cheatgrass, fire frequency, fire size, and fire intensity would increase. This would alter the landscape, which would probably be perceived negatively by most visitors as they would lose the opportunity to experience a natural setting, native plant communities, and native wildlife. Furthermore, the increased fire frequency, fire size, and fire intensity would require more frequent public use restrictions while fires are being suppressed and smoke generated by those fires would negatively impact the experience of visitors using other areas of the park or surrounding lands.

Cumulative Effects. Implementation of the no action alternative would mean that aerial herbicide application would not occur and the grass-fire cycle would be perpetuated, which would reduce the effectiveness of four other projects: native seed increase, aerial seeding of roadsides, exotic plant monitoring and control, and sensitive plant monitoring and spot control. The no action alternative would not have any relationship to other actions in the park, such as the boundary fence reconstruction and East Tunnel construction. As no action means there would be no helicopter or herbicide, there would be no associated public use closures and therefore no short-term, cumulative impacts to public health and safety. The increased flammability of the landscape would result in increased potential for public use closures or smoke impacts due to fire or fire suppression activities. This would result in long-term, minor cumulative negative impacts to visitor use and experience.

Conclusion. Implementation of the no action alternative would result in no short-term impacts to visitor use and experience, but would result in long-term, minor negative impacts to visitor use and experience due to public use closures associated with increased fires and fire suppression activities.

Effects of Alternative B – Proposed Action/Preferred Alternative

Under this alternative, a helicopter would be working in the project area for approximately 2 weeks to complete the herbicide application. There would be temporary visitor use restrictions in various sections of the project area to assure that there are no visitors where herbicide is actively being applied. Short-term, such restrictions would negatively impact the visitor experience of those people who are prevented from accessing the area. Furthermore, areas adjacent to the closures would still be open to visitor use but the sight and sound of the helicopter would be apparent to visitors using those areas and would negatively affect their experience.

Long-term, the herbicide treatment would interrupt the grass-fire cycle, which would allow native plant communities to regenerate and provide habitat for native wildlife. As the native plant communities and the native wildlife species are routinely cited as a primary attraction for park visitors, their perpetuation would serve to enhance the visitor experience. Furthermore, interruption of the grass-fire cycle would restore natural fire regimes, characterized by infrequent and small fires. This would reduce the negative effects on visitor experience caused by visitor use restrictions during fire suppression operations and smoke impacts to visitors in surrounding areas.

Cumulative Effects. The proposed action as well as the proposed aerial seeding of roadsides and the East Tunnel construction project would all temporarily restrict public use. While such restrictions would occur in different places and potentially on different days or weeks, the displacement of visitors from these project areas to other areas of the park and surrounding lands would alter visitor use patterns and may negatively affect visitor experience of some visitors. The temporary travel delays and increase in noise levels caused by these projects would also negatively impact visitor experience. Some of these impacts are lessened by the fact that the primary visitor destination, Zion Canyon, would not be directly affected by any of these projects and that visitation is relatively low during the late fall season when these projects would be undertaken. Overall, impacts of the actions described under the proposed action added to the impacts of other actions affecting visitor use and experience, would result in short-term, moderate cumulative negative impacts to visitor use and experience and long-term, minor cumulative positive impacts to visitor use and experience.

Conclusion. There would be short-term, minor negative impacts to visitor use due to public use closures and long-term, minor positive impacts to visitor experience due to perpetuation of native plant communities and wildlife that visitors can enjoy.

# CONSULTATION AND COORDINATION

## Public Involvement Summary

Public participation is an important part of any planning process. The purpose of the scoping process is to identify issues and concerns related to the project and to identify issues and alternatives to be addresses in the environmental assessment. In preparation for scoping, a mailing list of about 100 individuals was established. A scoping notice was prepared and mailed to those on the list on August 18, 2006. (Refer to Appendix 2) The scoping notice included a brief description of the proposed project and described opportunities for public participation. Information about the project and how to participate was posted on the park Web site. A press release was also issued by the park in August.

The park received three scoping comment letters. Two letters was in support of the proposed action. The other recommended the development and implementation of a process to evaluate the effectiveness and impacts of aerial Plateau® applications across a wide range of environmental settings. This recommendation is part of the proposed action.

Agency Consultation

National Historic Preservation Act – In accordance with the NHPA, letters requesting tribal consultation were mailed to the following tribes: Hopi Tribe, Kaibab Paiute Tribe, Moapa Band Paiute Tribe, Northern Ute Tribe, Paiute Indian Tribe of Utah, Shivwits Paiute Band, Las Vegas Paiute Tribe, White Mesa Ute, Navajo Tribe, Skull Valley Goshute Tribe, Goshute Indian Tribe, and Pueblo of Zuni. No comments were received.

State Historic Preservation Office – A letter requesting scoping comments was sent to the State Historic Preservation Office on August 18, 2006. No comments were received. A copy of this document will be sent to the SHPO for review and comment as part of the Section 106 process.

U.S. Fish and Wildlife Service – Park staff contacted the USFWS by telephone on August 7, 2006 to discuss the project proposal and to initiate consultation. A scoping letter was also sent on August 18, 2006. A copy of this document will be sent to the USFWS for review and comment.

- List of Preparers

| Name | Title | Project Role |
| --- | --- | --- |
| Jock Whitworth | Superintendent | Reviewer and Recommending Official |
| Kelly Fuhrmann | Fire Ecologist | Prepare Environmental Assessment |
| Jan Passek | Fire Management Officer | Reviewer |
| Cheryl Decker | Vegetation Program Manager | Prepare Environmental Assessment |
| Sarah Horton | Cultural Resource Program Manager | Reviewer |
| Juanita Bonnefield | Archeological Technician | BAR Plan Cultural Resource & Operations Resource Assessment |
| Jack Burns | Chief Resource Management & Research | Reviewer |
| Kezia Nielsen | Environmental Protection Specialist | Manage environmental compliance process |
| Dave Sharrow | Hydrologist | Prepare Environmental Assessment |
| Elena Robisch | GIS Specialist | GIS Support |
| Sandee Dingman | NPS Biologist, Lake Mead NRA | Prepare Environmental Assessment |
| Richard Gatewood | BAER Team Leader | BAR Plan |
| Kara Paintner | BAER Team Vegetation Specialist | BAR Plan Vegetation Assessment |
| Susanne Loadholt | BAER Team Soil Scientist | BAR Plan Soil Assessment |
| Michael Parenti | BAER Team Hydrologist | BAR Plan Watershed Assessment |
| Dan Hall | BAER Team Cultural Resource Specialist | BAR Plan Cultural Resource & Operations Resource Assessment |
| D.W. Ivans | BAER Team GIS Specialist | BAR Plan GIS |
| Cheryl Eckhardt | NEPA and Section 106 Specialist | Regional Office Reviewer |

## List of Agencies, Governments, Officials, and Organizations Contacted

Federal Agencies

Bureau of Land Management - St. George Field Office

National Park Service - Utah State Coordinator

U.S. Fish and Wildlife Service

U.S. Geological Survey

Natural Resources Conservation Service

State and Local Agencies and Governments

Five County Association of Governments

Mayor of Rockville, UT

Mayor of Springdale, UT

Mayor of Virgin, UT

Mayor of Hurricane, UT

Mayor of La Verkin, UT

Mayor of St, George, UT

Utah Division of Wildlife Resources

Utah Office of the Governor

Utah School and Institutional Trust Lands Administration

Utah State Clearinghouse

Utah State Historic Preservation Officer

Washington County Commissioners

Washington County Water Conservancy District

State and Federal Elected Officials

Congressman Jim Matheson

Senator Robert Bennett

Senator Orrin Hatch

Stephen Urquhart – Utah House of Representatives

David Clark – Utah House of Representatives

Bradley Last – Utah House of Representatives

Thomas Hatch – Utah State Senate

John Hickman – Utah State Senate

Indian Tribes

Goshute Indian Tribe

Hopi Tribe

Kaibab Paiute Tribe

Moapa Band Paiute Tribe

Navajo Tribe

Northern Ute Tribe

Paiute Indian Tribe of Utah

Pueblo of Zuni

Skull Valley Goshute

White Mesa Ute

Organizations

Grand Canyon Trust

Southern Utah Wilderness Alliance

Zion Canyon Chamber of Commerce

Virgin River Resource Management & Recovery Program

# REFERENCES

BASF. 2003a. A Burning Issue: Cheatgrass fuels rangeland fires.  12 pp.

BASF. 2003b. Cheatgrass: Non-native chatgrass cheats farmers and ranchers out of their land. Technical Bulletin. 8 pp.

BASF. 2004. Plateau®  Specimen Label. 16 pp.

BASF. 2005. Material Safety Data Sheet for Plateau®  Herbicide. 8 pp.

BASF. 2006. Plateau®  Herbicide: Toxicology, Ecotoxicology, and Environmental Fate Overview. 8 pp.

Barney, M.A., and N.C. Frischknecht. 1974. Vegetation Changes Following Fire in the Pinyon-Juniper Type of West Central Utah. Journal of Range Management. 17:91-96.

Belnap, Jayne, Julie H. Kaltenecker, Roger Rosentreter, John Williams, Steve Leonard and David Eldridge. 2001. Biological Soil Crusts: Ecology and Management. U.S. Bureau of Land Management, National Science and Technology Center, Technical Reference 1730-2, Denver, Colorado. 109 pp.

Brooks, M.L., C.M. D’Antonio, D.M. Richardson, J.B. Grace, J.E. Kelley, J.M. DiTomoso, R.J. Hobbs, M.Pellant, and D. Pyke. 2004. Effects of Invasive Alien Plants on Fire Regimes. Bioscience Vol. 54 No. 7, pp 677-688.

Brooks, M.L. 2005. Effectiveness of Postfire Seeding to Reduce Cheatgrass (Bromus tectorum) Growth and Reproduction in Recently Burned Sagebrush Steppe. Final Report for JFSP Project Number 01C-3-3-13. Delivered to the Joint Fire Science Program, National Interagency Fire Center, 3833 S. Development Ave., Boise, ID 83705-5354. 7 pages.

Bunting, S.C. 1987. Use of Prescribed Burning in Juniper and Pinyon Juniper Woodlands, p. 141-144. In: R.L. Everett (ed.). Proceedings-Pinyon-Juniper Conference. USDA Forest Service General Technical Report INT-215.

Campbell, R.B., Chappell, L., Greenhalgh, K. 2003. Personal communication, data from unpublished Fishlake National Forest Wildland Fire Use Plan. Table 16: Analysis by Vegetation Cover Type for the Combination of all Subsections for the Fishlake National Forest. Richfield, UT 84701 As Referenced by:  Merzenich, J. and L. Frid. 2005. Projecting Landscape Conditions in Southern Utah Using VDDT. In: M. Bevers and T.M. Barrett (comps.). 2005. Systems Analysis in Forest Resources: Proceedings of the 2003 Symposium; October 7-9, Stevenson, WA. General Technical Report PNW-GTR-000. Portland, OR: U.S. Department of Agriculture, Forest Service, Pacific Northwest Research Station. pp. 157-163.

Cogan, D., M. Reid, K. Schulz, and M. Pucherelli. 2004. Zion National Park Vegetation Mapping Project, Final Report and Map. Technical Memorandum 8260-03-01, Remote Sensing and GIS Group, Technical Service Center, Bureau of Reclamation, Denver, Colorado.

D’Antonio, C.M. and P.M. Vitousek. 1992. Biological Invasions by Exotic Grasses, the Grass/Fire Cycle, and Global Change. Annual Review of Ecology and Systematics 3: 63-87

Esque, T.C., S. VanderWall, C.R. Schwalbe, P.A. Medica, and R.H. Webb. 2004a. Effects of Fire on Small Animals and the Potential for Animals to Facilitate Restoration in Hot Desert Environments. Northwest Science Association Symposium. Session on: Fire and Hazardous Fuel Reduction. Ellensburg, WA.

Esque, T.C., L.A. DeFalco, S.J. Scoles, D.F. Haines, R.L. Kipp, and J.E. Rodgers. 2004b. Survivorship of Joshua Trees (Yucca brevifolia) in a Harsh Desert Environment: Effects of Fire, Drought, and Herbivory in Joshua Tree National Park. 16-18 November. Mojave Desert Science Symposium. University of Redlands, CA.

Everett, R. L. 1987. Proceedings-PinyonJuniper Conference. General Technical Report INT-215. U.S. Forest Service Intermountain Research Station, Ogden. UT, 581 pp.

Executive Office of the President, Council on Environmental Quality (CEQ). 1978. Regulations for implementing the procedural provisions of the National Environmental Quality Act. Code of Federal Regulations Title 40, Parts 1500-1508. Washington, D.C.

Executive Office of the President, CEQ. 1981. Forty most asked questions concerning CEQ’s National Environmental Quality Act regulations (40 CFR 1500-1508). Federal Register 46 (55):18026-18038.

Harris, G.A., and C.J. Goebel. 1976. Factors in Plant Competition in Seeding Pacific Northwest Ranges. Washington State University, Agricultural Experiment Station. Bulletin 820.

Jameson, D.A., J.A. Williams, and E.W. Wilton. 1962. Vegetation and Soils of Fishtail Mesa, Arizona. Ecology 43:403-410.

Hobbs, Chris and Micah Downing. 2003. The Natural Soundscape in Zion National Park, Wyle Report WR03-08. Wyle Acoustic Group, Wyle Laboratories, Arlington, Virginia.

Klemmedson, J. 0. and J. G. Smith. 1964. Cheatgrass (Bromus tectortum). Botanical Review 30:226-262.

Kury, B.K, J.D. Alexander, and J. Vollmer. 2002. Data Collection and Fire Modeling Determine Potential for the use of Plateau® Herbicide to Establish Fuel Breaks in Bromus tectorum-Dominated Rangelands.  2002 Association of Fire Ecology Annual Conference. San Diego, CA. December 3-5.

Louie, D.L., and M.L. Brooks, C. Deuser, and J. Passek. 2005. Evaluate Treatments to Reduce Hazardous Fine Fuels Created by Non-native Plants in Zion Canyon. A proposal for the Joint Fire Science Program, AFP 2005-2-Task 1 Project # 05-2-1-13.

Loveland Products. 2004a. Liberate® Specimen Label.

Loveland Products. 2004b. Liberate® Material Safety Data Sheet.

Loveland Products. 2004c. MSO® Concentrate Specimen Label.

Loveland Products. 2004d. MSO® Concentrate Material Safety Data Sheet.

Meyer, S.E. 2003. How Cheatgrass Won the West. Presentation at Wyoming Bureau of Land Management Cheatgrass Symposium.

Monaco, T.A., T.M. Osmond, and S.A. Dewey. 2005. Medusahead Control with Fall- and Spring-Applied Herbicides on Northern Utah Foothills. Weed Technology Volume 19:653–658

Mosley, J. C., S. C. Bunting, and M. E. Manoukian. 1999. Cheatgrass in R. L. Sheley and J. K. Petroff, eds. Biology and Management of Noxious Rangeland Weeds. Oregon State University Press, Corvallis.

Pellent, M. 2003. Lessons from the Great Basin. Presented at the Wyoming Cheatgrass Conference 2003.

United States Department of Agriculture (USDA). Soil Conservation Service. 1977. Soil Survey of Washington County Area, Utah. 94 pp.

United States Department of the Interior, Department of Agriculture, Department of Energy, Department of Defense, Department of Commerce, Environmental Protection Agency, Federal Emergency Management Agency, and National Association of State Foresters. 2001. Review and Update of the 1995 Federal Wildland Fire Management Policy.

USDI, NPS. 1996. Zion National Park Water Rights Settlement Agreement.

USDI, NPS. 2001a. Director’s Order-12 and Handbook, Conservation Planning, Environmental Impact Analysis, and Decision Making. Washington, D.C.

USDI, NPS. 2001b. Management Policies 2001. NPS D1416. Washington, D.C.

USDI, NPS.  2001c. Zion National Park. General Management Plan.

USDI, NPS. 2001d. Zion National Park. Peregrine falcon management and monitoring information.

USDI, NPS. 2002a. Director’s Order-77-1, Wetland Protection. Washington, D.C.

USDI, NPS. 2002b. Director’s Order-18 and Reference Manual, Wildland Fire Management.  Washington, D.C.

USDI, NPS, 2005. Wildland Fire Management Plan EA/FONSI. Zion National Park, Springdale, Utah.

USDI, NPS. 2006. Kolob Fire Burned Area Rehabilitation Plan. Zion National Park, Utah.

USDI, USFWS. 1995. Recovery Plan for the Mexican Spotted Owl: Vol. 1. Albuquerque, New Mexico.

USDI, USFWS. 2000. National Wetlands Inventory Maps. 20 USGS Quadrangles of enhanced NWI mapping of Zion National Park and vicinity.

USDI, USFWS. 2001a. Endangered and Threatened Wildlife and Plants; Determination of Endangered Status for Astragalus holmgreniorum (Holmgren milk-vetch) and Astragalus ampullarioides (Shivwits milk-vetch) Final Rule. Federal Register, Volume 66, No. 189, pages 49560-49567.

USDI, USFWS. 2001b. Final Designation of Critical Habitat for the Mexican spotted owl; 50 CFR Part 17, February 1, 2001.

USDI, USFW. 2005. Formal Section 7 Consultation for the Environmental Assessment for the Zion National Park Fire Management Plan. Utah Field Office US Fish and Wildlife Service, Ecological Services, West Valley City, Utah.

West, N.E., and W.C. Loope. 1977. Annual Report to Zion National Park on Studies of Fire Frequency in Backcountry Ecosystems of Zion National Park. Zion National Park, Utah.

Zouhar, Kris. 2003. Tamarix spp. In: Fire Effects Information System, [Online]. U.S. Department of Agriculture, Forest Service, Rocky Mountain Research Station, Fire Sciences Laboratory (Producer). Available: http://www.fs.fed.us/database/feis/ [2006, July 20].

GLOSSARY and ACRONYMS

Annual Plant – A plant growing from seed, producing flowers and seeds, and dying the same year.

Appropriative Water Rights – Water Rights administered by the state for the use of water by diverting it from the natural channel and applying it to state recognized beneficial uses (e.g. irrigation, domestic or municipal water supply).  Among their attributes are date of appropriation, point of diversion, place of use, amount and type of use.  (Zion has purchased appropriative water rights from private owners as the Federal Government acquired lands.)

Backcountry – Zion backcountry constitutes most of the undeveloped area of the park, where no roads or substantial human-made structures exist.  Much of Zion’s backcountry, however, does contain maintained trails.  Primary backcountry travel is by foot, and on specified trails, by horseback.  Camping is regulated in the backcountry:  in some areas camping is allowed nearly anywhere, while in other areas camping is only permitted in designated campsites.

Base Flow – River or spring flow that is low and steady, typically from the discharge of groundwater and not as a result of recent precipitation.

Biological Soil Crusts – Where the soil surface is bound together by a community of organisms that can include cyanobacteria, algae, fungi, mosses and lichens.  These create a soft crust at the soil surface that is resistant to raindrop impact, erosion from wind and water, and its roughness greatly increase the soil’s ability to capture and hold water.

Canyoneering – Hiking and rappelling through narrow canyons.  In Zion, permits are issued for canyoneering routes requiring the use of rappelling equipment.

Class I Area – Lands designated through the Clean Air Act, including National Parks and Wilderness, that are given the highest protection of existing air quality (prevention of significant deterioration), and where visibility and other air quality related values are protected.

Cool Season Plant Species – A plant that makes most of its growth during winter and spring and sets seed in late spring or early summer.

Cumulative Impacts – The impacts of cumulative actions - includes impacts of actions in the past, the present, and the reasonable foreseeable future.

Desired Future Conditions – The goals or end results park managers are striving to achieve.  Desired conditions can be set for park resources, visitor experiences, management activities, and facilities.  Desired conditions reflect the park's purpose and mission goals, and ensure that Zion's resources are conserved and quality experiences are provided.

Direct Effect – An impact that occurs as a result of the proposed action or alternative in the same place and at the same time as the action.

Ecosystem Sustainability – Perpetuation of the biological, cultural, and physical processes such that dependent resources are maintained in high condition within a natural range of variability.

Environmental Assessment – Environmental assessments were authorized by the National Environmental Policy Act (NEPA) of 1969. They are concise, analytical documents prepared with public participation that determine if an Environmental Impact Statement is needed for a particular project or action. If an environmental assessment determines an environmental impact statement is not needed, the environmental impact statement becomes the document allowing agency compliance with NEPA requirements.

Environmental Impact Statement – Environmental impact statements were authorized by the National Environmental Policy Act (NEPA) of 1969. Prepared with public participation, they assist decision makers by providing information, analysis and an array of action alternatives, allowing managers to see the probable effects of decisions on the environment. Generally, environmental impact statements are written for large-scale actions or geographical areas.

Extirpated – When a species no longer exists in the wild in a certain place, but exists elsewhere.

Federal Reserved Water Rights – Water Rights associated with lands by virtue of their withdrawal by the Congress of the United States for Specific Purposes.  The use and amount of water is that necessary to meat the purposes of the reservation, which may differ from state appropriative uses, and the date of appropriation is the date of withdrawal.

Fire Frequency – The return interval or recurrence interval of fire in a given area over a specific time.

Fire Intensity – A general term relating to the heat energy released by a fire.

Fire Management Plan (FMP) – A strategic plan that defines a program to manage wildland and prescribed fires and documents the Fire Management Program in the approved land use plan.  The plan is supplemented by operational plans such as preparedness plans, preplanned dispatch plans, prescribed fire plans and prevention plans.

Fire Regime – The combination of fire frequency, predictability, intensity, seasonality and size characteristics of fire in a particular ecosystem.

Fire-Return Interval – The number of years between two successive fire events at a specific site or an area of a specified size.

Floodplain – Part of a river channel that is inundated only during time of high flow.  A 100-year floodplain is the area inundated by a flood that has a 1% chance of occurring in any given year, or occurs on average once every 100 years. Floods of this magnitude occur frequently enough to pose a serious threat to facilities and people.

Fuel – Combustible material. Includes, vegetation, such as grass, leaves, ground litter, plants, shrubs, and trees that feed a fire.

Fuel Type – An identifiable association of fuel elements of a distinctive plant species, form, size, arrangement, or other characteristics that will cause a predictable rate of fire spread or difficulty of control under specified weather conditions.

Grass-Fire Cycle – An alteration of fire regime that may occur where nonnative invasive grass species dominate the herbaceous layer in a plant community. The process occurs in this way: The nonnative grass colonizes an area and provides a continuous fine fuel that is readily ignited and facilitates fire spread. Larger and perhaps more intense fires then occur more frequently in the invaded area than in similar, uninvaded communities. Following these grass-fueled fires, nonnative grasses typically recover more rapidly than native species, further increasing the probability of fire and the possibility of greater fire size, greater fire intensity, and decline of native species.

Herbicide – Any chemical substance used to control plant growth.

Impact Topics – Specific natural, cultural, or socioeconomic resources that would be affected by the proposed action or alternatives (including no action).  The magnitude, duration, and timing of the effect to each of these resources are evaluated in the impact section of the environmental assessment.

Indirect Impact – Reasonably foreseeable impacts that occur removed in time or space from the proposed action.  These “downstream” impacts, future impacts, or the impacts of reasonably expected connected actions (e.g., growth in an area after a highway is completed).

Issue(s) – In NEPA, issues are environmental, social, and economic problems or effects that may occur of the proposed action or alternatives (including no action) are implemented or continue to be implemented.

Invetebrate – Any animal without a backbone or spinal column; any animal other than a fish, amphibian, reptile, bird, or mammal.

Litter – Top layer of the forest, scrubland, or grassland floor, directly above the fermentation layer composed of loose debris of dead sticks, branches, twigs, and recently fallen leaves or needles, little altered in structure by decomposition.

Management Zones – In the Visitor Experience and Resource Protection Plan, identify how different areas in the park will be managed to achieve a combination of desired conditions.  Each zone represents a unique combination of physical, biological, social, and managerial conditions.

Mutagenic – Any agent or substance capable of noticeably increasing the frequency of mutations.

National Environmental Policy Act (NEPA) – NEPA is the basic national law for protection of the environment, passed by Congress in 1969. It sets policy and procedures for environmental protection, and authorizes Environmental Impact Statements and Environmental Assessments to be used as analytical tools to help federal managers make decisions.

Natural Sound and Soundscape – Any sounds produced by nature, such as the wind in the trees, songs of birds, flow of water in rivers and streams, etc.  Unnatural sound would include any sounds produced by people or their devices, such as human voices, vehicles, and motorized tools.

Non-native Plant – A plant that is not native to the area, exotic.

Perennial Plant – Plants living more than two years.

Rehabilitation – The activities necessary to repair damage or disturbance caused by wildland fires or the fire suppression activity.

Riparian – Adjacent to, or living on, the bank of a river, or sometimes a lake or pond.

Scoping – Internal NPS decision-making on issues, alternatives, mitigation measures, the analysis boundary, appropriate level of documentation, lead and cooperating agency roles, available references and guidance, defining purpose and need, and so forth.  External scoping is the early involvement of the interested and affected public.

Slickrock – Flat areas or, more commonly, slopes with large exposures of bare rock.  This is typically on exposures of Navajo sandstone in Zion.

Teratogenic – An agent, as a chemical, disease, etc., that causes malformation of a fetus.

Warm Season Plant Species – A plant that makes of its growth during spring and summer and sets seed in late summer or early fall. It is normally dormant in winter.

Watershed – The area drained by a river or river system.

Wildland – Any natural landscape not maintained for buildings, road, fence or other human development.

BAER		Burned Area Emergency Response

BAR		Burned Area Rehabilitation

BLM		Bureau of Land Management

CEQ		Council on Environmental Quality

CFR		Code of Federal Regulations

cfs		Cubic Feet per Second

dBA		A-Weighted Decibels

DFC		Desired Future Condition

DO		Director’s Order

EA		Environmental Assessment

ESA		Endangered Species Act

FMP		Fire Management Plan

GMP		General Management Plan

NEPA		National Environmental Policy Act

NHPA		National Historic Preservation Act

NPS		National Park Service

NRHP		National Register of Historic Places

SHPO		State Historic Preservation Officer

TSD		Total Dissolved Solids

USC		United States Code

USDI		U.S. Department of the Interior

USFWS	U.S. Fish and Wildlife Service