##### National Park Service

U.S. Department of the Interior

Glacier National Park

Montana

Many Glacier/Swiftcurrent Sewage System and Wastewater Treatment Improvements

Environmental Assessment

July 2007

Many Glacier Hotel

## Environmental Assessment

_____________________________________________________________________________

Many Glacier Sewage System and Wastewater Treatment Improvements

Glacier National Park • Montana

A portion of Waterton-Glacier International Peace Park

Summary

Glacier National Park is considering making improvements to the sewage system and wastewater treatment facilities for the Swiftcurrent and Many Glacier developed areas. The facilities have deteriorated consistent with age and exposure to the relatively harsh environmental conditions. Both the sewage lines and treatment systems are over 30 years old and are showing signs of deterioration. In addition, the Many Glacier wastewater treatment plant (WWTP) does not currently have a Montana Department of Environmental Quality (MDEQ) discharge permit for surface water or groundwater. The park proposes to replace or rehabilitate parts of the sewage and treatment system to meet the standards for a permit.

Two alternatives are being analyzed: a No Action Alternative which would maintain the current sewage system conditions and a Preferred Alternative. The Preferred Alternative is to: replace or re-line the sewer pipes coming from the Swiftcurrent developed area and the force mains from both the Swiftcurrent and Many Glacier Districts, upgrade the Swiftcurrent lift station, and make several improvements to the control building, headworks, lagoon, percolation ponds, and fencing at the wastewater treatment plant.

The impact topics determined to potentially be affected by the alternatives are Archeological Resources, Soils, Wildlife, Special Status Species, Water Quality, and Park Operations. The No Action Alternative would have minor to moderate long-term site-specific adverse impacts to Archeological Resources and minor long-term or short-term local adverse impacts to Water Quality. Effects on Special Status Species would be minor to moderate, short-term or long-term, local adverse with the No Action Alternative. Impacts to all other topics under the No Action Alternative would be minor long-term local adverse. The Preferred Alternative would have negligible long-term site-specific adverse impacts to Archeological Resources, moderate long-term local beneficial impacts to water quality, and moderate long-term local beneficial impacts to park operations. Impacts to Soils would be minor short-term local adverse and wildlife would have moderate short-term local adverse impacts. Special Status Species would be impacted as follows: gray wolf and bald eagle – negligible short-term local adverse; grizzly bear, Canada lynx, and Species of Concern – minor short-term local adverse; bull trout – negligible long-term local beneficial; and no effect to listed plant species.

How to Comment

Comments can be provided directly through the Park’s planning website (http://parkplanning.nps.gov/parkHome.cfm?parkId=61) by selecting this project. Or write to: Superintendent, Glacier National Park, Attn: Many Glacier Wastewater EA, West Glacier, Montana 59936. This environmental assessment (EA) will be on public review for 30 days. Before including your address, phone number, e-mail address, or other personal identifying information in your comment, you should be aware that your entire comment – including your personal identifying information – may be made publicly available at any time. While you can ask us in your comment to withhold your personal identifying information from public review, we cannot guarantee that we will be able to do so. We will always make submissions from organizations or businesses, and from individuals identifying themselves as representatives of or officials of organizations or businesses, available for public inspection in their entirety.

Table of Contents

Summary	i

How to Comment	ii

Introduction	1

Background	1

Purpose and Need	2

Public Involvement (Scoping)	4

Relationship of the Proposed Action to Previous Planning Efforts	4

Impact Topics	5

Topics Selected for Detailed Study	5

Topics Eliminated from Detailed Study	6

Alternatives Considered	10

No Action Alternative	10

Preferred Alternative	10

Alternatives Considered But Eliminated	13

Environmentally Preferred Alternative	18

Affected Environment and Environmental Consequences	19

Consultation and Coordination	45

Agencies/Tribes/Organizations/Individuals Contacted (EA Recipients)	45

Preparers and Consultants	46

Summary of Compliance with Federal and State Regulations	46

References	47

# Introduction

Background

Glacier National Park (Glacier or the park) is located on the Canadian border in the northwestern section of Montana. The park is in the northern Rockies, and contains the rugged mountains of the Continental Divide. Together with Canada’s Waterton National Park, it forms the Waterton-Glacier International Peace Park, and is an international Biosphere Reserve and a World Heritage Site. Outstanding natural and cultural resources are found in both parks.

Glacier National Park is an investment in the heritage of America. Its primary mission is the preservation of natural and cultural resources, ensuring that current and future generations have the opportunity to experience, enjoy, and understand the legacy of Waterton-Glacier International Peace Park.

The purpose of Glacier National Park is to:

- preserve and protect natural and cultural resources unimpaired for future generations (1916 Organic Act);

- provide opportunities to experience, understand, appreciate, and enjoy Glacier National Park consistent with the preservation of resources in a state of nature (1910 legislation establishing Glacier National Park); and

- celebrate the on-going peace, friendship, and goodwill among nations, recognizing the need for cooperation in a world of shared resources (1932 International Peace Park legislation).

Glacier’s significance is explained relative to its natural and cultural heritage:

- Glacier’s scenery dramatically illustrates an exceptionally long geological history and the many geological processes associated with mountain building and glaciation;

- Glacier offers relatively-accessible, spectacular scenery and an increasingly rare primitive wilderness experience;

- Glacier is at the core of the “Crown of the Continent” ecosystem, one of the most ecologically intact areas remaining in the temperate regions of the world;

- Glacier’s cultural resources chronicle the history of human activities (prehistoric people, Native Americans, early explorers, railroad development, and modern use and visitation) and show that people have long placed high value on the area’s natural features; and

- Waterton-Glacier is the world’s first international peace park.

The proposed project would include sewage system improvements within the Many Glacier subdistrict of the park (Figure 1). Sewage systems would be inspected and repaired from the Swiftcurrent Historic Districts to the wastewater treatment facility located approximately two miles east of the Many Glacier Hotel. These utilities are within the visitor service zone as identified in the park’s General Management Plan (NPS 1999a).

Figure 1. Location of project within Glacier National Park.

Purpose and Need

Glacier National Park is considering making improvements to the sewage system and wastewater treatment facilities for the Swiftcurrent and Many Glacier developed areas. In general, the Many Glacier wastewater treatment plant (WWTP) is providing adequate treatment of the wastewater according to fecal contamination levels measured in waters around the WWTP in 2005 that were found to be below EPA recreational standards (Gannon et al. 2006). However, the facilities have deteriorated consistent with age and exposure to the relatively harsh environmental conditions. Both the sewage lines and treatment systems are over 30 years old and are showing signs of deterioration. In addition, the Many Glacier WWTP does not currently have a Montana Department of Environmental Quality (MDEQ) discharge permit for groundwater. MDEQ has indicated that a groundwater discharge permit for the WWTP is required for continued operation. The discharge requirements that will be defined in the permit will be based on the water quality standards that MDEQ has adopted (Kennedy Jenks Consultants 2006). The park proposes to replace or rehabilitate parts of the sewage and treatment system to meet the standards for a permit, as described below.

The sewer system includes two lines, one starting at Swiftcurrent and the other in Many Glacier, that merge and flow to the WWTP. The Many Glacier sewage lines, including all pipes running to the main line, were repaired or replaced during rehabilitation of the hotel between 2002 and 2004. According to a recent engineering report, the Swiftcurrent section has experienced recent blockages and some pipes are too narrow to meet MDEQ standards. Additional testing and inspection of these segments and the main line to the WWTP are required to determine how much of the system would need replacement.

The dry well area which contains the pumps and valves at the Swiftcurrent lift station is considered a confined space. This requires that all WWTP operators obtain confined space training and a refresher course be conducted every 12 months. The categorization also requires at least two operators that are confined space trained be at the facility when repairs are made. The WWTP is already short-staffed and having two people at one location is inefficient and often difficult to schedule. Major modifications to the lift station are recommended to enhance safety and efficiency for operators by eliminating the confined space designation.

The Many Glacier wastewater treatment facility was built in 1975 and is located approximately two miles east of the Many Glacier Hotel. The facility is operated from May to October and consists of the headworks, an aerated lagoon, and percolation ponds. There are deficiencies with the aeration system within the lagoon and the original lagoon liner has been damaged several times by bears that have accessed the lagoon. The lagoon has not been emptied of accumulated solids since its construction, reducing the capacity and efficiency of the lagoon. In addition, the lagoon’s recirculation system is inoperable due to pump failure. The percolation ponds are currently not fenced, they pose a potential human health risk and some people have attempted to fish in them.

There are three problems with the existing fencing that surrounds the WWTP and lagoon: 1) there are several sections that require repair to prevent bears from entering the area, 2) bears can also enter the fenced area by climbing trees adjacent to the fence and climbing over the top of the fence, and 3) its location at the bottom of the berm prevents Facilities Management personnel from mowing the slope of a berm. Mowing, conducted once every summer, is needed to prevent trees and shrubs from becoming established and eventually compromising the berm.

In summary, the following objectives would be met with this project:

- Replace aging and deteriorating sewage pipes.

- Make changes to sewage system to meet the standards for a MDEQ groundwater discharge permit.

- Modify the Swiftcurrent lift station to enhance safety and efficiency for operators by eliminating the confined space designation.

- Repair deficiencies with the aeration system and provide a quiescent zone prior to the discharge point.

- Replace the original lagoon liner that has been damaged several times.

- Empty the lagoon of accumulated solids to increase capacity and efficiency of the lagoon.

- Replace inoperable recirculation system pump at lagoon.

- Fence the percolation ponds to reduce a potential human health risk.

- Replace and move the existing fence around the lagoon to prevent bears from entering and allow operators better access for mowing the surrounding berm.

Public Involvement (Scoping)

Scoping is an early and open process to determine which environmental issues and alternatives should be addressed in an environmental assessment. Glacier National Park conducted both internal scoping with appropriate National Park Service (NPS) staff and external scoping with the public and interested and affected groups and agencies.

The interdisciplinary process of internal scoping defined the purpose and need for the project, identified potential alternatives to address these needs, determined what the issues were and what resources would be affected and identified the relationship, if any, of the proposed action to other planning efforts at the park.

Public scoping began with letters announcing the project and a press release issued on June 30, 2006. Letters were sent to the park’s mailing list for EAs, which included various federal, state, and local agencies, such as the U.S. Fish and Wildlife Service (USFWS) and the Blackfeet Tribal Business Council, and the Confederated Salish and Kootenai Tribes. The announcement was also posted on the National Park Service’s public comment website.

In accordance with 36CFR800.8(c), Glacier National Park also notified the Montana State Historic Preservation Officer and the Advisory Council on Historic Preservation that it intended to prepare a combined Environmental Assessment/Finding of Effect for the proposed project.

Six comments were received during scoping for this project. The Alberta Minister of Environment had no concerns with the project. Both the Blackfeet Tribal Historic Preservation Office and the Salish and Kootenai Tribal Preservation Office requested further consultation on the project. The Advisory Council on Historic Preservation (ACHP) had no comments but recommended that we continue to consult with the Montana State Historic Preservation Office and local tribes. One respondent requested additional information on the location and type of upgrades to be done. These are presented in this EA. A sixth respondent thought the project was worthwhile.

Relationship of the Proposed Action to Previous Planning Efforts

The proposed action is consistent with the objectives of Glacier National Park’s General Management Plan (GMP)/Environmental Impact Statement and Record of Decision (1999a). According to the GMP and the park’s Commercial Services Plan (NPS 2004), the Many Glacier developed area is not expected to expand. However, housing and lodging at the Swiftcurrent developed area may be expanded as recommended in the Commercial Services Plan. All improvements to the sewage and wastewater treatment system would be capable of handling increases to the system if needed. The wastewater treatment system has changed little since its installation in the 1970s. The Many Glacier sewage lines, including all pipes running to the main line, were repaired or replaced during rehabilitation of the hotel between 2002 and 2004. Underground sewer lines at Swiftcurrent have primarily been replaced on an as-needed basis. Rehabilitation projects at the Many Glacier Hotel have occurred during the spring and fall from 2001-2006 and are scheduled to continue for another 4-5 years if funding is secured. The Hotel work was initially approved as part of the GMP and a Biological Assessment and consultation with the US Fish and Wildlife Service was completed in 2002 for later phases of the work.

# Impact Topics

Resources that may be affected by the project were identified by National Park Service staff, other federal and state agencies, and the public. Impact topics were derived from these resources. The following impact topics were identified on the basis of federal laws, regulations, orders, and National Park Service Management Policies (2006), and input received during scoping. Four impact topics were identified for analysis in this document. A brief rationale for the selection of these impact topics is given below, as well as the rationale for dismissing other potential impact topics from further consideration.

Topics Selected for Detailed Study

### Archeological Resources

The proposed changes would require a considerable amount of digging with the potential to disturb archeological resources. Therefore, impacts to this topic are analyzed.

### Soils

The proposed sewer upgrades would require substantial trenching and improvements could reduce the possibility of soil contamination. Therefore, impacts to this topic are analyzed.

### Wildlife

The work would be conducted during the park’s “shoulder seasons”, from late March to mid-May and again from September to late December. The Many Glacier/Swiftcurrent areas are usually closed at these times with very little human activity and wildlife can use the area with little human disturbance. The proposed work could disrupt usual wildlife patterns in the area during these periods.

### Special Status (Federal and State Listed) Species

Several federally-listed species use the area during the proposed construction times. Wildlife are more prevalent in the developed areas outside of the main visitor season when much of the proposed work would occur. The endangered gray wolf and the threatened bald eagle, grizzly bear, and Canada lynx are known to live in the Many Glacier area. Rehabilitation construction occurring at the Many Glacier Hotel has occurred from 2002-2008 and this project would essentially extend that seasonal work an additional 4-5 years and include outside construction in a larger area.

### Water Quality

The proposed changes address potential problems with pollution of adjacent water sources. Therefore, impacts to this topic are analyzed.

### Park Operations

Changes that may require a new level of maintenance or staffing, or that may affect existing facilities are considered under this topic. The proposed changes are designed to improve facilities and reduce the level of maintenance required at the site. Therefore, this topic is analyzed for impacts.

Topics Eliminated from Detailed Study

NEPA, Council on Environmental Quality (CEQ) regulations, and NPS procedures for implementing NEPA specify that an EA should address only those resource areas potentially subject to impacts. In addition, the level of analysis should be commensurate with the anticipated level of environmental impact. The following resources were considered but eliminated from further analysis.

### Historic Buildings and Districts

Though some ground disturbance may occur within the boundaries of the National Register of Historic Places listed Swiftcurrent (Many Glacier) Ranger Station Historic District (24GL0154); the Swiftcurrent Auto Camp Historic District (24GL0884); the Many Glacier Hotel Historic District (24GL0864); and near the Many Glacier Barn and Bunkhouse (24GL0880), no permanent visible changes would result. Most work would be conducted outside of the visitor season though minor construction could occur during the summer within the historic districts and be apparent to visitors. The proposed utility improvements would benefit the historic properties by preventing sewage leaks that could damage the historic settings and sewage backups into buildings. Impacts to historic buildings or districts would be minor or less in scope and this topic was dismissed from further analysis.

### Cultural Landscapes

Some ground disturbance may occur within the boundaries of the National Register of Historic Places listed Swiftcurrent (Many Glacier) Ranger Station Historic District (24GL0154); the Swiftcurrent Auto Camp Historic District (24GL0884); the Many Glacier Hotel Historic District (24GL0864); and near the Many Glacier Barn and Bunkhouse (24GL0880). Some cultural landscape features have been identified in the Many Glacier Hotel Historic District, but no associated landscapes have been evaluated or documented for the other historic properties. No permanent visible changes would result from the proposed project. Most work would be conducted outside of the visitor season though minor construction could occur during the summer within cultural landscapes. The proposed utility improvements would benefit cultural landscapes by preventing sewage leaks that could damage the setting of the District. Impacts to cultural landscapes would be minor or less in scope and this topic was dismissed from further analysis.

### Ethnographic Resources

No ethnographic resources were identified by the Confederated Salish and Kootenai Tribes Historic Preservation Department or the Blackfeet Tribal Business Council in the Many Glacier area during scoping for this project. However, Glacier National Park recognizes that the tribes hold a body of knowledge that may result in the identification of ethnographic resources in the area in the future. If ethnographic resources are identified, consultation will occur in accordance with federal legislation and regulations and National Park Service policy. This topic was dismissed from further analysis.

### Museum Collections

No Glacier National Park museum collection items would be disturbed as a result of this project. Therefore, museum collections were dismissed from further analysis.

### Vegetation

The National Park Service strives to maintain all components and processes of naturally evolving park unit ecosystems, including the natural abundance, diversity, and ecological integrity of plants (NPS 2006). Most trenching would occur within previously disturbed corridors where there are no shrubs or trees. Some native forbs could be disturbed; however, topsoil would be conserved and replaced over the disturbed area upon completion. The seeds for the annual vegetation in these areas would remain in the soil and vegetation should quickly regenerate. The disturbed areas would be monitored for weeds for several years after the work is completed to ensure weeds do not become established because of the disturbance. Impacts to native vegetation from the proposed project would be no more than minor and this topic has been dismissed from further discussion.

### Wetlands

For regulatory purposes under the Clean Water Act, the term wetlands means "those areas that are inundated or saturated by surface or ground water at a frequency and duration sufficient to support, and that under normal circumstances do support, a prevalence of vegetation typically adapted for life in saturated soil conditions. Wetlands generally include swamps, marshes, bogs, and similar areas."

Executive Order 11990 Protection of Wetlands requires federal agencies to avoid, where possible, adversely impacting wetlands. Further, Section 404 of the Clean Water Act authorizes the U.S. Army Corps of Engineers to prohibit or regulate, through a permitting process, discharge or dredged or fill material or excavation within waters of the United States. National Park Service policies for wetlands as stated in 2006 Management Policies and Director’s Order 77-1 Wetlands Protection, strive to prevent the loss or degradation of wetlands and to preserve and enhance the natural and beneficial values of wetlands. In accordance with DO 77-1 Wetlands Protection, proposed actions that have the potential to adversely impact wetlands must be addressed in a Statement of Findings for wetlands. There are wetlands in the vicinity of the WWTP; though, they are not within the area of disturbance for the proposed actions. No adverse impacts to wetlands are expected and replacing the deteriorating sewage pipes would reduce the possibility of sewage leaking from broken pipes into nearby wetlands. Therefore, a Statement of Findings for wetlands will not be prepared, and the impact topic of wetlands has been dismissed.

### Floodplains

Executive Order 11988 Floodplain Management requires all federal agencies to avoid construction within the 100-year floodplain unless no other practicable alternative exists. The National Park Service under 2006 Management Policies and Director’s Order 77-2 Floodplain Management will strive to preserve floodplain values and minimize hazardous floodplain conditions. Floodplains for the developed areas were estimated in 2002 for development of the Commercial Services Plan (NPS 2004). It was estimated that the Many Glacier Hotel lies within the 100-year floodplain of Swiftcurrent Lake and portions of the Swiftcurrent Campground likely lie within the 100-year floodplain of Wilbur Creek. Floodplains around the WWTP have not been mapped and portions may be located within the floodplain of Lake Sherburne or Swiftcurrent Creek and its smaller tributaries.

According to Director’s Order 77-2 Floodplain Management, certain construction within a 100-year floodplain requires preparation of a Statement of Findings for floodplains; however, the Director’s Order “does not apply to historic or archaeological structures, sites, or artifacts whose location is integral to their significance.” Ground disturbance would be temporary and would not affect water flows. Since the proposed project would be conducted to benefit and support two historic districts and does not involve building a new structure within the floodplain, a Statement of Findings for floodplains will not be prepared. The topic of floodplains has been dismissed.

### Prime and Unique Farmlands

In 1980, the Council on Environmental Quality directed that Federal Agencies must assess the effects of their actions on farmland soils classified by the U.S. Department of Agriculture’s Natural Resources Conservation Service as prime or unique. There are no “prime or unique farmlands” in Glacier National Park (NPS 1999a); therefore it was dismissed as an impact topic.

### Recommended Wilderness

All parts of the sewage system and wastewater treatment plant are within the visitor service zone of the park which is not included within recommended wilderness. Noise from equipment such as a backhoe used for trenching could be heard within adjacent wilderness areas. This noise would be minor and temporary and is not expected to exceed the amount of man-made noise present in the area during the visitor season. However, during the shoulder seasons there is usually little man-made noise in the area. The area gets little recreational use by visitors at this time and the amount of wilderness that would be impacted by the temporary noise would be minor. Therefore, wilderness is not discussed further as a topic.

### Air Quality

The Clean Air Act establishes specific programs that provide special protection for air resources and air quality related values associated with National Park Service units. Section 118 of the Clean Air Act requires a park unit to meet all federal, state, and local air pollution standards. Glacier National Park is classified as a mandatory Class I area under the Clean Air Act, where emissions of particulate matter and sulfur dioxide are to be restricted. The act gives the federal land manager the responsibility for protecting air quality and related values (i.e., including visibility, vegetation, wildlife, soils, water quality, cultural resources, recreational resources, and public health) in Class I lands from adverse air pollution impacts; and to consider, in consultation with EPA, whether proposed industrial facilities will have an adverse impact on these values. Federal land managers are also required to determine whether existing industrial sources of air pollution must be retrofitted to reduce impacts on Class I areas.

Air quality is considered good in Glacier National Park. There are no major metropolitan areas within 125 miles of the park, and no regional smog typical of highly populated areas with a high amount of vehicle traffic. Exhaust emitted by a backhoe or other mechanical equipment used during this project would be negligible and temporary. The Class I air quality designation would not be affected by the proposal. Therefore, air quality has been dismissed as an impact topic.

### Visual Resources

The improvements, with the exception of a new fence, would be installed underground or within existing structures and would only be visible to visitors during installation. The existing chain link fence at the treatment plant would be removed from the bottom of the surrounding berm and a new dark green chain link fence would be installed, possibly along the top of the berm. This green fence would blend in with the background better than the original silver fence, though it may be placed in a more visible location. A wood split rail fence or other aesthetically appealing boundary system would be installed adjacent to the road in front of the percolation ponds to discourage use of the area by the public. The impact on visual resources would be minor or less in degree and this topic was dismissed from further consideration.

### Natural Soundscapes

In accordance with 2006 Management Policies (NPS 2006) and Director’s Order 47 Sound Preservation and Noise Management, an important component of the National Park Service’s mission is the preservation of natural soundscapes associated with national park units. Natural soundscapes exist in the absence of human-caused sound. The natural ambient soundscape is the aggregate of all the natural sounds that occur in park units, together with the physical capacity for transmitting natural sounds. The frequencies, magnitudes, and durations of human-caused sound considered acceptable varies among National Park Service units as well as potentially throughout each park unit, being generally greater in developed areas and less in undeveloped areas.

Noise from equipment such as a backhoe used for trenching would be minor and temporary and is not expected to exceed the amount of man-made noise present in the area during the visitor season. However, during the shoulder seasons when the construction is scheduled there is usually little man-made noise in the area. Sounds generated from construction would be temporary and would add a minor adverse impact on current noise levels. Noise impacts to wildlife are discussed under the wildlife impact topic. Therefore, the topic of soundscape management was dismissed as an impact topic.

### Visitor Experience

Much of the work would be conducted before and after the visitor season because it would require that the sewage system be shut down at both Many Glacier and Swiftcurrent. Some minor work could be conducted during summer months but would not affect visitors.  All activities would occur within previously disturbed areas. Some temporary disturbance would be visible to the public but would be minor and have little effect on visitor experience. Therefore, this topic was dismissed from further analysis.

### Health and Safety

Health and safety issues associated with the sewage system include the potential for water contamination and employee safety. Repairing and replacing deteriorating water lines could reduce the chances of water contamination and proposed changes to the Swiftcurrent lift station would remove the “confined space” designation that requires employees to have special training. These two issues are analyzed under the topics of Water Quality and Park Operations; therefore, impacts to health and safety were dismissed as an impact topic.

#### Environmental Justice

Executive Order 12898 “General Actions to Address Environmental Justice in Minority Populations and Low-Income Populations,” requires all federal agencies to identify and address disproportionately high and adverse human health or environmental effects of their programs and policies on minorities and low-income populations or communities. The proposed action would not have disproportionate health or environmental effects on minorities or low-income populations or communities as defined in the Environmental Protection Agency's Environmental Justice Guidance because no one lives in the area when construction would occur and their would be no impacts outside of the immediate area. Therefore, Environmental Justice was dismissed as an impact topic in this document.

#### Socioeconomics

The proposed action would neither change local and regional land use nor appreciably impact local businesses or other agencies. The construction work would be contracted to a private company. Some local businesses may receive a small increase in revenue from contractors working at the site; however, any increase in revenue would be temporary and negligible. Therefore, this topic has been dismissed.

# Alternatives Considered

No Action Alternative

The No Action Alternative describes the conditions that would continue to exist if the proposed action is not implemented. This alternative provides a baseline for evaluating the changes and related environmental impacts that would occur under the Proposed Action (Table 1).

Under this alternative, improvements to the entire sewage system and wastewater treatment facilities in the Many Glacier subdistrict would not be implemented. Individual repairs to sewage lines and the wastewater treatment plant would occur on an as needed basis. Digging would occur only in locations where repairs were immediately needed. The lagoon liner would continue to be damaged by bears. The public would continue to easily access the percolation ponds. The treatment system would eventually be closed if it could not obtain a discharge permit from MDEQ.

Preferred Alternative

This alternative presents the National Park Service’s preferred alternative and is the proposed undertaking for Section 106 compliance. Under this alternative, the following improvements to the sewage system (Figure 3) and wastewater treatment facility (figures 3 and 4) would be made:

Gravity Sewers: The sewer pipes coming from the Swiftcurrent developed area, including the Swiftcurrent Ranger Station (Many Glacier) Historic District and Swiftcurrent Auto Camp Historic District, are old clay pipes that require replacement. Pipe replacement would be conducted using a pipe bursting method. This would require digging, with a backhoe, a 5 foot long trench, approximately 3 feet wide at each manhole. The old pipe is then burst by slipping a new pipe within it. This method reduces the amount of trenching required as the old and new piping remains in the ground. Approximately 2150 feet of gravity lines would be replaced. Gravity lines coming from the Many Glacier developed area (Many Glacier Hotel Historic District) were replaced during renovations made in 2002-2004 and no additional improvements are required at this time.

Force Mains: The force mains running from both the Swiftcurrent and Many Glacier areas to the WWTP would be inspected at the lift stations. In addition, several 3-foot sections of pipe would be removed along the line as representative samples for corrosion analysis. Depending on the outcome of this analysis, the force main lines may require replacement. The force main is installed under the road bed of Glacier Route Three (i.e. the main Swiftcurrent/Many Glacier road). If only small sections require replacement, the work would be done by pipe bursting. If the entire main line (approximately 9350 feet) requires replacement, or a large section, it would be done by open trenching along its length.

Lift Stations: At the Swiftcurrent lift station a vacuum-primed pump package system would be installed. This system eliminates the confined space designation, provides better access for maintaining equipment, and provides standardized pumping equipment throughout the park.

Wastewater Treatment Plant: Approximately 60 percent of the area within the fenced compound (Figure 4) of the WWTP would be disturbed. The exact amount would depend on the condition of pipes and equipment. The following improvements would be made to the wastewater treatment system:

Headworks:

- The initial sewage intake filtering unit in the western channel of the headworks would be replaced with an in-line grinding unit;

In Lagoon:

- Accumulated solids would be removed from the lagoon;

- The lagoon liner would be replaced;

- The aeration diffuser system within the lagoon would be replaced and the capacity of the aeration system (including existing blowers) would be evaluated to provide aeration capacity for additional nitrification;

- A floating baffle would be provided in the lagoon to create a quiescent zone for settling that would have a hydraulic detention time of one to two days;

- The recirculation pump system in the lagoon would be replaced to provide additional flexibility and process reliability;

In Control Building:

- The existing aeration control system would be replaced;

- Effluent flow measurement equipment and an electronic data recorder would be installed;

- The effluent flow control system would be replaced;

- The roof of the control building would be resealed or replaced;

- The concrete sidewalks around the control building would be replaced;

- The existing steel access hatch at the control building would be replaced with a lighter material such as an aluminum hydraulic assisted hatch;

At Percolation Ponds:

- Percolation Pond No. 3 and Pond No. 4 would be cleaned, by dredging;

- The drain pipes through the berms of pond nos. 1, 3, and 4 would be removed or plugged;

- Leaking areas of pond levees would be located and repaired;

- A cover with hatch would be installed over the pond distribution box;

Fencing:

- The existing chain link fence around the WWTP and lagoon would be removed from the bottom of the lagoon berm and a new chain link fence with green vinyl would be installed in the best location for maintenance and to discourage bear entry; this may be along the top of the berm. Two solar-powered electrical strands would be added to the fence to further discourage bears from entering;

- A wood split rail fence or other aesthetically appealing boundary system would be installed adjacent to the road in front of the percolation ponds to discourage use of the area by the public.

Because this work would require that the sewage system be shut down at both Many Glacier and Swiftcurrent, the work would be conducted before and after the visitor season. Weather permitting, the primary work would occur from late March to mid-May and again from September to late December. Some minor work (e.g. fencing, concrete flatwork, and control building renovation) that does not interrupt the flow or treatment of wastewater or visitor access could be conducted during summer months. The work would be conducted in phases starting in 2008 and take approximately 4-5 years. The park has currently budgeted $1.5 million dollars for these improvements.

### Mitigation Measures during Construction of the Proposed Action

The following mitigation measures would be implemented.

- All construction would be limited to a designated area required to complete the work; all activity, including vehicle and material use and storage would not be allowed outside predetermined, marked construction/staging zones.

- Construction zones would be identified and fenced with construction tape or similar material prior to any construction activity. The fencing would define the construction zone and confine activity to the minimum area required for construction.

- No materials would be moved off site or out of the Park during this project.

- All earth-moving equipment including haul vehicles would be thoroughly cleaned of mud and weed seed prior to entering the National Park.

- If fuels and hazardous materials are used, a spill-protection plan must be approved by the Park Safety Officer.

- Soil erosion would be minimized by appropriate erosion control measures. Only certified weed-seed free erosion control materials (e.g. straw or coir bales) will be permitted as erosion control or siltation devices to reduce probability of weed contamination. In addition, coconut silt rolls and fabric silt fences are allowed depending upon site constraints.

- Excavated soil may be used in the construction project; excess soil would be stored in approved areas.

- Topsoil would be removed and conserved separately then placed back on top after the work is completed.

- Where trenching has occurred, the surface of the trench will be left adequately mounded to allow for ground settling along the line. Park inspection of all fill, gravel or soil materials into Glacier would be required.

- Disturbed areas would be promptly revegetated with native species to reduce erosion potential and hasten recovery of soils. In accordance with Glacier’s genetic guidelines, only seeds and plants originating from the park will allowed in restoration activities. All revegetation efforts would be in accordance with the park’s Road Maintenance Guidelines (NPS 2005), including those for safety and visibility of signs.

- If during construction previously undiscovered archeological resources are discovered, all work in the immediate vicinity of the discovery would be halted until the resources could be identified and documented and an appropriate mitigation strategy developed in consultation with the State Historic Preservation Officer.

- Initial ground-disturbing activities would be monitored by an archaeologist or para-archaeologist. A decision to continue monitoring would be made based upon examination of soils.

- All work would be conducted during daylight hours and workers would commute to and from the site each day and be confined to the day’s work area.

- If the golden eagle nest on Mt. Altyn is active, construction within the 800-m buffer area (Figure 2) would be limited to one hour after sunrise and one hour before sunset between April 1 and August 15. If work is conducted during March and April near Swiftcurrent Creek at the WWTP, restrictions on time of day (i.e. no work until one hour after sunrise or one hour before sunset) would be necessary to allow eagles to forage in these areas.

- The presence of a park law enforcement ranger and biological technician would ensure compliance with park regulations and enforcement of garbage and food-storage regulations to minimize the risk of habituation or mortality of wildlife. If garbage cans are needed, certified bear-proof garbage containers will be required.

Alternatives Considered But Eliminated

To avoid disturbing wildlife in the shoulder season, doing the work during the summer season was considered but rejected. This would not be possible without closing most of the services provided at Many Glacier and Swiftcurrent including the hotels and campground. Traffic delays on the road would also occur because of the amount of trenching that is required. Closing the facilities for an entire summer and traffic delays in conjunction with work on the GTSR resulted in this option being eliminated from further consideration.

Figure 2. Historic Districts and area of concern for eagles. If the golden eagle nest is active, construction within the 800-m buffer area would be limited to one hour after sunrise and one hour before sunset between April 1 and August 15.

Figure 3. Locations of sewage lines and treatment plant in relation to Many Glacier and Swiftcurrent developed areas.

Figure 4. Wastewater treatment plant layout.

Table 1. Summary comparison of alternatives analyzed in this document.

| Objective | No Action Alternative | ##### Preferred Alternative |
| --- | --- | --- |
| Replace aging and deteriorating sewage pipes | Sewer lines would continue to fail and be replaced on an as needed basis. | All sewer lines older than 10 years would be replaced and/or tested to determine their level of deterioration. |
| Make changes to sewage system to meet the standards for a MDEQ groundwater discharge permit | Some of the sewer lines are, and would remain, too small to meet MDEQ standards. | All sewer lines that do not meet MDEQ standards would be replaced and upgraded to meet standards. |
| Modify the Swiftcurrent lift station to eliminate the confined space designation | The Swiftcurrent lift station would remain a confined space and require two employees be present when working at the site. | A new a vacuum-primed pump package system would be installed. This system eliminates the confined space designation, provides better access for maintaining equipment, and provides standardized pumping equipment installed throughout the park. |
| Repair deficiencies with aeration system at WWTP and provide a quiescent zone prior to the discharge point | Only 2 of the 12 static tube aerators are currently working and there is no quiescent zone prior to the discharge point. | New equipment would solve these deficiencies including a new filtering system, floating aeration diffuser system, and floating baffle divider. |
| Replace the original lagoon liner that has been damaged several times | The current liner would not be replaced and may leak into surrounding soils. | The liner would be removed and replaced with a new liner. |
| The lagoon has not been emptied of accumulated solids | The lagoon would not be emptied of accumulated solids. | The lagoon would be emptied of accumulated solids. |
| Replace inoperable recirculation system pump at lagoon | The lagoon recirculation system is no longer functioning due to pump failure. It would not be replaced. | The recirculation pump system in the lagoon would be replaced to provide additional flexibility and process reliability. |
| Fence the percolation ponds to reduce a potential human health risk | No new fencing would be installed around the percolation ponds. | A wood split rail fence or other aesthetically appealing boundary system would be installed adjacent to the road in front of the percolation ponds to discourage use of the area by the public. |
| Replace and move the existing fence around the lagoon | The current fence would remain in place. Because of its location it can be crossed by bears climbing trees adjacent to the fence and its location at the bottom of the berm prevents Facilities Management personnel from mowing the slope of the berm. | A new chain link fence with green vinyl would be installed, possibly on the top of the berm, where bears could not breach it and where the slope of the berm would be accessible for mowing. |
| Does the alternative meet project objectives? | No | Yes |

Table 2. Summary comparison of effects on resources under the No Action and Preferred alternatives. Some resources may not be changed at all from current conditions and these topics have consequently been given a rating of “no effect”.

| Impact Topic | No-Action Alternative | Preferred Alternative |
| --- | --- | --- |
| Archeological Resources | Minor to moderate long-term site-specific adverse due to the potential for excavation in previously unexcavated areas | Negligible long-term site-specific adverse due to a low chance of encountering artifacts |
| Soils | Minor to moderate long-term local adverse due to the potential for leaks | Minor short-term local adverse due to temporary disturbance from trenching |
| Wildlife | Minor long-term local adverse due to the continued access of the lagoon to bears | Moderate short-term local adverse due to noise and human presence during the work |
| Special Status Species | Minor to moderate, short-term or long-term, local adverse due to the continued access of the lagoon to bears and the potential for water contamination that could affect aquatic habitats | Gray wolf: negligible short-term local adverse due to noise and human presence Grizzly bear: minor short-term local adverse due to noise and human presence Canada lynx: minor short-term local adverse due to noise and human presence Bald eagle: negligible short-term local adverse due to noise and human presence Bull trout: negligible long-term local beneficial due to reduction in potential for water contamination Plants: no effect Species of Concern: minor short-term local adverse due to noise and human presence |
| Water Quality | Minor long-term and short-term local adverse due to continued pipe deterioration and potential for leaks | Moderate long-term local beneficial reduction in potential for leaks in pipes |
| Park Operations | Minor long-term local adverse due to the continued use of a “confined space” | Moderate long-term local beneficial due to the elimination of the “confined space” and easier access to lagoon berm |

Environmentally Preferred Alternative

The environmentally preferred alternative is determined by applying the criteria suggested in the National Environmental Policy Act of 1969 (NEPA), which is guided by the Council on Environmental Quality (CEQ). The CEQ provides direction that the “environmentally preferable alternative is the alternative that will promote the national environmental policy as expressed in NEPA Section 101”:

- fulfill the responsibilities of each generation as trustee of the environment for succeeding generations;

- assure for all generations safe, healthful, productive, and esthetically and culturally pleasing surroundings;

- attain the widest range of beneficial uses of the environment without degradation, risk of health or safety, or other undesirable and unintended consequences;

- preserve important historic, cultural and natural aspects of our national heritage and maintain, wherever possible, an environment that supports diversity and variety of individual choice;

- achieve a balance between population and resource use that will permit high standards of living and a wide sharing of life’s amenities; and

- enhance the quality of renewable resources and approach the maximum attainable recycling of depletable resources.

The Preferred Alternative fulfills all six criteria better than the No Action Alternative. The Preferred Alternative accomplishes criteria 1, 2, and 3 by eliminating sewage pollution, or the threat of pollution, into the soils, groundwater, and nearby streams. Criteria 2 and 4 are fulfilled by providing long-term maintenance solutions and by protecting the historic districts. The proposed improvements to the sewer system would provide a system better equipped to handle the volume of sewage produced in the area, thus fulfilling criteria 5. The No Action Alternative does not fulfill any of the criteria because the chance of sewage leaks into the surroundings and sewage backups into historic buildings would remain. Neither alternative addresses criteria 6. Therefore, the Preferred Alternative is the environmentally preferred alternative.

# Affected Environment and Environmental Consequences

Methodology

The effects of each alternative are assessed for direct, indirect, and cumulative effects on selected impact topics. Actions are first analyzed for their direct and indirect effects. Direct effects are impacts that are caused by the alternatives at the same time and in the same place as the action. Indirect effects are impacts caused by the alternatives that occur later in time or are farther in distance than the action. For example, construction grading may result in the direct removal of vegetation and soil from a site and result indirectly in increased erosion at the site at a later time when it rains, and to water quality off-site. Effects to historic properties listed in or eligible for listing in the National Register of Historic Places also have been described in accordance with Section 106 of the National Historic Preservation Act of 1966, as amended, and its implementing regulations, 36 CFR 800.

Potential impacts are described in terms of type, spatial context, duration, and intensity.

- Type: impacts are either beneficial or adverse. A resource may be affected both beneficially and adversely (e.g., one wildlife species may benefit while another is harmed), however an overall impact for the resource as a whole is determined.

- Spatial Context: impacts are 1) site-specific at the location of the action, 2) local on a drainage- or district-wide level, 3) widespread throughout the park, or 4) regional outside of the park.

- Duration: impacts are short-term or long-term. The definitions for these time periods depend upon the impact topic and are described in Table 4.

- Intensity: the impacts are negligible, minor, moderate, or major. Definitions of intensity vary by impact topic and are provided in Table 4.

Impairment of Park Resources or Values

National Park Service Management Policies (NPS 2006) require analysis of potential effects to determine whether or not actions would impair park resources or values. The fundamental purpose of the National Park System, established by the Organic Act and reaffirmed by the General Authorities Act, as amended, begins with a mandate to conserve park resources and values. NPS managers must always seek ways to avoid, or to minimize to the greatest degree practicable, actions that would adversely affect park resources and values.

These laws give the National Park Service the management discretion to allow impacts to park resources and values when necessary and appropriate to fulfill the purposes of a park, as long as the impact does not constitute impairment of the affected resources and values. Although Congress has given the National Park Service the management discretion to allow certain impacts within parks, that discretion is limited by the statutory requirement that the National Park Service must leave park resources and values unimpaired, unless a particular law directly and specifically provides otherwise.

The prohibited impairment is an impact that, in the professional judgment of the responsible NPS manager, would harm the integrity of park resources or values, including the opportunities that otherwise would be present for the enjoyment of those resources or values. An impact to any park resource or value may constitute an impairment. Impairment may result from NPS activities in managing the park, from visitor activities, or from activities undertaken by concessionaires, contractors, and others operating in the park. An impact would be more likely to constitute an impairment to the extent that it has a major or severe adverse effect upon a resource or value whose conservation is:

- Necessary to fulfill specific purposes identified in the establishing legislation or proclamation of the park;

- Key to the natural or cultural integrity of the park or to opportunities for enjoyment of the park; or

- Identified as a goal in the park’s general management plan or other relevant NPS planning documents.

Each alternative was analyzed to determine if impacts constituted an impairment to park resources and values.

Cumulative Impacts

The Council on Environmental Quality (CEQ) regulations, which implement the National Environmental Policy Act of 1969 (42 USC 4321 et seq.), require assessment of cumulative impacts in the decision-making process for federal projects. Cumulative impacts are defined as "the impact on the environment which results from the incremental impact of the action when added to other past, present, and reasonably foreseeable future actions regardless of what agency (federal or non-federal) or person undertakes such other actions" (40 CFR 1508.7). Cumulative impacts are considered for both the no-action and preferred alternatives.

Cumulative impacts were determined by combining the impacts of the alternative with other past, present, and reasonably foreseeable future actions. Therefore, it was necessary to identify other ongoing or reasonably foreseeable future projects at Glacier National Park and, if applicable, the surrounding region. The following are past, present and reasonably foreseeable future actions that have and could occur in the vicinity of the project area:

- Many Glacier Housing and Hotel rehabilitation (2001-2006; if funding is secured, 4-5 more years of work needed to complete rehabilitation; occurring during the same “shoulder season” periods as the proposed WWTP work)

- Many Glacier bunkhouse repairs (fall 2007 or spring 2008)

- Installation of limited fencing along eastern park boundary (scheduled for summer 2007)

- New activities proposed in the 2004 Final Commercial Services Plan and Environmental Impact Statement (NPS 2004) including: upgrading facilities for accessibility, parking modifications, extending the operating season two weeks, constructing a new employee dormitory and recreational facilities near the Many Glacier Hotel, converting an existing employee dorm to guest accommodations, returning employee rooms in the hotel to guest rooms, and new cabin-style guest accommodations and employee housing at Swiftcurrent (no future date set for these actions).

Impacts to Cultural Resources and Section 106 of the National Historic Preservation Act

In this environmental assessment/assessment of effect, impacts to cultural resources are described in terms of type, context, duration, and intensity, which is consistent with the regulations of the Council on Environmental Quality (CEQ) that implement the National Environmental Policy Act (NEPA). These impact analyses are intended to comply with the requirements of NEPA, but not fully with Section 106 of the National Historic Preservation Act (NHPA). This Environmental Assessment will serve the public notification requirements of the NHPA, but consultation with the State Historic Preservation Office and the Tribal Historic Preservation Officers to reach a finding of effect would be carried out independent of this EA.

In accordance with the Advisory Council on Historic Preservation’s regulations implementing Section 106 of the NHPA (36 CFR Part 800, Protection of Historic Properties), impacts to historic properties were identified and evaluated by (1) determining the area of potential effects; (2) identifying cultural resources present in the area of potential effects that were either listed in or eligible to be listed in the National Register of Historic Places; (3) applying the criteria of adverse effect to affected cultural resources either listed in or eligible to be listed in the National Register; and (4) considering ways to avoid, minimize or mitigate adverse effects.

Under the Advisory Council’s regulations a determination of either adverse effect or no adverse effect must also be made for affected National Register eligible cultural resources. An adverse effect occurs whenever an impact alters, directly or indirectly, any characteristic of a cultural resource that qualify it for inclusion in the National Register (e.g. diminishing the integrity of the resource’s location, design, setting, materials, workmanship, feeling, or association). Adverse effects also include reasonably foreseeable effects caused by the preferred alternative that would occur later in time, be farther removed in distance or be cumulative (36 CFR Part 800.5, Assessment of Adverse Effects). A determination of no adverse effect means there is an effect, but the effect would not diminish in any way the characteristics of the cultural resource that qualify it for inclusion in the National Register.

CEQ regulations and the National Park Service’s Conservation Planning, Environmental Impact Analysis and Decision-making (Director’s Order #12) also call for a discussion of the appropriateness of mitigation, as well as an analysis of how effective the mitigation would be in reducing the intensity of a potential impact, e.g. reducing the intensity of an impact from major to moderate or minor. Any resultant reduction in intensity of impact due to mitigation, however, is an estimate of the effectiveness of mitigation under NEPA only. It does not suggest that the level of effect as defined by Section 106 is similarly reduced. Although adverse effects under Section 106 may be mitigated, the effect remains adverse.

A Section 106 summary is included in the impact analysis sections under the preferred alternative. The Section 106 Summary is not intended to meet the requirements of Section 106 and is an assessment of the effect of the undertaking (implementation of the alternative) on cultural resources, based upon currently available information.

Table 3. Impact thresholds for intensity and duration.

| Impact Topic | Negligible | Minor | Moderate | Major | Duration |
| --- | --- | --- | --- | --- | --- |
| Archeological Resources | Impact is at the lowest levels of detection, barely measurable, with no perceptible consequences, either adverse or beneficial, to archeological resources. For purposes of Section 106, the determination of effect would be no adverse effect. | Disturbance of a site(s) is confined to a small area with little, if any, loss of important information potential. For purposes of Section 106, the determination of effect would be no adverse effect. | Disturbance of the site(s) would not result in a substantial loss of important information. For purposes of Section 106, the determination of effect would be adverse effect. | Disturbance of the site(s) is substantial and results in the loss of most or all of the site and its potential to yield important information. For purposes of Section 106, the determination of effect would be adverse effect. | Short-term: Any disturbance of an archeological resource would be permanent, thus there are no short-term impacts. Long-term: Any disturbance of an archeological resource would be permanent, thus all impacts are long-term. |
| Soils | Soil productivity or soil fertility would not be affected or the effect would be below or at the lower end of detection. Any effects to soil productivity or soil fertility would be slight and not measurable. | The effects to soil productivity or soil fertility would be detectable, but small. The area affected would be local. | The effect to soil productivity or soil fertility would be readily apparent. Effects would result in a change in soils over a relatively wide area or multiple locations. | The effect on soil productivity or soil fertility would be readily apparent and would substantially change the character of soils over a large area. | Short-term: After implementation, would recover in less than 3 years. Long-term: After implementation, would take more than 3 years to recover or effects would be permanent. |
| Wildlife | Effects would be at or below the level of detection and the changes would be so slight that they would not be of any measurable or perceptible consequence to the wildlife species’ population. | Effects on wildlife would be detectable, although the effects would be local and would be small and of little consequence to the species’ population. | Effects on wildlife would be readily detectable and widespread, with consequences at the population level. | Effects on wildlife would be obvious and would have substantial consequences to wildlife populations in the region. | Short-term: After implementation, would recover in less than 1 year. Long-term: After implementation, would take more than 1 year to recover or effects would be permanent. |
| Special Status Species | The alternative would affect an individual of a listed species or its critical habitat, but he change would be so small that it would not be of any measurable or perceptible consequence to the protected individual or its population. Negligible effect would equate with a “no effect” determination in U.S. Fish and Wildlife Service terms. | An individual(s) of a listed species or its critical habitat would be affected, but the change would be small. Minor effect would equate with a “may affect, not likely to adversely affect” determination for the species in U.S. Fish and Wildlife Service terms and would require informal consultation. | An individual or population of a listed species, or its critical habitat would be noticeably affected. The effect could have some long-term consequence to individuals, populations, or habitat. Moderate effect would equate with a “may affect” determination in U.S. Fish and Wildlife Service terms and would be accompanied by a statement of “likely…” or “not likely to adversely affect” the species and would require either informal or formal consultation. | An individual or population of a listed species, or its critical habitat, would be noticeably affected with a vital consequence to the individual, population, or habitat. Major effect would equate with a “may affect, likely to adversely affect” or “not likely to adversely affect” determination in U.S. Fish and Wildlife Service terms and would require formal consultation. | Short-termAfter 4-5 years of construction, would return to area in less than 1 year. Long-term After 4-5 years of construction, would take more than 1 year to return or effects would be permanent. |
| Water Quality | Water quality would not be affected, or changes would be either non-detectable or if detected, would have effects that would be considered slight and local. | Changes in water quality would be measurable, although the changes would be small and the effects would be local. | Changes in water quality would be measurable and would be noticeable on a widespread scale. | Changes in water quality would be readily measurable, would have substantial consequences, and would be noticed on a regional scale. | Short-term: Effects last less than 1 year. Long-term: Effects last more than 1 year. |
| Park Operations | Park operations would not be affected, or the effects would not be noticeable. | The effect would be detectable, but would not have an appreciable effect on park operations. | The effects would be readily apparent, and would result in a substantial change in park operations in a manner noticeable to staff and the public. | The effects would be readily apparent, would result in a substantial change in park operation in a manner noticeable to staff and the public, and be markedly different from existing operations. | Short-term - Effects lasting for the duration of the project Long-term - Effects lasting longer than the duration of the project. |

### Archeological Resources

There have been two archeological surveys in the area of potential effect for the proposed project. Mark R. Guthrie (NPS 1978) surveyed the Swiftcurrent and Many Glacier developed areas, and western portion of Apikuni Flat, including the site of the WWTP. Brian Reeves, Ph.D. carried out archeological inventory in the Swiftcurrent Valley, including the entire area of potential effect for the proposed project. Results of the survey are being withheld from public disclosure in this EA because of the risk of harm to archeological sites. However, the information has been shared with the Blackfeet Tribal Historic Preservation Officer and the Confederated Salish and Kootenai Tribal Historic Preservation Department during consultation.

No Action Alternative: This alternative might impact archeological resources. The sewer system would continue to deteriorate and require periodic maintenance. There is the potential for damage to unidentified archeological resources if excavation to repair failed sewer lines extended into unexcavated areas.  Although the chance of work impacting sites appears slight based upon currently available information, if it should occur, the impact of not upgrading the utilities would range from minor long-term site-specific adverse impacts to moderate long-term site-specific adverse impacts.

Cumulative Effects: Recent rehabilitation work at the Many Glacier Hotel did not impact archeological resources. New construction proposed under the Commercial Services Plan is not within the area of identified archeological sites, and would be preceded by an archeological survey and, if required, monitoring during ground disturbing activities. The cumulative impact on archeological sites from recent and foreseeable future projects combined with the no action alternative would be the same as described above and range from minor long-term site-specific adverse impacts to major long-term site-specific adverse impacts.

Conclusion: The result of the No Action Alternative would range from minor long-term site-specific adverse impacts to major long-term site-specific adverse if unidentified archeological resources were damaged during maintenance activities. Cumulative impacts would be the same.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

Preferred Alternative:  The findings of previous archeological surveys document the potential of disturbing artifacts in the area of potential effect in undisturbed ground. However, as planned the excavation work proposed under this alternative would occur in ground previously disturbed, both horizontally and vertically, by past utility construction activities. Consequently, archeological resources are not expected to be encountered during digging for this project.

The park’s cultural resource specialist consulted with the Blackfeet Tribal Historic Preservation Officer and the Confederated Salish and Kootenai Tribal Historic Preservation Department regarding this project. It was agreed that an archeologist would resurvey the areas proposed for disturbance and would monitor initial ground-disturbing activities. A decision to continue monitoring would be made based upon examination of soils. Should artifacts be identified during the project, work would cease in the immediate area and the park’s cultural resource specialist would be notified. Additional consultation in accordance with 36 CFR 800.13 for post-review discoveries would occur. Based upon current knowledge, the impacts to archeological resources from the Preferred Alternative would be negligible long-term site-specific adverse.

Section 106:  In accordance with 36 CFR 800.5(b), Glacier National Park has reached a preliminary finding of no historic properties (archeological) affected for the preferred alternative.

Cumulative Effects: Recent rehabilitation work at the Many Glacier Hotel did not impact archeological resources. New construction proposed under the Commercial Services Plan is not within the area of identified archeological sites, and would be preceded by an archeological survey and, if required, monitoring during ground disturbing activities. The cumulative impact on archeological sites from recent and foreseeable future projects combined with the preferred alternative would be the same as described above, negligible long-term site-specific adverse.

Conclusion: The impacts to archeological resources from the Preferred Alternative would be negligible long-term site-specific adverse. The cumulative impacts would be the same.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Soils

“The National Park Service will actively seek to understand and preserve the soil resources of parks, and to prevent, to the extent possible, the unnatural erosion, physical removal, or contamination of the soil or its contamination of other resources” (NPS 2006). The soils in the area are mainly rocky and sandy alluvial soils of fans and high terraces (map type A2). They are dominated by loam or sandy loam textures with abundant, well-rounded gravels and cobbles. These soils have relatively high moisture content and nutrient holding capacity in the surface layers. They also have moderate erosion potential and moderate to high revegetation potential. When disturbed, susceptibility to weed infestation is high, especially on exposed subsoils. Rock types are mostly quartzite and argillite with occasional limestone or granitic rock fragments. The soil around the WWTP is mapped as alluvial grassland soils which are rated as having moderate potential for waste disposal due to rapid permeability and relatively poor filtration where distance to ground water or surface water is small. However, recent testing by Gannon et. al (2006) indicates there has been minimal contamination (well below EPA recreational standards) of surface waters with the current system. The soils near the Many Glacier Hotel are mapped as limestone outcrops and shallow soils which have low revegetation and moderate erosion potential when disturbed (Dutton et al. 2001).

Impact Analysis – Soils

No Action Alternative: No immediate impacts to soils are expected under this alternative. Repairs to the sewage system would be made when leaks or breaks are detected and soils would be disturbed during those repairs. However, the soils surrounding the current system were previously disturbed when the system was installed. Soils could become contaminated if the old pipes break, especially if a sewage leak goes undetected for a long period of time. Impacts to soils would be minor to moderate long-term local adverse depending on the size and duration of a leak.

Cumulative Effects: The hotel rehabilitation had little impact on soils in the area, and those soils that were disturbed were revegetated. Boundary fencing, if constructed, would also have little impact on soils. The CSP EIS concluded that approximately 2 acres of previously undisturbed soil would be disturbed by new activities and the impacts would be minor. These projects in conjunction with the No Action Alternative would result in minor long-term site-specific impacts on soils in the area, and this alternative would account for very little of that impact.

Conclusion: The No Action Alternative would result in minor long-term local adverse impacts due mostly to the potential for sewage contamination. Cumulative effects under this alternative would be negligible long-term site-specific adverse.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

Preferred Alternative: If the full length of gravity and main line sewers were replaced via trenching, then a total of 0.8 acres of soils would be disturbed. Additional disturbance would occur in the area around the WWTP. The total disturbance would not exceed one acre and digging would occur in previously disturbed areas. Topsoil would be removed and conserved separately then placed back on top after the work is completed. In addition, disturbed areas would be revegetated to reduce erosion potential and hasten recovery of soils. Soil fertility and productivity are not expected to be diminished in the long-term. Improvements to the sewage system would reduce the possibility of contamination to soils in the area. Impacts to soils would be minor short-term local adverse.

Cumulative Effects: The hotel rehabilitation and proposed boundary fencing had/would have little impact on soils. Actions proposed in the CSP EIS are expected to impact approximately 2 acres of previously undisturbed soil and this impact was considered minor. The Preferred Alternative would add less than an acre more of soil disturbance; however this would occur in previously disturbed locations. Therefore, the Preferred Alternative would add little to the overall impacts on soils which would be minor short-term local adverse.

Conclusion: The Preferred Alternative would result in minor short-term local adverse impacts that are expected to recover quickly due to topsoil conservation and revegetation efforts. Cumulative effects under the Preferred Alternative would be minor short-term local adverse.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Wildlife

The National Park Service strives to preserve and restore the natural abundances, diversities, dynamics, distributions, habitats, and behaviors of native animal populations and the communities and ecosystems in which they occur (NPS 2006). The Many Glacier area is an important crossroads for wildlife because it lies at the confluence of three valleys and contains a wide variety of habitats. Numerous avalanche chutes and shrubfields provide important grizzly and black bear habitat in spring, summer, and fall. Highly productive riparian woodlands, sedge meadows, and other wetlands are important habitats for many other species including fisher, marten, mink, beaver, small mammals, bats, songbirds, raptors, and amphibians. Considerable portions of the drainage lie in the alpine zone and contain steep talus fields and cliff bands. These areas provide habitat for mountain goats and cliff-nesting raptors such as golden eagles and prairie falcons. Isolated, forested mountain ridges provide secure habitat for large herds of elk throughout the spring, summer, and fall.

The area provides year-round habitat for all of Glacier’s ungulate species including moose, white-tailed deer, mule deer, bighorn sheep, elk, and mountain goats. The drainage contains critical winter and spring range for bighorn sheep as well as important lambing and rutting grounds. Several bighorn sheep migration corridors that have probably been in continual use for over 4,000 years traverse the drainage providing connectivity among seasonally important habitats. Most of the park’s large carnivores including grizzly bears, Canada lynx, wolverine, and gray wolves are also found here, partly because of the healthy ungulate populations.

Many wildlife species avoid the developed areas of the Many Glacier area during the summer when visitors and employees are present. However, during the late fall, winter, and early spring the remoteness and relative lack of human activity encourages shy species like lynx, marten, fisher, and wolverine to use all available habitats including those found within the developed areas. The availability of denning habitat, the presence of diverse and healthy ungulate populations, and the inaccessibility to humans of much of the terrain (especially in winter and late spring) make the Many Glacier drainage highly suitable wolverine habitat. Family groups of both lynx and wolverine have been documented by extensive research in the drainage in recent years. Grizzly bears, bighorn sheep, lynx, wolverine, elk, and moose are known to use the wildlife corridors in and around the Swiftcurrent developed area. Golden eagles nest on cliffs near the Swiftcurrent developed area, and northern goshawks have been documented in the area.

The Many Glacier Valley floor is narrow and contains several large lakes. There is north-south movement of many species of wildlife in the limited forest habitat between the lakes, including the Swiftcurrent developed area. Meadows in the area, including Apikuni Flats, provide late fall, winter and spring habitat for elk. Bald eagles frequent the several lakes in the drainage, and there is a known nest location at nearby Sherburne Reservoir. The Sherburne Dam, constructed in 1919, inundated several small lakes, reaches of Swiftcurrent Creek and highly productive riparian/wetland areas. Today, the area surrounding Sherburne Reservoir supports little vegetation because of the fluctuating water levels and is marginally important wildlife habitat. The Many Glacier/Swiftcurrent drainage historically had no fish, but kokanee salmon and cutthroat trout were introduced beginning around 1912.

Impact Analysis – Wildlife

No Action Alternative: The No Action Alternative would not change current conditions for wildlife. Repairs to the sewage system and WWTP would be done on an as needed basis, and some noise and disturbance would be created during repairs. Because of the current location and condition of the fence surrounding the WWTP and lagoon, bears would continue to occasionally access the area inside of the fence, possibly leading to confrontations with employees. This could also result in a habituated bear that may eventually need to be removed from the population. The impact to wildlife from the No Action Alternative would be minor long-term local adverse.

Cumulative Effects: Rehabilitation work at the Many Glacier Hotel has been occurring during the shoulder seasons (October-December, April-May) since 2001, and is expected to continue through 2008. The work primarily occurs inside of the hotel or within the area immediately surrounding the hotel. Impacts also include worker vehicle traffic on the Many Glacier Road during times when the road is usually closed. Boundary fencing should not prohibit the movement of wildlife. Native ungulates are able to jump over cattle fencing and other species, including bears, can move over or through the fences without injury. The Final Commercial Services Plan and Environmental Impact Statement concluded that impacts to wildlife from new proposals in the Preferred Alternative would be moderate long-term adverse due mostly to proposed construction in a wildlife corridor and the lengthening of the operating season by two weeks. The overall impact of these projects in conjunction with the No Action Alternative would be moderate long-term local adverse, however the No Action Alternative would be responsible for very little of this impact.

Conclusion: The No Action Alternative would result in minor long-term local adverse impacts to wildlife largely due to the potential for conflicts with bears that may access the site. Cumulative impacts under this alternative would be moderate long-term local adverse, however the No Action Alternative would account for very little of this impact.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

Preferred Alternative: Under the Preferred Alternative, wildlife in the area would be temporarily disturbed during the work but there would be no permanent habitat changes due to the improvements. During trenching, wildlife in the area would experience an increase in noise disturbance due to the amount of trenching required. Road plowing and the presence of workers would also disturb some species. Working through December and early in the spring would disturb wildlife that use the area at these times, especially elk, bighorn sheep and their predators. Groups of elk forage in the meadows near the WWTP during this period, and bighorn sheep come onto the road during movements north and south, or to escape deeper snows above. Human activity during these periods, when ungulates are more restricted to lower elevations, can result in increased habituation, especially for some bighorn sheep, or displacement to less favorable locations. Loss of foraging opportunities and increased stress levels may affect reproduction or even survival for some individuals. After the four-five year construction period, wildlife movements are expected to return to levels observed prior to the start of hotel rehabilitation. Disturbance would be most pronounced near the WWTP where there is usually little disturbance during the fall, winter, and spring. Some species would likely alter their usual movements through the area when construction is occurring. After December, and before work resumes in March, movements are expected to return to normal. The lagoon at the WWTP occasionally attracts bears and installing a new electric fence away from trees would reduce the possibility of wildlife gaining access to the site, and would thus provide long-term beneficial impacts for bears. The Preferred Alternative would result in moderate short-term local adverse impacts to wildlife. Effects are expected to be short-term because no permanent habitat changes would occur.

Cumulative Effects: The proposed construction would likely overlap with rehabilitation work at the Many Glacier Hotel. The sewer system work would occur over a larger area and likely produce more noise than the hotel rehabilitation, most of which occurred indoors or immediately outside of the hotel. In conjunction with the hotel rehabilitation, at least twelve years of extended construction seasons in Many Glacier will have occurred once the improvements are completed. Commercial services occur in the area between June and October and sewer system improvements would not occur at this time. Boundary fencing should not prohibit the movement of wildlife. Native ungulates are able to jump over cattle fencing and other species such as bears can move over or under the fences without injury. The overall impact to wildlife in the Many Glacier area from the combination of these projects would be moderate long-term local adverse.

Conclusion: The Preferred Alternative would result in moderate short-term local adverse impacts to wildlife due to noise and human presence during construction. The cumulative effects would be moderate long-term local adverse due to the proposed project and ongoing Many Glacier Hotel rehabilitation work.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Special Status (Federal and State Listed) Species

The Endangered Species Act of 1973 requires examination of impacts on all federally-listed threatened, endangered, and candidate species. Section 7 of the Endangered Species Act requires all federal agencies to consult with the U.S. Fish and Wildlife Service to ensure that any action authorized, funded, or carried out by the agency does not jeopardize the continued existence of listed species or critical habitats. In addition, the 2006 Management Policies (NPS 2006) and Director’s Order 77 Natural Resources Management Guidelines require the National Park Service to examine the impacts on federally-listed species, as well as state-listed threatened, endangered, candidate, rare, declining, and sensitive species.

Further protection under the Migratory Bird Treaty Act makes it unlawful to pursue, hunt, kill, capture, possess, buy, sell, purchase, or barter any migratory bird, including the feathers or other parts, nests, eggs, or migratory bird products. In addition, this act serves to protect environmental conditions for migratory birds from pollution or other ecosystem degradations. Some migratory birds are present in the vicinity of the historic district, but the area of ground disturbance consists primarily of non-native vegetation. Construction-related noise could potentially disturb bird species, but these adverse impacts would be 1) temporary, lasting only as long as construction, and 2) negligible, because of the relatively small amount of avian habitat affected, compared to suitable habitat for birds available in the vicinity.

Table 4. Federally-listed species known to occur in Glacier National Park.

Status Species

Endangered Gray wolf (Canis lupus)

Threatened Bald eagle (Haliaeetus leucocephalus)

Grizzly bear (Ursus arctos horribilis)

Canada lynx (Lynx canadensis)

Bull trout (Salvelinus confluentus)

Candidate Slender moonwort (Botrychium lineare)

The endangered gray wolf and the threatened bald eagle, grizzly bear and Canada lynx are known to live in the Many Glacier area. A 2002 Biological Assessment (BA) analyzed the impacts of rehabilitation construction occurring at the Many Glacier Hotel from 2002-2008. Work was scheduled to occur during the same seasons (March-May and September-December) as the work proposed in this EA and most of the hotel work was to occur inside or in the immediate area of the hotel. The project analyzed in this EA would essentially extend that seasonal work an additional 4-5 years and include outside construction in a larger area.

There are 28 wildlife species of concern in the Many Glacier area, including the fisher, wolverine, Rocky Mountain bighorn sheep, peregrine falcon, northern goshawk, golden eagle, harlequin duck, pileated woodpecker, olive-sided flycatcher, trumpeter swan, white-tailed ptarmigan, brown creeper, Clark’s nutcracker, Barrow’s  goldeneye, ruffed grouse, calliope hummingbird, three-toed woodpecker, Hammond’s flycatcher, winter wren, veery, lazuli bunting, Brewer’s sparrow, Columbia spotted frog, boreal toad, and long-toed salamander.

Gray Wolf. Management and recovery of wolves in the Northwest Montana Recovery Zone, of which Glacier National Park is a part, is directed by the Northern Rocky Mountain Gray Wolf Recovery Plan (USFWS 1987). Glacier National Park’s predominately-natural landscape contains some of the most secure and productive wolf habitat in the Northwest Montana Recovery Zone. Despite fluctuating wolf numbers since 1986, Glacier’s established wolf population continues to serve as a source for natural recolonization in northwest Montana and southern Canada (Boyd-Heger 1997).

Gray wolves are wide-ranging and their distribution is tied primarily to that of their principal prey (deer, elk, and moose). Important components of wolf habitat are: 1) a sufficient, year-round prey base of ungulates and alternate prey; 2) suitable and somewhat secluded denning and rendezvous sites; and 3) sufficient space with minimal exposure to humans (USFWS 1987). Low elevation river bottoms that are relatively free from human influence provide important winter range for ungulates and wolves. Wolves are especially sensitive to disturbance from humans at den and rendezvous sites. Pups are born in late March to early May and remain near the den through most of the summer (USFWS 1987). Human activity near den sites can lead to pack displacement or physiological stress perhaps resulting in reproductive failure or pup mortality (Mech et al. 1991). Rendezvous sites are resting and gathering areas occupied by wolf packs during summer and early fall after the natal den is abandoned.

The two closest wolf packs to the Many Glacier area are in the Belly River drainage and in the St. Mary area. Individuals from these packs probably occasionally hunt or travel in the Many Glacier area, especially during fall, spring, and winter. A wolf collared in the Belly River drainage in Alberta in February moved south to Swiftcurrent Ridge, within a couple miles of the WWTP, before moving back north. No denning or rendezvous sites for wolves have been documented in the Valley.

Grizzly Bear. Field work conducted during 1998-2000 identified 246 grizzly bears in the park; however, many of these bears were also found in the areas surrounding the park (Stetz 2003, Kendall and Waits 2002). Grizzly bear habitat is found throughout the park and ranges from the lowest valley bottoms to the summits of the highest peaks. Grizzly bears require large areas of undeveloped habitat (including a mixture of forests, moist meadows, grasslands, and riparian habitats) and a substantial amount of solitude from human interactions (USFWS 1993). They have home ranges of 130 to 1,300 square kilometers (USFWS 1993). Results from a recent bear DNA project conducted in the ecosystem have not yet been completed but will include a population estimate for the park.

Grizzly bear seasonal movements and habitat use are tied to the availability of different food sources. In spring, grizzly bears feed on dead ungulates and early greening herbaceous vegetation at lower elevations (Martinka 1972). During the summer, some bears move to higher elevations in search of glacier lilies and other roots, berries, and army cutworm moths (Euxoa auxiliaris).

Grizzly bear/human interaction is a management concern that can threaten the safety of visitors as well as that of bears. Bears that are familiar with humans have the potential to become habituated to human presence, leading to further habituation and increased potential for bear/human encounters. Habituated bears are usually relocated or hazed from developed areas, and food-conditioned bears are oftentimes removed from the population. Bears not habituated to humans are likely displaced from foraging areas and travel routes in proximity to developed areas.

Glacier National Park was placed into grizzly bear management “situations” in accordance with the Grizzly Bear Recovery Plan (USFWS 1993). Over 1 million acres of the park (proposed wilderness) are established as Management Situation 1, in which management decisions would favor the needs of the grizzly bear when grizzly habitat and other land-use values compete, and grizzly-human conflicts would be resolved in favor of grizzlies, unless a bear is determined to be a nuisance. The remainder of the park, which is developed front-country, is established as Management Situation 3, in which grizzly habitat maintenance and improvement are not the highest management considerations, grizzly bear presence would be actively discouraged, and any grizzly involved in a grizzly-human conflict would be controlled. The project location is within Management Situation 3; however, there is Management Situation 1 areas immediately adjacent to the project area.

Grizzly bears make extensive use of the Swiftcurrent Valley throughout the year. From August through October, grizzly and black bears typically forage on the southerly slopes of Mt. Altyn and other sites in the area, feeding on the berries of serviceberry and kinnikinnik and other plants. Grizzly bears have also been observed moving through the project area en route to seasonally important habitats in more remote areas. Important grizzly bear travel corridors have been identified by park biologists within the project area including near the Many Glacier Hotel and the Swiftcurrent Motor Inn. Five grizzly bear dens have been documented in the Many Glacier geographic area, all at least 3 km from the Many Glacier Hotel (Blackfeet Tribe Land Dept., D. Carney, wildlife biologist, pers. comm. and NPS files). The mean elevation for documented grizzly bear dens (n=73) east of the Continental Divide in Glacier is 6,782 feet (D. Carney, pers. comm.). Grizzly bears east of the Continental Divide in the park typically enter their dens in late November and emerge in late March or early April (GNP files). From 1986 to 2006, there were 14,467 grizzly bear observations within the park. Of those, 4,890 were from the Many Glacier subdistrict, more than any other subdistrict. However, reported observations are biased because they are dependent on where people go within the park.

Canada Lynx. Lynx habitat generally is described as climax boreal forest with a dense undercover of thickets and windfalls (Ruediger et al. 2000). Advanced successional stages of forests and dense conifer stands often are preferred habitats of lynx for denning and foraging respectively. Lynx generally forage in young conifer forests especially where their primary prey, snowshoe hare (Lepus americanus), is abundant. Other prey includes squirrels, grouse, martens, and voles. They also forage in dense, multi-storied mature conifer forest, especially during winter. Travel corridors are thought to be an important factor in lynx habitat because of their large and variable home ranges, generally 8-738 km2 (Ruediger et al. 2000). Lynx are most susceptible to disturbance during the denning period and while newborns are developing (May–August) (Claar et al. 1999).

Historically, lynx were considered “more or less common” throughout the park (Bailey and Bailey 1918). Documented sightings declined during the 1970s and 1980s and have increased in recent years (NPS files); however, sightings may not be particularly sensitive to population changes and should be interpreted with caution. Systematic lynx surveys involving snow tracking and DNA sampling were initiated in 1994 and 1999 respectively; lynx were detected in many drainages throughout the park including the Many Glacier Valley, although no estimates of population numbers nor trend were attempted. A preliminary map of lynx habitat in the park defined moist conifer forest above 4,000 feet elevation as the most likely areas supporting lynx. Little is known about lynx habitat use in the park and these criteria are general in nature. There is some habitat in the project area that meets these criteria and lynx tracks have been documented. The park’s wildlife observation database contains many records of Canada lynx in the Many Glacier area, including several records of family groups. Many of the records are from around Swiftcurrent Lake and along the Many Glacier Road in the winter. Most recently, lynx tracks were observed in the project area in December 2006, and approximately 35 lynx were captured and released from traps used for live-trapping wolverines in the Many Glacier Valley during the winters of 2004-7, representing an unknown number of individuals. The amount of development and human presence in the area makes it unlikely that lynx frequent the area during the summer and early fall, but once the road is closed to visitor use lynx use of the area likely increases.

Bald Eagle. Bald eagles use portions of Glacier National Park on a year-round basis as nesting and wintering residents (Yates 1989), and as seasonal migrants (McClelland et al. 1994, Yates et al. 2001). There are 14 known bald eagle breeding areas in the park. Bald eagles have nested at nearby Sherburne Dam in the past. In 2003, the nest fledged two young eagles. This nest was inactive in 2005 and 2006, though there may be additional nest sites that have not been located. The park is also within a major bald eagle migration corridor (McClelland et al. 1994, Yates et al. 2001). Occasional observations are made in the fall and winter in the Many Glacier Valley, though once the lakes freeze over they probably do not remain in the area.

The Montana Bald Eagle Management Plan (Montana Bald Eagle Working Group 1994) provides guidance for conservation and management efforts for bald eagles in Montana. The Montana plan is an extension of the Pacific States Bald Eagle Recovery Plan (USFWS 1986) developed by the USFWS and identifies nest site management zones and general guidelines to be used in lieu of more site-specific data. The plan also provides guidance for management based on minimum human disturbance and provides for various levels of protection within nesting territories. The park’s Bald Eagle Operational Plan and Habitat Management Guidelines (NPS 1999b) provides site-specific information and outlines habitat management actions for the protection and perpetuation of bald eagle use areas in the park. Productivity of Glacier’s nesting bald eagle population is lower than productivity documented for the rest of Montana (NPS files), and less than that recommended in the Pacific States Bald Eagle Recovery Plan for maintaining viable populations of nesting bald eagles. Reasons for lower productivity in the park may include severe winter and spring weather, deterioration of native fisheries (prey species), and human disturbance near nest and forage sites.

Bald eagles are especially sensitive to human disturbance during the breeding period (Hamann et al. 1999). The breeding period includes courtship (late February to mid-April), egg laying and incubation (late March to late May), nestling stage (mid-May to early August), and fledging (early August to late September). Effects of disturbance on breeding birds during incubation could include short-term nest abandonment or nest desertion resulting in exposure of the eggs to detrimental temperature extremes and predators (Hamann et al. 1999). Disturbance during rearing can result in trampling of young, young jumping or falling from nests before they can fly, and/or separation of young from parents. Chronic disturbance can cause nest abandonment. The potential for nest failure and nestling death due to human disturbance is reduced, but not eliminated, after nestlings reach an age of about 4 weeks (usually early to late June in GNP). Nestlings usually fledge at 10 to 12 weeks of age (by mid-August), but young eagles do not migrate from breeding areas until sometime between mid-September and early October (McClelland et al. 1996). Outside of the breeding season, disturbance by humans may cause birds to alter their feeding habits, thereby reducing normal food intake (Hamann et al. 1999).

Bull Trout. The federally listed threatened bull trout does not occur in the Many Glacier area; however, this species is found in Swiftcurrent Creek below Lake Sherburne, outside the park boundary. Bull trout require habitats offering cold summer water temperatures, complex large woody debris accumulations, and clean cobble and boulder substrates (Rieman and Mcintyre 1993, Rich 1996). Historically, bull trout were one of four native salmonid species distributed throughout the Flathead drainage. Monitoring data from 1992, 1993, and 1994 of spawning runs from Flathead Lake were the lowest on record and led to the listing of the bull trout. These recent declines in the spawning population of bull trout in virtually all monitored streams throughout the North and Middle Forks of the Flathead River indicate that changes in the system are the primary threat to bull trout at this time. Establishment of opossum shrimp (Mysis relicta) and the proliferation of predatory non-native lake trout no doubt play a key role in this decline, but complex mechanisms involving bull trout prey species or behavioral interactions with lake trout may also be involved (Montana Bull Trout Scientific Group 1995).

Slender Moonwort. Glacier has one plant species designated as a candidate species by the U.S. Fish and Wildlife Service, slender moonwort (Botrychium lineare). Slender moonwort is known to occur in the Many Glacier area, but it has not been found in or near the proposed project area.

Species of Concern. State listed species of concern to Glacier National Park are those species that are rare, endemic, disjunct, vulnerable to extirpation, in need of further research, or likely to become threatened or endangered if limiting factors are not reversed. Likewise, a species may be of concern because of characteristics that make them particularly sensitive to human activities or natural events. In addition, species of concern may also include big game, upland game birds, waterfowl, carnivores, predators, and furbearers whose populations are protected in the park but subject to hunting and trapping outside of the park.

There is no comprehensive information on the exact composition of species of concern found in the Many Glacier and Swiftcurrent developed areas. However, the visitor service zone provides habitat for fisher, wolverine, Rocky Mountain bighorn sheep, peregrine falcon, northern goshawk, golden eagle, harlequin duck, common loon, trumpeter swan, Barrow’s goldeneye, ruffed grouse, Columbia spotted frog, boreal toad, and long-toed salamander.

There are 18 vascular plants of concern in the Many Glacier area: western moonwort, slender moonwort, peculiar moonwort, pink corydalis, dense-leaf draba, northern eyebright, three-flowered rush, Hudson’s Bay bulrush, little false asphodel, cushion townsendia, Banff loose-flowered bluegrass, one-flowered cinquefoil, stalked-pod crazyweed, alpine glacier poppy, autumn willow, tufted club-rush, simple kobresia, and timberline buttercup. There are also two moss species of concern in this area: Bryum pallens and Sphagnum girgensohnii. There is one historical record of pink corydalis near the Many Glacier Hotel. However, no observations of this species have been made since 1948, and a survey in 2001 could not relocate this plant that typically grows in recently burned areas. There are no records of plants of special concern in the Swiftcurrent developed area.

Impact Analysis – Special Status Species

No Action Alternative: The No Action Alternative would not change current conditions for special status species. Repairs to the sewage system and WWTP would be done on an as-needed basis, and some noise and disturbance would be created during repairs. Because of the current location and condition of the fence surrounding the WWTP and lagoon, grizzly bears could occasionally access the area inside of the fence, possibly leading to confrontations with employees. This could result in a habituated bear that may eventually need to be removed from the population. If a significant sewage leak occurred due to aging facilities it could temporarily and adversely impact any species that are reliant on aquatic habitats including bull trout, bald eagle, waterfowl, and amphibians. Conditions for all other special status species would not change. The impact to special status species from the No Action Alternative would be minor to moderate, short-term or long-term, local adverse.

Cumulative Effects: Rehabilitation work at the Many Glacier Hotel has been occurring during the shoulder seasons (October-December, April-May) since 2001, and is expected to continue through 2008. The work primarily occurs inside of the hotel or within the area immediately surrounding the hotel. Impacts also include worker vehicle traffic on the Many Glacier Road during times when the road is usually closed. Boundary fencing should not prohibit the movement of wildlife. Native ungulates are able to jump over cattle fencing and other species, including bears, can move over or through the fences without injury. The Final Commercial Services Plan and Environmental Impact Statement concluded that impacts to wildlife from new proposals in the Preferred Alternative would be moderate long-term adverse due mostly to proposed construction in a wildlife corridor and the lengthening of the operating season by two weeks. Species of Concern would be similarly impacted by these projects. The overall impact of these projects in conjunction with the No Action Alternative would be moderate long-term local adverse, however the No Action Alternative would be responsible for very little of this impact.

Conclusion: The No Action Alternative would result in minor long-term local adverse impacts due primarily to the potential access bears currently have to the WWTP. Cumulative effects would be moderate long-term local adverse, however the No Action Alternative would be responsible for very little of this impact.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

Preferred Alternative:

Gray wolf: No denning or rendezvous sites are known in the Many Glacier Valley though wolves probably forage in and travel through the area occasionally. Wolves and their prey could be displaced from the immediate area during construction due to noise and human presence but would likely return during the winter months when no construction is occurring. Impacts are expected to be negligible short-term local adverse.

Grizzly bear: The impacts to grizzly bears from construction outside of the visitor use season were analyzed for the 2002 Many Glacier Hotel rehabilitation BA. The BA concluded that the project may affect grizzly bears but was not likely to adversely affect the species or its habitat. This was based on: 1) the concentrated nature of the activity in a developed area, 2) the availability of displacement areas, 3) the mortality risk to grizzly bears would not increase, 4) no attractants would be created, and 5) no alteration of habitat would occur. These factors would be largely unchanged by the current project. However, the work would occur over a larger area and likely create more noise. Construction would occur within MS3 habitat though nearby MS1 habitat could be impacted temporarily by noise. No permanent loss of grizzly bear habitat would occur. Construction and road plowing would occur during daylight hours when grizzly bears are more likely to be in daybeds or forested areas.

The greatest number of grizzly bear observations recorded in the park’s Bear Information Management System (BIMS) database are from the months of May through August, with considerably fewer records from the months of March, April, September, October, and November. The number of records is likely correlated with visitors being in the area to see them and not necessarily an indicator of relative grizzly bear presence and habitat use. Some bears have habituated to the high level of human activity in this area during these months, and continue to use open habitats along roads and within line of site of visitor facilities, roads, and park administrative offices, when people are present. Other bears that are more sensitive to human disturbance probably avoid the area entirely or concentrate their activity at night or in remote areas relatively free from human influence. Recent observations suggest that some grizzly bears continued to use productive year-round habitats found on Mt. Altyn and an important movement corridor encompassing the Many Glacier developed area, while construction activities were occurring at the Many Glacier Hotel.

The road into the Many Glacier Valley usually remains open to the public until mid-late November (when grizzlies in this area typically enter dens for the winter); consequently, construction activity at this time would have the potential to affect grizzly bears preparing for hibernation. However, workers would commute to and from the site each day and be confined to the day’s work area. Thus, effects of their activity may be less than that of visitors who would have greater access to surrounding MS1 grizzly habitat. After closure of the road to public access in November, when only construction personnel and affiliated NPS employees are present, bear activity will have diminished as bears enter winter dens. Grizzly bear dens (mean elevation 6,782 feet [D. Carney, pers. comm.]) are typically located at elevations far above that of the valley bottom floor (elevation 4,875 feet); making impacts to denning bears unlikely from the proposed construction.

Additional people in the valley during a time of typically low human use and relatively high bear use may increase the potential for habituation leading to food-conditioned behavior (if bears obtain human food or garbage). Habituated bears that become food-conditioned or exhibit aggressiveness toward people are at risk of mortality from control actions within the park or when they display that behavior outside the park. The replacement and relocation of the existing fence around the lagoon would reduce the likelihood of bears entering the area and becoming habituated to the site. In consideration of the nature, timing, and location of the project, mortality risk to grizzly bears from the proposed action would be negligible. The presence of a park law enforcement ranger and biological technician would ensure the enforcement of food-storage regulations and would minimize the risk of habituation or mortality. Consequently, potential effects to grizzly bears would be minor short-term local adverse.

Canada lynx: The impacts to Canada lynx from construction outside of the visitor use season were analyzed for the 2002 Many Glacier Hotel rehabilitation BA. The BA concluded that the project may affect lynx but was not likely to adversely affect the species or its habitat. This conclusion was based on: 1) the concentrated nature of the activity in an established park developed area, 2) the availability of displacement areas, 3) mortality risk to the species would not increase, 4) snowshoe hare populations would be unaffected, 5) no expansion of the range of competitors and/or predators, and 6) no alteration of habitat would occur. These factors would be largely unchanged by the current project. However, the work would occur over a larger area and likely create more noise.

The proposed project would occur within an area known to contain suitable and occupied lynx habitat. Actions that adversely affect lynx would include elevated levels of human access into lynx habitat, human activity at or near den sites, modification of forested habitat, expansion of the range of competitors and/or predators, or reduction of prey species populations (Ruediger et al. 2000). The proposed action would not result in the modification of forested habitat, the reduction of prey species, or the expansion of the range of competitors/predators. Coyotes and mountain lions appear to be present in the Many Glacier developed area year-round, but are not typically detected in remote areas in winter (Hahr 2001). Late and early season plowing of the Many Glacier Road is not expected to alter movement patterns of lynx competitors/ predators as snow accumulation on the road does not appear to currently limit use of the area by these species in winter. The proposed action would not alter snow conditions along trails that could serve as travel routes for potential predators /competitors of lynx in remote areas. Late and early season administrative use of the Many Glacier Road is also not expected to negatively affect lynx movement in the Swiftcurrent Valley. Lightly used roads do not appear to affect lynx habitat use while high traffic volumes may preclude lynx use (Ruediger et al. 2000).

Although no lynx den sites have been documented in the park, observations of lynx family groups indicate the likelihood of a resident lynx population in several areas including the Many Glacier area. Intrusive human activities such as construction during the denning period (May to August) have the potential to disturb lynx at den sites; however, the effects of the proposed action on denning lynx are expected to be minimal due to the selection of remote forested areas for den sites.

No studies have specifically examined the effects of construction related activities on lynx behavior, although several authors have suggested that lynx are “generally tolerant of humans” and probably not displaced by human presence (Ruediger et al. 2000). This alternative would not alter habitats or human use patterns in or near areas that could potentially serve as den sites. Sewer line and treatment plant construction, travel of workers to and from the site, and plowing of the Many Glacier Road would occur during daylight hours when lynx are less active (Ruediger et al. 2000). If lynx are displaced, they would likely return to the area during winter months when no construction is occurring. The overall impact on lynx would be minor short-term local adverse and would likely be manifested by temporary disturbance to resting lynx present in the road corridor or the developed areas. Disruption of foraging is unlikely given the natural nocturnal activity patterns of lynx.

Bald eagle: The treatment plant is located within the home range of a bald eagle nest site located on nearby Lake Sherburne. Eagles are frequently seen foraging along Swiftcurrent Creek near the WWTP, though the eagles do not use the WWTP. These eagles could be displaced by sewage line construction or from work at the WWTP, especially if conducted in March and April. If work is conducted at this time in these locations, restrictions on time of day (i.e. no work until one hour after sunrise or one hour before sunset from November through April) would be necessary to allow eagles to forage in these areas. Eagles are not known to use Swiftcurrent Lake for foraging. With restrictions in March and April, impacts to bald eagles are expected to be negligible short-term local adverse because there are additional foraging areas nearby and Swiftcurrent Creek is sufficiently distant from the WWTP.

Bull trout: No adverse impacts are expected to bull trout and the sewage system changes would improve water quality in the adjacent stream. Impacts would be negligible long-term local beneficial.

Listed plants: There are no known locations of Spalding’s catchfly, water howellia, or slender moonwort near the proposed project. There would be no effect on these species or their habitat.

Species of Concern. Fisher and wolverine likely use the Many Glacier area throughout the year particularly in the winter when there are no visitors. They could be displaced by construction in late fall and early spring. Bighorn sheep use the Many Glacier Valley throughout the year. Some of the sheep are habituated to human presence and they were observed licking salt from the vehicles of construction workers during Hotel rehabilitation. Nesting waterfowl, if present, could be displaced by noise from the northern end of Swiftcurrent Lake if the sewage line in this area is excavated during the spring. There is a golden eagle nest located on Mt. Altyn that was known to be active in 2007. If the nest is confirmed to be active, construction within an 800-meter buffer area (Figure 2) around the nest should be limited to one hour after sunrise and one hour before sunset between April 1 and August 15. The peregrine falcon appears to be extremely rare and is not known to nest anywhere in the park. The northern goshawk is not known to nest in the project area, and there are few sighting reports from Many Glacier, though no surveys have been conducted for this species. Harlequin ducks are present between early April and mid-September, nesting on at least one stream in the valley, but are not found close enough to the project area to be affected by construction activity. Common loons are found on lakes in the valley, but do not nest near the project area. Trumpeter swans are primarily spring and fall migrants over the Many Glacier Valley and are not likely to be affected by the project. Since wetlands will not be affected by the project, Columbia spotted frogs, boreal toads, and long-toed salamanders will not likely be impacted. There are not expected to be impacts to any other species of concern. If conservation measures are followed impacts are expected to be minor short-term local adverse to species of concern.

Cumulative Effects: Rehabilitation work at the Many Glacier Hotel has been occurring during the shoulder seasons (October-December, April-May) since 2001, and is expected to continue through 2008. The work primarily occurs inside of the hotel or within the area immediately surrounding the hotel. Impacts also include worker vehicle traffic on the Many Glacier Road during times when the road is usually closed. Construction proposed under the Preferred Alternative would extend shoulder season work an additional four years. Boundary fencing should not prohibit the movement of wildlife. Native ungulates are able to jump over cattle fencing and other species, including bears, can move over or through the fences without injury. The Final Commercial Services Plan and Environmental Impact Statement (NPS 2004) concluded that impacts to wildlife from new proposals in the Preferred Alternative would be moderate long-term adverse due mostly to proposed construction in a wildlife corridor and the lengthening of the operating season by two weeks. The overall impact of these projects in conjunction with the Preferred Alternative would be moderate long-term local adverse. If conservation measures are followed, the Preferred Alternative would add only temporarily to this impact.

Conclusion: Impacts from the Preferred Alternative would be: negligible short-term local adverse for the gray wolf and bald eagle, minor short-term local adverse on grizzly bears and Canada lynx, negligible long-term local beneficial to bull trout, minor short-term local adverse on species of concern, and no effect on federally listed plant species. Cumulative impacts in conjunction with the Preferred Alternative would be moderate long-term local adverse and would result primarily from extending shoulder season work for an additional four years. Glacier National Park biologists have determined that this project may affect but is not likely to adversely affect the Canada lynx and grizzly bear and are preparing a Biological Assessment (BA) in accordance with Section 7 of the Endangered Species Act. The BA will be submitted to the USFWS as required.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Water Quality

National Park Service policies require protection of water quality consistent with the Clean Water Act. The purpose of the Clean Water Act is to "restore and maintain the chemical, physical, and biological integrity of the Nation's waters". To enact this goal, the U.S. Army Corps of Engineers has been charged with evaluating federal actions that result in potential degradation of waters of the United States and issuing permits for actions consistent with the Clean Water Act. The U.S. Environmental Protection Agency also has responsibility for oversight and review of permits and actions, which affect waters of the United States. Under Section 401 of the Clean Water Act, states and tribes can review and approve, condition, or deny all Federal permits or licenses that might result in a discharge to State or Tribal waters, including wetlands. States and Tribes make their decisions to deny, certify, or condition permits or licenses primarily by ensuring the activity will comply with state water quality standards. In addition, states and tribes look at whether the activity will violate effluent limitations, new source performance standards, toxic pollutants, and other water resource requirements of state/tribal law or regulation. The Section 401 review allows for better consideration of state-specific concerns.

Currently, the Many Glacier WWTP does not have a discharge permit for surface water or ground water. Wastewater from the Many Glacier facility is currently discharged using percolation ponds, which is included under the category of groundwater discharge.

Several water bodies are in the immediate vicinity of the sewage system and WWTP including Swiftcurrent Lake, Sherburne Reservoir, and Swiftcurrent Creek. The Many Glacier and Swiftcurrent developed areas are located in the Swiftcurrent Creek drainage. The Many Glacier developed area is located on the eastern shore of Swiftcurrent Lake near its outlet. The Swiftcurrent developed area is located north of Swiftcurrent Creek, which flows into Swiftcurrent Lake.

Water quality in Glacier National Park is considered to be very good. The water use classification for the streams in the park is A-1 (Montana Water Quality Act ARM 17.30.608). This classification is for high quality water suitable for drinking and culinary food processing following conventional treatment, as well as bathing, swimming, and recreation. It is also suitable for growth and propagation of salmonid fishes and aquatic life, waterfowl, furbearers, and agricultural and industrial water supplies (Montana Water Quality Act ARM 17.30.622).

Between 1984 and 1990, the National Park Service and the Flathead Lake Biological Station conducted a monitoring program to establish a baseline for water quality in the park (Ellis et al. 1992). The study included the chemical, physical, and biological sampling of several lakes including Swiftcurrent Lake. Results of the monitoring program (Ellis et al. 1992) indicated that water quality in Swiftcurrent Lake is extremely good. Swiftcurrent Lake contains few dissolved solids, is low in nutrients and productivity, and would be sensitive to phosphorus loading.

During the spring and summer of 2005, a study was conducted to determine potential sources of fecal contamination entering local ponds and Swiftcurrent Creek (Gannon et al. 2006). This study determined that the overall level of fecal contamination in Many Glacier water bodies was very low and well below EPA recreational standards. It also determined that wildlife including waterfowl and bears were the primary contributors to overall fecal load, though horses may also contribute significantly. Human fecal input was not found to reach Swiftcurrent Creek. The sewage collection and treatment system for the Many Glacier/Swiftcurrent area was also evaluated in 2005 (Kennedy and Jenks 2006). Needed improvements were identified and alternatives for implementing the improvements were developed. The actions proposed under the Preferred Alternative represent the recommendations from the report and would meet the requirements for a MDEQ discharge permit.

Impact Analysis – Water Quality

No Action Alternative: Under the No Action Alternative, deteriorating sewage pipes would not be replaced until a leak was detected. Though no contamination from the sewage system has been detected at this point, continued deterioration would threaten nearby water sources. The high potential for future sewage leaks would result in minor local adverse impacts. Impacts would be short-term or long-term depending on how quickly a leak was detected. In addition, the current system does meet the standards for obtaining a discharge permit from MDEQ because some pipes are too narrow.

Cumulative Effects: The Many Glacier Hotel rehabilitation and fencing along the eastern boundary are not expected to impact water quality in the area. Actions proposed in the Commercial Services Plan including new housing and parking areas could impact water quality by creating new areas that pollutants could runoff of, but these impacts would be negligible because proper design and mitigation would be required prior to construction. In conjunction with the No Action Alternative, the cumulative impacts would be minor long-term local adverse due mostly to the potential for sewage leaks.

Conclusion: Impacts from the No Action Alternative, including cumulative impacts, would be minor long-term local adverse impacts due to the potential for future sewage leaks.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

Preferred Alternative: No adverse impacts to water quality are expected from the Preferred Alternative. During construction, best management practices would be implemented to ensure erosion into lakes or streams would not occur. Improvements proposed under this alternative, including re-lining the sewage ponds and replacing deteriorating sewage pipes, would result in less potential for future sewage leaks. This alternative would result in moderate long-term local beneficial impacts to water quality. The improvements proposed under this alternative would qualify the system for a discharge permit from MDEQ.

Cumulative Effects: The Many Glacier Hotel rehabilitation and fencing along the eastern boundary are not expected to impact water quality in the area. Actions proposed in the Commercial Services Plan including new housing and parking areas could impact water quality by creating new areas that pollutants could runoff of, but these impacts would be negligible because proper design and mitigation would be required prior to construction. Benefits from the Preferred Alternative in conjunction with these projects would result in negligible long-term local beneficial cumulative impacts.

Conclusion: The Preferred Alternative would result in minor long-term local beneficial impacts to water quality by making improvements to the deteriorating sewage system. Cumulative impacts would be negligible long-term local beneficial.

Because there would be no major, adverse impacts to a resource or value whose conservation is (1) necessary to fulfill specific purposes identified in the establishing legislation and proclamation of Glacier National Park; (2) key to the natural or cultural integrity of the park; or (3) identified as a goal in the park’s general management plan or other relevant National Park Service planning documents, there would be no impairment of the park’s resources or values.

### Park Operations

The WWTP is operated approximately May through October each year. The existing Many Glacier service area consists of two primary wastewater collection basins: Many Glacier and Swiftcurrent. The average number of visitors to these areas over the past three years was 232,300 annually. The WWTP also receives monthly sewage dumps from four vault toilets, bi-weekly dumps from five portable toilets, and sewage deliveries from the Logan Pass Visitor Center.

Wastewater flow and waste constituent loads could increase. The park’s Commercial Services Plan recommends: upgrading facilities for accessibility, parking modifications, extending the operating season two weeks, constructing a new employee dormitory and recreational facilities near the Many Glacier Hotel, and new cabin-style guest accommodations and employee housing at Swiftcurrent. These additions could slightly increase wastewater flow but the increase would be largely offset with water conservation measures, such as low-flow toilets, that would be installed in new buildings or in existing rooms when they are renovated. The WWTP and sewage system would be able to handle these potential increases.

Impact Analysis – Park Operations

No Action Alternative: There would be no change to current staffing levels, equipment use, or facilities under this alternative. The dry well at the Swiftcurrent lift station would continue to be considered a confined space. This requires that all operators obtain confined space training and that at least two operators are at the facility when repairs are made. The WWTP is already short-staffed and having two people at one location is inefficient and often difficult to schedule. Repairs to the sewage line and WWTP would continue to be made on an as-needed basis making scheduling difficult. Therefore, the impacts to park operations would be minor long-term local adverse.

Cumulative Effects: The boundary fencing project and Many Glacier Hotel rehabilitation would not impact park operations. Actions proposed in the Commercial Service’s Plan are not expected to change staffing levels or equipment use. The recommendations would be beneficial to operations by better separating employee housing from guest accommodations and upgrading facilities and utilities. Adverse impacts from the No Action Alternative would somewhat offset the beneficial impacts of actions proposed in the Commercial Service’s Plan, and the overall cumulative impact would be negligible long-term local beneficial.

Conclusion: Impacts to park operations from the No Action Alternative would be minor long-term local adverse because of the confined space designation and the potential for future sewage leaks. Adverse impacts from the No Action Alternative would somewhat offset the beneficial impacts of actions proposed in the Commercial Service’s Plan, and the overall cumulative impact would be negligible long-term local beneficial.

Preferred Alternative: Actions proposed in the Preferred Alternative would benefit park operations. A new pump system would be installed at the Swiftcurrent lift station that would eliminate the confined space designation and require fewer employees for maintenance, provide better access for maintaining equipment, and provide standardized pumping equipment throughout the park. Moving the chain-link fence around the WWTP and lagoon would allow personnel to properly mow and maintain the berm. Replacing all of the old sections of pipe at one time would prevent staff from periodically having to schedule system repairs. The overall impact on park operations would be moderate long-term local beneficial.

Cumulative Effects: The boundary fencing project and Many Glacier Hotel rehabilitation would not impact park operations. Actions proposed in the Commercial Service’s Plan are not expected to change staffing levels or equipment use. The recommendations would be beneficial to operations by better separating employee housing from guest accommodations and upgrading facilities and utilities. Improvements to the sewage system would help support the increases in housing and accommodations proposed for the Swiftcurrent area. In conjunction with beneficial impacts of the Preferred Alternative, cumulative impacts to park operations in the area would be moderate long-term local beneficial.

Conclusion:  The overall impact on park operations from the Preferred Alternative would be moderate long-term local beneficial. Proposed actions in the Commercial Service’s Plan in conjunction with the Preferred Alternative would result in moderate long-term local beneficial cumulative impacts to park operations.

# Consultation and Coordination

Agencies/Tribes/Organizations/Individuals Contacted (EA Recipients)

Federal/International/Tribes

Advisory Council on Historic Preservation

Max Baucus, United States Senate

Flathead National Forest (Kalispell, Hungry Horse)

Premier of the Province of Alberta, Honorable Ralph Klein

Premier of the Province of British Columbia

Dennis Rehberg, United States House of Representatives

Jon Tester, United States Senate

U.S. Army Corps of Engineers

U.S. Environmental Protection Agency

U.S. Fish and Wildlife Service (Helena and Creston)

U.S. Geological Survey, Biological Resources Division

U.S. Department of the Interior, Office of the Solicitor (Billings)

Waterton Lakes National Park

Earl Oldperson, Chair, Blackfeet Tribal Business Council w/copies to Tribal Council and the Blackfeet Tribal Cultural Liaison

James Steele, Chair, Confederated Salish and Kootenai Tribes of the Flathead Reservation w/copies to Tribal Council and Confederated Salish and Kootenai Tribal Preservation Department

State

Environmental Quality Council, Director

Montana Department of Environmental Quality, Board of Environmental Review

Montana Department of Environmental Quality Permitting & Compliance

Montana Department of Environmental Quality, Water Protection Bureau

Montana Department of Environmental Quality, Air Quality Division

Montana Department of Natural Resources and Conservation

Montana Fish, Wildlife, and Parks, Region One Supervisor, Kalispell

Montana State Historic Preservation Office

Brian Schweitzer, Governor of Montana

Stillwater State Forest

County and City

Flathead County Board of Commissioners

Glacier County Commissioners

Mayors and City Councils of Browning, Kalispell, Columbia Falls, and Whitefish, Montana

Public Libraries: Bigfork, Columbia Falls, Kalispell, Whitefish

Others

Flathead Basin Commission

Friends of the Wild Swan

Glacier National Park Fund

Glacier Natural History Association

Glacier Park Inc.

Glacier Raft Company

Great Northern Whitewater Resort

Montana Preservation Alliance

Montana Raft Company

Montana Wilderness Association

National Parks Conservation Association

Wilderness Watch

Wild River Adventures

Individuals

If you would like to see a list of individuals on the park mailing list, please contact the park.

Preparers and Consultants

Tara Carolin, Ecologist, Glacier National Park

Cory Davis, Environmental Compliance Technician, Glacier National Park

Jim Foster, P.E., Civil Engineer, Glacier National Park

Lon Johnson, Historical Architect, Cultural Resource Specialist, Glacier National Park

Bill Michels, Air & Water Quality Resource Specialist, Glacier National Park

Steve Gniadek, Wildlife Biologist, Glacier National Park

Mary Riddle, Environmental Protection and Compliance Officer, Glacier National Park

John Waller, PhD, Wildlife Biologist, Glacier National Park

Del Zimmerli, Facility Manager, Glacier National Park

# Summary of Compliance with Federal and State Regulations

National Environmental Policy Act (NEPA) and Regulations of the Council on Environmental Quality — The National Environmental Policy Act applies to major federal actions that may significantly affect the quality of the human environment. This generally includes major construction activities that involve the use of federal lands or facilities, federal funding, or federal authorizations. If the environmental effects are undetermined then an Environmental Assessment is prepared to evaluate potential impacts. This Environmental Assessment meets the requirements of the NEPA and regulations on the Council on Environmental Quality in evaluating potential effects associated with activities on federal lands. If no significant effects are identified a finding of no significant impact (FONSI) would be prepared. If significant impacts are identified, then a notice of intent (NOI) would be filed for preparation of an Environmental Impact Statement.

The Endangered Species Act of 1973, as amended (16 U.S.C. 1531 et seq.) — Section 7 of the Endangered Species Act is designed to ensure that any action authorized, funded, or carried out by a federal agency likely would not jeopardize the continued existence of any endangered or threatened plant or animal species. If a federal action may affect threatened or endangered species, then consultation with the U.S. Fish and Wildlife Service (USFWS) is required. Glacier National Park biologists have determined that this project may affect but is not likely to adversely affect the Canada lynx and grizzly bear and are preparing a Biological Assessment (BA) in accordance with Section 7 of the Endangered Species Act. The BA will be submitted to the USFWS as required.

Section 404 of the Clean Water Act (CWA) and State and Local Water Quality — Section 404 of the Clean Water Act authorizes the U.S. Army Corps of Engineers to prohibit or regulate, through a permitting process, discharge or dredged or fill material or excavation within waters of the United States. No work would be conducted within a stream or other body of water. Therefore, no permits are required for construction work. Improvements to the WWTP and sewage system would qualify the system for a MDEQ groundwater discharge permit.

Executive Order 11988 for Floodplain Regulations — Ground disturbance would be temporary and would not affect water flows. The proposed project would be conducted to benefit and support two historic districts and does not involve building a new structure within the floodplain. A Statement of Findings for floodplains will not be prepared.

Executive Order 11990, Protection of Wetlands and Director’s Order 77-1 Wetlands Protection — Executive Order 11990 requires federal agencies to avoid, where possible, adversely impacting wetlands. In accordance with DO 77-1 Wetlands Protection, proposed actions that have the potential to adversely impact wetlands must be addressed in a Statement of Findings for wetlands. No wetlands would be affected by actions proposed in either alternative according to the USFWS (1992) National Wetland Inventory Mapping.

National Historic Preservation Act of 1996, as amended (16 U.S.C. 470, et Seq.) — Section 106 of the National Historic Preservation Act of 1966 (as amended) requires federal agencies to consider effects of any federal action on cultural resources eligible for or listed in the National Register of Historic Places (NHRP), prior to initiating such actions. This Environmental Assessment serves the public notification requirements of the act. Consultation with the State Historic Preservation Office and the Tribal Historic Preservation Officers to reach a finding of effect would be carried out independent of this EA.

# References

Bailey, V. and F.M. Bailey. 1918. Wild animals of Glacier National Park. Department of the Interior, National Park Service.

Boyd-Heger, D. 1997. Dispersal, genetic relationships, and landscape use by colonizing wolves in the Central Rocky Mountains. Ph.D. diss., University of Montana, Missoula, MT.

Claar, J.J., N. Anderson, D. Boyd, M. Cherry, B.Conard, R. Hompesch, S. Miller, G. Olson, H. Ihsle Pac, J. Waller, T. Wittinger, H. Youmans. 1999. Carnivores. Pages 7.1-7.63 In Joslin, G. and H. Youmans, coordinators. Effects of recreation on Rocky Mountain wildlife: A review for Montana. Committee on Effects of Recreation on Wildlife. Montana Chapter of the Wildlife Society.

Dutton, B.L., J. Hadlock, M. Arthur, D. Marrett, A. Goldin, and A. Zhu.  2001.  Soils of Glacier National Park.  Land and Water Consulting, Inc., Missoula, MT.

Ellis, B.K., Stanford, J.A., Craft, J.A., and Chess, D.W. 1992. Monitoring Water Quality of Selected Lakes in Glacier National Park, Montana, Analysis of Data Collected, 1984 - 1990, Open File Report 129-92, Flathead Lake Biological Station, The University of Montana, Polson, MT.

Gannon, J.E., M. Kinnersley, C. Wall, and W.E. Holben. 2006. Bacterial Source Tracking in Glacier National Park – Final Report. Prepared for Glacier National Park through the University of Montana Microbial Ecology Program.

Hahr, M. 2001. Winter distribution and habitat use of lynx, fisher, and wolverine detected along track transects in Glacier National Park, Montana. Master of Science thesis, University of Montana, Missoula, MT.

Hamann, B., H. Johnston, P. McClelland, S. Johnson, L. Kelly and J. Gobielle. 1999.  Birds Pages 3.1-3.34.  In Effects of recreation on Rocky Mountain wildlife:  A review for Montana.  Committee on Effects of Recreation on Wildlife, Montana Chapter of the Wildlife Society.

Kendall, K. C., L.P. Waits. 2002. Using DNA to monitor grizzly bear populations in the Greater Glacier Area. Northern Rocky Mountain Science Center, U.S. Geological Survey, Glacier National Park, West Glacier, MT. 2 pp. Available at: http://nrmsc.usgs.gov/research/beardna.htm.

Kennedy Jenks Consultants. 2006. Preliminary Engineering Report Many Glacier Sewer System and Wastewater Treatment Plant. Prepared for Glacier National Park, West Glacier, MT.

Martinka, C. 1972 Habitat relationships of grizzly bears in Glacier National Park, Progress Report, USDI National Park Service, Glacier National Park, West Glacier, MT.  19pp.

McClelland, B.R., P.T. McClelland, R.E. Yates, E.L. Caton, and M.E. McFadzen. 1996.  Fledging and migration of juvenile bald eagles from Glacier National Park, Montana.  Journal of Raptor Research 30 (2): 79-89.

McClelland, B.R., L.S. Young, P.T. McClelland, J.G. Crenshaw, H.L. Allen, D.S. Shea. 1994. Migration ecology of bald eagles from autumn concentrations in Glacier National Park, Montana. Wildlife Monographs 125. 61pp.

Mech, L.D., T.J. Meier, and J.W. Burch. 1991. Denali Park wolf studies: implications for Yellowstone. In Transactions of the North American Wildlife and Natural Resources Conference 56:86-90.

Montana Bald Eagle Working Group. 1994. Montana Bald Eagle management plan.  U.S. Bureau of Reclamation, Billings, MT. 104pp.

Montana Bull Trout Scientific Group. 1995. Flathead River Drainage Bull Trout Status Report. Prepared for the Montana Bull Trout Scientific Group.

National Park Service (NPS). 1978. “Cultural Resource Inventory of Specified Areas within Glacier National Park, Montana,” by Mark R. Guthrie, Midwest Archeological Center, National Park Service, Lincoln, Nebraska.

——. 1999a. General Management Plan. Glacier National Park, West Glacier, MT. 340pp.

——. 1999b. Glacier National Park Bald Eagle Operational Plan and Habitat Management Guidelines. Glacier National Park, West Glacier, MT.

——. 2000. Management Policies 2001. NPS D1416. Washington D.C.

——. 2004. Final Commercial Services Plan and Final Environmental Impact Statement. Glacier National Park. West Glacier, MT.

——. 2006. Management Policies, 2006. Prepared by U.S. Department of the Interior, National Park Service, Washington, D.C.

Reeves, B., Ph.D. “Glacier National Park Archaeological Inventory:  1993 Field Season Final Report.” Montana State University and Museum of the Rockies.  1995.

Rich, C. 1996. Influence of abiotic and biotic factors on the occurrence of resident bull trout in fragmented habitats in western Montana. M.S. Thesis. Montana State University, Bozeman, MT.

Rieman, B.E. and McIntyre, J.D. 1993. Demographic and habitat requirements for conservation of bull trout. USDA Forest Service, General Technical Report INT-302, Intermountain Research Station, Ogden, Utah.

Ruediger, Bill, Jim Claar, Steve Gniadek, Bryon Holt, Lyle Lewis, Steve Mighton, Bob Naney, Gary Patton, Tony Rinaldi, Joel Trick, Anne Vandehey, Fred Wahl, Nancy Warren, Dick Wenger, and Al Williamson. 2000. Canada lynx conservation assessment and strategy. USDA Forest Service, USDI Fish and Wildlife Service, USDI Bureau of Land Management, and USDI National Park Service. Forest Service Publication #R1-00-53, Missoula, MT.

Stetz, J. 2003. Personal communication on January 7 with wildlife biologist, Northern Rocky Mountain Science Center, Glacier National Park, West Glacier, MT.

US Fish and Wildlife Service (USFWS). 1987. Northern Rocky Mountain Wolf Recovery Plan.  U.S. Fish and Wildlife Service, Denver, Colorado.  119 pp.

——. 1986. Recovery plan for the pacific bald eagle.  US Fish and Wildlife Service, Portland, OR.

——. 1993. Grizzly bear recovery plan.  US Fish and Wildlife Service, Missoula, MT. 181 pp.

Yates, R.E. 1989. Bald eagle nesting ecology and habitat use: Lake McDonald, Glacier National Park, Montana. M.S. thesis, University of Montana, Missoula, MT.

Yates, R.E., B.R. McClelland, P.T. McClelland, C.H. Key, and R.E.Bennetts. 2001. The influence of weather on golden eagle migration in Northwestern Montana. Journal of Raptor Research 35(2):81-90.