## National Park Service

## U.S. Department of the Interior

# Lake Clark National Park and Preserve

# Alaska

# Two Lakes Land Exchange

# Environmental Assessment

# May 2010

Flooded channel

Table of Contents

Purpose and Need…………………………………………………………………………………..1

Description of Alternatives………………………………………………………………………2-4

Alternative 1: No Action

Alternative 2: Exchange of Lands (NPS Preferred Alternative)

Environmental Consequences……………………………………………………………………4-8

Alternative 1: No Action

Alternative 2: Exchange of Lands (NPS Preferred Alternative)

Consultation and Coordination…………………………………………………………………..9

List of Maps

Map 1. Proposed Land Exchange

Appendices

Appendix A. Channel Assessment

Appendix B. Section 810 Evaluation

# PURPOSE AND NEED

This assessment evaluates a proposal for Lake Clark National Park and Preserve to acquire the eastern 3 acre portion of U.S. Survey 5035 Alaska (NPS Tract LACL 05-104), a five acre parcel located on the shore of Two Lakes, in exchange for a parcel of an equivalent value abutting the western boundary of U.S. Survey 5035 (see Map 1: Proposed Land Exchange).

U.S. Survey 5035 is within the floodplain of the Tlikakila River and within an area that is subject to flooding from increasing spring snowmelt and run-off (see Appendix A: Channel Assessment). A low drainage transects the tract where the Tlikakila River has recently eroded a channel and now compromises a cabin, which is more than 50 years old, and an outhouse. The land exchange would provide the landowner sufficient room to move the cabin, which is presently in danger of being washed into the river and deposited in Two Lakes.

The property owners have proposed to exchange the eastern portion of their property to provide NPS with ownership of the property from the west bank of the Tlikakila River channel to the eastern boundary of U.S. Survey 5035, including all undeveloped land east of the river/creek. The exchange would be for a similar amount of acreage abutting their western boundary.

The parcel being proposed for exchange is in designated Wilderness. The Wilderness Boundary follows the northeast shoreline of Two Lakes.

Such exchanges are authorized by Section 1302(h) of ANILCA.

(GAAR Housing Management Plan 2007)

This environmental assessment (EA) analyzes the potential environmental impacts which could result from the alternatives considered, including the No Action alternative.  This EA has been prepared in accordance with the National Environmental Policy Act (NEPA) of 1969, regulations of the Council of Environmental Quality (CEQ) (40 Code of Federal Regulations 1508.9), and  the NPS NEPA compliance guidance handbook (Director’s Order [DO]-12, Conservation Planning, Environmental Impact Analysis, and Decision Making) (NPS 2001).

Figure 4.  Anaktuvuk Pass Lot Diagram

Photo 1. Aerial view of private property at the mouth of the new river channel.  The new channel flows into Two Lakes at the bottom of the image. The structures at risk are on the left side of the image.

# DESCRIPTION OF ALTERNATIVES

## Alternative 1: No Action

Under the no-action alternative, no exchange land would occur. The landowner may or may not be able to move the cabin and other structures to a limited area of his property northwest of the new river channel and far enough away to prevent destruction from erosion (see Photo 1 and Map 1: Proposed Land Exchange).

## Alternative 2:  Exchange of Lands (NPS Preferred Alternative, and Environmentally Preferred Alternative).

Under the preferred alternative, Lake Clark National Park and Preserve would acquire the eastern 3 acre portion of U.S. Survey 5035, in exchange for a parcel of similar value abutting the western boundary of the property (See Map 1: Proposed Land Exchange). The western boundary of the parcel to be acquired by the NPS would follow the right bank (as you’re looking downstream) of the river. Similar to a coastline boundary, it would be dynamic; regardless of where the channel migrates, NPS would always retain the entirety of the channel.

The NPS would continue to retain similar acreage with no structures. The private parcel would retain the existing cabin, moved to a new location, with no net loss in acres. The acquired parcel would become part of the designated Wilderness.

The proposed exchange may include but may not be limited to:

- The cost of the surveys shall be borne by the owners of U.S. Survey 5035, subject to NPS selection approval.

- Costs of the federally required title report and title insurance policy shall be borne by the NPS.  Costs of recording of deeds/patent will be borne by the owners of U.S. Survey 5035 and NPS, respectively.

- The NPS will conduct all necessary natural and cultural resource assessments, as mandated by the various applicable federal laws and regulations.

Alternatives Considered but Dismissed from Further Consideration

Rerouting the stream on or upstream of the private parcel to revert it to the pre-flooded channel:  This alternative would require importation of a large bulldozer in winter, taking it up the floodplain, and building a large dam/levee in the current river channel.  This alternative was dismissed due to logistical difficulties of getting heavy equipment on site; resource damage to the stream, Two Lakes, and wilderness resource values; and the high probability of failure.

Photo 2. The flooded channel is on the left side of the image.

# ENVIRONMENTAL CONSEQUENCES

## Alternative 1: No Action

Water Quality and Fish:  Under this alternative, no land exchange would occur so the structures on the private parcel may or may not be moved away from the river. The western edge of the landowner’s property is approximately 200 feet from the new channel. If the structures are not moved, or if they cannot be moved far enough from the new channel due to topography, drainage, or other considerations, the cabin, fuel drums and outhouse could be flooded and/or washed out, depositing debris into Two Lakes.

Two Lakes is a pristine glacial lake important to the Stony River and Kuskokwim River watershed and associated salmon fisheries.  Two Lakes is best classified as an oligotrophic discontinuous cold polymictic lake.  Lewis (1983) defines discontinuous cold polymictic lakes as ice-covered part of the year, ice-free above 4 °C and stratified during the warm season for periods of several days to weeks, but with irregular interruption by mixing.  Among major lakes in LACL, water quality in Two Lakes is most similar to Twin Lakes, Lachbuna, and Lake Clark (Chamberlain 1989).  All of these lakes are influenced by runoff from glacial melting in headwater tributaries and are dominated by calcium ions and bicarbonate anions.  During June - October 1985 - 1987, Chamberlain’s (1989) measurements of average specific conductivity and pH in Two Lakes were 69.6 µS/cm and 7.45.  Water quality in the Tlikakila River is unknown, but is likely similar to the water quality of the Tlikakila River, a glacial tributary of Lake Clark, documented by Brabets (2002).

Fish species present in Two Lakes include lake trout Salvelinus namaycush, Dolly Varden Salvelinus malma, round whitefish Prosopium cylindraceum, pygmy whitefish Prosopium coulteri, sockeye salmon Oncorhynchus nerka, northern pike Esox lucius, longnose sucker Catostomus catostomus, slimy sculpin Cottus cognatus, and ninespine stickleback Pungitius pungitius (Russell 1980).

Once in Two Lakes, human-made debris would scatter along the shore, creating hazards to fish, and potentially depositing fuel on the shoreline. Such flooding would likely occur during high water events in summer and early fall during the height of salmon migration.

Cumulative Impacts: Erosion would continue to add naturally occurring sediments to the channel and to the delta being formed near the shore of Two Lakes.  This causes increased turbidity at the mouth of the delta. The contribution of impacts from this alternative would constitute the majority of the impact. The cumulative impact of this alternative plus the impact of past, present, and reasonably foreseeable actions would be minor.

Conclusion:  This alternative would create minor adverse impacts to water quality and fish from human-made debris entering Two Lakes. This alternative would not lead to the impairment of park unit natural resources that are key to the purposes and values for which the park unit was established.

Wilderness: The Wilderness boundary follows the shoreline of Two Lakes, with the uplands to the northeast in designated Wilderness.  This area contains ecosystems with natural ecological processes fully intact, an environment free from modern human control or manipulation, lack of developments and signs of human activities, and opportunities for solitude.

Under this alternative, no land exchange would occur so the structures on the private parcel may or may not be moved far enough away from the new channel. Erosion could cause the cabin, fuel drums and outhouse to be flooded and/or washed out, depositing debris into Two Lakes. Once in Two Lakes, human-made debris would scatter along the shore, creating a visual intrusion of signs of modern human developments, which would degrade the undeveloped character of the wilderness in that location.  This debris would also diminish opportunities for solitude or primitive and unconfined recreation if visitors can see and encounter such litter and signs of modern human developments. The natural and untrammeled qualities of wilderness character would not be affected.

Cumulative Impacts: Minor impacts on the undeveloped quality of wilderness character in the Two Lakes area have been created by the presence of the owners of U.S. Survey 5035 and include activities associated with a remote cabin inholding: refuse disposal, aircraft noise, generator noise, and exhaust that provide power to the cabin. These actions have created a minor adverse impact to the undeveloped character of wilderness as well as for opportunities for solitude. The contribution of impacts from this alternative would be very small. The cumulative adverse impact of this alternative plus the impact of past, present, and reasonably foreseeable actions would be minor.

Conclusion: This alternative would create minor adverse impacts to the undeveloped quality of wilderness character and opportunities for solitude or primitive and unconfined recreation if erosion causes the cabin and other structures to be swept into the river and lake. This alternative would not lead to the impairment of park unit natural resources that are key to the purposes and values for which the park unit was established.

Land Ownership and Use: A cabin that is more than 50 years old exists on the landowner’s property. Under this alternative the cabin would continue to be exposed to damage or destruction from erosion by the river.

Under this alternative it would be more difficult for the landowners to access the eastern portion of their property if their property continued to be divided by the new river channel. Safety to landowners and facilities could be compromised by spring and fall floods if the landowners aren’t able to relocate the structures far enough from the new channel.

Cumulative Impacts: There are no past, present, or reasonably foreseeable actions that would affect land ownership and cultural values. Impacts from this alternative create all of the impact to land ownership and cultural values; therefore, the cumulative impact is the same as the impact from this alternative (see conclusion below).

Conclusion: This alternative could create moderate adverse impacts to the landowner’s structures and ability to use the property.

## Alternative 2:  Exchange of Similar Lands (NPS Preferred Alternative, and Environmentally Preferred Alternative).

Water Quality and Fish:  The land exchange would allow the property owner to move the cabin, material piles, equipment, and fuel drums away from the rapidly eroding channel.  Human-derived materials in the structures and buildings would not wash into Two Lakes.  Only natural sediments and vegetation debris would form the fan.  Consequently, water quality and fish would not be affected (see Impacts to Water Quality and Fish for Alternative 1 for a description of water quality and fish in the project area).  Acquisition of the lands now encompassing the river channel is highly desirable and consistent with the park's enabling legislation.

Cumulative Impacts: Erosion would continue to add naturally occurring sediments to the channel and to the delta being formed near the shore of Two Lakes.  This would cause increased turbidity at the mouth of the delta. This alternative would not contribute to the overall impact to water quality and fish. The cumulative impact of this alternative plus the impact of past, present, and reasonably foreseeable actions would be negligible.

Conclusion: This alternative would create a minor beneficial impact to water quality and fish by acquiring the only portion of the Tlikakila River that is not currently on NPS land. There would be no change from the existing natural condition of the area. The exchange of similar lands would not lead to the impairment of park unit natural resources that are key to the purposes and values for which the park unit was established.

Wilderness:  The Wilderness boundary follows the shoreline of Two Lakes, with the uplands to the northeast in designated Wilderness.  This area contains ecosystems with natural ecological processes fully intact, an environment free from modern human control or manipulation, lack of developments and signs of human activities, and opportunities for solitude.

Acquisition of the eastern portion of U.S. Survey 5035 and disposal of equivalent acres of NPS preserve lands abutting the property would put into federal ownership the full course of the Tlikakila River channel that now transects this property, increasing the level of protection by placing this river within designated Wilderness.

The proposed land exchange would not lead to significant changes in the use of the properties exchanged; wilderness character for the undeveloped lands exchanged would be essentially the same, resulting in no net loss of Wilderness acres, or degradation of wilderness character. The NPS would continue to retain identical acreage with no structures. Under this alternative the amount of NPS lands with undeveloped, untrammeled, natural character, as well as opportunities for solitude and primitive or unconfined recreation, would remain the same as in the past.

Cumulative Effects:  Minor impacts on the undeveloped quality of wilderness character in the Two Lakes area have been created by the presence of the owners of U.S. Survey 5035 and include activities associated with a remote cabin inholding: refuse disposal, aircraft noise, generator noise, and exhaust that provide power to the cabin. The contribution of impacts from this alternative would be insignificant. The cumulative impact of this alternative plus the impact of past, present, and reasonably foreseeable actions would be minor.

Conclusion: This alternative would create a minor beneficial impact to wilderness by acquiring the only portion of the Tlikakila River that is not currently on NPS land. The exchange of similar lands would not lead to the impairment of park unit natural resources that are key to the purposes and values for which the park unit was established.

Land Ownership and Uses: A cabin on the landowner’s property could be moved so that the owners could continue to use it. This alternative would provide more accessible land for the landowners to move their cabin to avoid potential damage or destruction from erosion by the river.

Under this alternative the private parcel would retain the existing cabin with no net loss in acres. Spring and fall floods would not pose a risk to safety of landowners or to their facilities if the landowner moves the structures away from the river. There are no recorded cultural sites on the NPS parcel proposed for exchange, and NPS personnel have identified the parcel as a low probability site for cultural resources.

Cumulative Impacts: There are no past, present, or reasonably foreseeable actions that would affect land ownership and cultural values. Impacts from this alternative create all of the impact to land ownership and cultural values; therefore, the cumulative impact is the same as the impact from this alternative (see conclusion below).

Conclusion: This alternative would create moderate beneficial impacts to the landowner and their ability to use the cabin because the landowner could move the cabin to a location away from the river, thereby enhancing their safety and the preservation of the structures.

Literature Cited

Brabets, T. P. 2002. Water quality of the Tlikakila River and five major tributaries to Lake Clark, Lake Clark National Park and Preserve, Alaska, 1999-2001.  U.S. Geological Survey Water Resources Investigations Report 02-4127.  pp. 29.

Chamberlain, D. M. 1989. Physical and biological characteristics and nutrients limiting primary productivity, Lake Clark, Alaska.  M.S. Thesis, Michigan Technical University, Houghton, Michigan  pp. 135.

Lewis, W. M., Jr. 1983. A revised classification for lakes based on mixing.  Canadian Journal of Fisheries and Aquatic Sciences 40: 1779-1787.

Russell, R. 1980. Fisheries inventory of the waters within the Lake Clark National Monument area. Alaska Department of Fish and Game. Administrative Report. Juneau.

# CONSULTATION AND COORDINATION

Chuck Gilbert, Lands, Alaska Regional Office

Joel Hard, Superintendent, Lake Clark National Park and Preserve

Adrienne Lindholm, Outdoor Recreation Planner, Lake Clark National Park and Preserve

Buck Mangipane, Wildlife Biologist, Lake Clark National Park and Preserve

Kristen Pearson, Lands, Alaska Regional Office

Bud Rice, Environmental Protection Specialist, Alaska Regional Office

Page Spencer, Chief of Natural Resources, Lake Clark National Park and Preserve

Dale Vinson, Archeologist, Lake Clark National Park and Preserve

# Dan Young, Fisheries Biologist, Lake Clark National Park and Preserve

APPENDIX A: Tlikakila River Channel Assessment

Jack Smith, Volunteer-in-Parks, Lake Clark National Park and Preserve

August 31, 2009

Issue: Flows during the 2009 runoff have altered the primary course of the Tlikakila River at Two Lakes in Lake Clark National Park and Preserve. Structures associated with a park inholding on the north shore of Two Lakes are currently threatened by this new, primary river channel (see Photo 1). An aerial assessment of the situation was made on August 19, 2009 to evaluate: 1) the potential for this new primary channel to persist over the short term (next five to ten years); and 2) the potential for channel alteration actions that could be taken to redirect river flow out of this new channel and away from the structures.

Physical Setting. The Tlikakila River originates from glaciers on the northern flanks of 6,000foot peaks immediately northwest of Telaquana Pass in Lake Clark National Park and Preserve. The river flows in a northwesterly direction joining the Necons River at its confluence with Two Lakes. Upon entering the Necons River valley, the relatively high gradient Tlikakila River drops substantial bedload materials to form a broad alluvial deposition area (Figures 1 and 2).

The Tlikakila River channel in this alluvial deposition area appears to be a D3 Stream Type (Rosgen, 1996). The D3 stream type is typified by multiple channels found on broad alluvial fans consisting of coarse depositional materials. These braided systems consist of interconnected distributary channels formed in depositional environments. Channel bed materials in a D3 stream type are predominantly cobbles, with a strong bi-modal distribution of sands containing imbedded gravels. This braided channel system is characterized by flashy flow conditions, high bank erosion rates, excessive longitudinal deposition when flows are contained within the channel, annual shifts of the bed locations, and excessive lateral deposition associated with channel overflow and lateral shift. The historic lateral shifting of channels in the Tlikakila River depositional area is well demonstrated in Figures 2 and 3.

Figure 1. Tlikakila and Necons Rivers entering Two Lakes. USGS Lime Hills A2 and A3 Quads. Lateral shift of the primary channel in 2009 to a secondary channel is approximately shown by the red line.

High bank erosion associated with the 2009 runoff event shifted the primary channel to an older channel location slightly to the west (Figure 3). Tree and brush establishment along this new reach of primary channel suggests that although this reach appears to have been a major channel in the past, it has only carried minor, secondary flows in more recent times. Increased bank erosion along this new primary channel and the increased transport of bank and bed materials as a result of increased flow volumes and velocities can be readily seen in the recent depositional fan feature in Two Lakes (see Photo 1).

Figure 2. Looking southwest across the Tlikakila River depositional area toward Two Lakes. The new reach of primary channel is immediately outside of the right-center of the photograph (arrow).

Assessment.

1.	What is the potential this new reach of primary channel will persist over the short-term (5-10 years)?

In a D3 system, a single high runoff event has the potential to immediately redirect flows to a new primary channel almost anywhere in the depositional area. Such a dramatic shift could once again result in only minor flows in the current channel entering Two Lakes near the structures. It is impossible to predict when such an event might occur; however, current channel erosional processes reduce the probability of this occurring in this immediate area in the near future. The amount of depositional material in Two Lakes suggests elevated bank and bed erosion are occurring in this new primary channel. Longitudinal depositions will begin to occur in the channel with reduced velocities as bank erosion widens the channel. However, years of such deposition and channel widening may be required before an aggregated channel enables typical flow regimes cause a major lateral shift in this immediate area. It is postulated this new primary channel has a high potential to remain over the short term (5-10 years).

Figure 3. Shift point from old primary channel (left) to new primary channel (upper center). Direction of flow is from bottom left to upper center of photograph.

2.	What is the stability of this new reach of primary channel?

D3 stream types are naturally very unstable. The new reach of primary channel appears to be in an even higher state of accelerated bank and bed erosion at this time. High levels of bank erosion and associated channel widening can be expected to continue for the near future. There is a real potential that structures along the shore of Two Lakes will ultimately be impacted by bank erosion and channel shift in the next 5 or 10 years.

- Can channel manipulation efforts that either return a majority of flows to the old primary channel or redirect the primary flows to a new channel be a long-term solution?

D3 stream types are very sensitive to disturbance. These systems have very high streambank erosion potential, very high sediment supply characteristics, and poor recovery potential (Rosgen, 1996). Rosgen also reports this stream type has poor to only fair suitability for channel structures such as bank placed boulders, single-wing deflectors, double-wing deflectors, cross vein weirs, “w” weirs, or j-hook weirs. In most instances, lateral movement of the channel in a D3 stream type will be directed by material deposition and structures built in these channels to redirect flows have the high potential to experience accelerated bank and bed erosion that render them ineffective. It is postulated that channel manipulation features to redirect the flows of the Tlikakila River from its current primary flow channel will likely have limited effect and, even if immediately successful, those results will be very short-term at best. Any structures built on the left bank (looking downstream) of the Necons River from upstream of the Tlikakila River depositional area (NE¼ Section 4) to the extreme northeast corner of Two Lakes (NW¼ Section 16) ultimately have a real potential for impact as a result of river channel shift in this depositional area (Figure 1).

References

Rosgen, D. 1996. Applied River Morphology. Wildland Hydrology. Pagosa Springs, CO. 352pp.

# APPENDIX B

# Alaska National Interest Land Conservation Act (ANILCA), Section 810(a) Summary Evaluations and Findings

I.  INTRODUCTION

This section was prepared to comply with Title VIII, Section 810 of the Alaska National Lands Conservation Act (ANILCA). It summarizes the evaluations of potential restrictions to subsistence uses that could result from the proposed action by the National Park Service (NPS) to exchange similar properties at Two Lakes in Lake Clark National Park and Preserve.

II. EVALUATION PROCESS

Section 810(a) states:

“In determining whether to withdraw, reserve, lease, or otherwise permit the use, occupancy, or disposition of public lands...the head of the federal agency...over such lands...shall evaluate the effect of such use, occupancy, or disposition on subsistence uses and needs, the availability of other lands for the purposes sought to be achieved, and other alternatives which would reduce or eliminate the use, occupancy or disposition of such lands which would significantly restrict subsistence uses shall be effected until the head of such Federal agency–

(1) gives notice to the appropriate State agency and the appropriate local committees and regional councils established pursuant to Section 805;

(2) gives notice of, and holds, a hearing in the vicinity of the area involved; and

(3) determines that (A) such a significant restriction of subsistence uses is necessary, consistent with sound management principles for the utilization of the public lands, (B) the proposed activity will involve the minimal amount of public lands necessary…and (C) reasonable steps will be taken to minimize adverse impacts upon subsistence uses and resources resulting from such actions.”

ANILCA created new units and additions to existing units of the national park system in Alaska. Lake Clark National Park and Preserve was created by ANILCA Title II Section 201(7)(a) for the following purposes:

“To protect the watershed necessary for perpetuation of the red salmon fishery in Bristol Bay; to maintain unimpaired the scenic beauty and quality of portions of the Alaska Range and the Aleutian Range, including active volcanoes, glaciers, wild rivers, lakes, waterfalls, and alpine meadows in their natural state; and to protect habitat for and populations of fish and wildlife including but not limited to caribou, Dall sheep, brown/grizzly bears, bald eagles, and peregrine falcons.”

The potential for significant restriction of subsistence uses must be evaluated for the proposed action’s effect upon “...the availability of other lands for the purposes sought to be achieved and other alternatives which would reduce or eliminate the use, occupancy, or disposition of public lands needed for subsistence purposes.” (Section 810 (a) of ANILCA).

III. PROPOSED ACTION ON FEDERAL PUBLIC LANDS

Under the preferred alternative, Lake Clark National Park and Preserve would acquire the eastern portion of U.S. Survey 5035 in exchange for a similar amount of acreage abutting the western boundary of the property. The NPS would continue to retain identical acreage with no structures. The private parcel would retain the existing cabin with no net loss in acres.

# IV.  AFFECTED ENVIRONMENT

A summary of the affected environment pertinent to subsistence uses is presented here.

Lake Clark National Park and Preserve is located in South-central Alaska adjacent to Cook Inlet and was established in 1980 by Title II Section 201(7) of ANILCA. Subsistence uses are allowed within Lake Clark National Park and Preserve in accordance with Title II, Section 201(1) and Title VIII of ANILCA.

Section 803 of ANILCA defines subsistence uses as:  “the customary and traditional uses by rural Alaska residents of wild, renewable resources for direct personal or family consumption as food, shelter, fuel, clothing, tools, or transportation; for the making and selling of handicraft articles out of non-edible by-products of fish and wildlife resources taken for personal or family consumption; for barter, or sharing for personal or family consumption; and for customary trade.”

In accordance with Title 36 CFR Part 13 regulations, residents of the NPS designated resident zone communities of Iliamna, Lime Village, Newhalen, Nondalton, Pedro Bay and Port Alsworth are qualified to engage in subsistence uses within Lake Clark National Park and Preserve. Local rural residents who do not live in these communities, but who have customarily and traditionally engaged in subsistence activities within the park and preserve may continue to do so with a subsistence use permit issued by the park superintendent.

Major resources used for subsistence by resident zone communities include caribou, brown bear, moose, beaver, Dall sheep, snowshoe hare, fox, lynx, mink, wolf, wolverine, ptarmigan, waterfowl, otter, marine mammals, salmon, trout, Dolly Varden, grayling, pike, suckers, humpback and round whitefish, halibut, crab, clams, berries, wild edible plants, and wood.

Lake Clark National Park (2,439,000 acres) and Preserve (1,214,000 acres) include Game Management Units (GMUs) 9A, 9B, 16B, 17B and 19B. These GMUs also include other federal public lands such BLM administered lands in 9B, 16B and 17B; the Denali National Park and Preserve in 16B; and the Upper Mulchatna Controlled Use Area in 17B.

The proposed action at Two Lakes is located in the Southcentral Area, GMU 19B, on the west side of Alaska Range within the boundaries of Lake Clark National Preserve. Federal subsistence fishery regulations currently allow Federally-qualified rural residents to take all fish other than salmon, Dolly Varden, trout, grayling, char and burbot for subsistence purposes. Federal subsistence game regulations allow residents of GMU 19B to harvest black bears, caribou, Dall sheep, moose, coyote, arctic fox, red fox, hares, wolves, wolverine, beaver, lynx, marten, mink, muskrat, otter, grouse and ptarmigan for subsistence uses.

V.  SUBSISTENCE USES AND NEEDS EVALUATION

To determine the potential impact on subsistence activities, three evaluation criteria were analyzed relative to current subsistence resources that could be impacted.

The evaluation criteria are:

- The potential to reduce important subsistence fish and wildlife populations by (a) reductions in abundance; (b) redistribution of subsistence resources; or (c) loss of habitat.

- Potential impacts the action may have on access for subsistence hunters and fishermen

- The potential for the action to increase competition among hunters and fishermen for subsistence resources.

1.  The potential to reduce populations:

No significant reductions in populations of subsistence fish and wildlife resources are anticipated as a result of the proposed action. Two Lakes is primarily a sport hunting destination and very little subsistence activity takes place on park lands in the area. The occasional subsistence hunting, trapping, fishing and gathering activities that may occur in the vicinity of the cabin site should not be affected. The proposed project does not alter habitats important for subsistence or result in any measurable reduction in or redistribution of wildlife or other subsistence resources.

2.  Restriction of Access:

All rights of access for subsistence harvest on NPS lands are granted by Section 811 of ANILCA. Lake Clark National Park and Preserve are managed according to legislative mandates, NPS management policies and guidelines within the approved LACL General Management Plan. The proposed action does not limit or restrict the access of subsistence users to natural resources within the park or preserve.

- Increase in Competition:

The proposed action would not result in increased competition for fish, wildlife or other resources that would significantly impact subsistence users. NPS regulations and provisions of ANILCA mandate that if and when it is necessary to restrict taking of fish or wildlife, subsistence users will be given a priority over other user groups.

# VI. AVAILABILITY OF OTHER LANDS

The proposed action is site-specific to a private inholding. To keep the property contiguous, no other lands are suitable for the project. The proposed action is consistent with NPS mandates and the General Management Plan and is not expected to impact subsistence uses.

# VII. ALTERNATIVES CONSIDERED

A No Action Alternative to preserve the status quo was considered in preparing this analysis.

#### FINDINGS

This analysis concludes that acquiring the eastern 3 acre portion of U.S. Survey 5035 Alaska (NPS Tract LACL 05-104), in exchange for an equivalent amount of acreage abutting the western boundary of U.S. Survey 5035 would not result in a significant restriction of subsistence uses.

There are two Tlikakila Rivers. This one flows into Two Lakes. The other one flows into Lake Clark.

There are two Tlikakila Rivers. This one flows into Two Lakes. The other one flows into Lake Clark.